3GPP TSG-WG SA2 Meeting #166	S2-2412320

# Orlando, USA, 18th – 22nd Nov, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5879 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On the calculation for the Energy Consumption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In previous meeting the CR with basic principle (S2-2411072) was approved. One of the leftover issue is the way of calculating/estimating the energy consumption (EnC) for the certain granularity. Based on the analyses in S2-2410421 and S2-2410425, we propose to include the informative example equation in the Annex. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new Annex for calculating the energy consumption. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how to generate the energy consumption within EIF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * * all text is new

Annex x (informative):
Evaluation of Energy related parameters

# X.1	General

This informative Annex provides the procedure to estimate the energy related parameters.

## X.2	Evaluation of Energy Consumption parameters

NOTE 1:	The equations illustrated below can be implemented by EECF or SMF. And other equations with using the same information as the input (i.e., EnC and Data Volume of the NFs, Data Volume of the targeted granularity) are not excluded by implementation.

The evaluated EnC of the required granularities within the requested time span denotes the total energy consumption as mentioned in clause 5.X. The information from OAM on Data Volume and EnC of RAN and (I-)UPF is measured within a measuring interval (e.g., typically can be 15 mins), while the requested time span of the EnC may not be aligned with the measuring interval. Within the measuring interval, the UE might be handed over to a new RAN node, and the EnC at RAN nodes should consider all involved RAN nodes within such measuring interval.

NOTE 2:	It can be assumed that the EnC and Data Volume of the RAN/NF is evenly distributed within the time interval. One can base on this information to evaluate the total EnC and Data Volume of the RAN/NF for the requested time span.

NOTE 3:	The requested time span may not be fully aligned with the measuring interval, i.e., the requested time span may start at the middle of collection interval, and/or end at the middle of collection interval. Since in this release only the data volume is considered, such misalignment will not affect the EnC evaluation of the required granularity in this measuring interval.

NOTE 3:	The evaluation of EnC can be based on the information of the previous collection interval or the current collection interval, and for the latter case, the result of evaluation has to be provided at the end of the collection interval.

The EnC per UE, EnCUE, per PDU sesion per UE, EnCUE, PDU Session, and per QoS Flow oper PDU Session per UE, EnCUE, PDU Session, QoS Flow, within a single measuring interval are evaluated as described below. And the EnC of the required granularities within the requested time span, EnCUE, time span, can be the sum of EnCUE for the involved measuring interval.

The EnCUE , is estimated as the sum of the EnC for all PDU sessions which are active for the UE in the measuring interval.

| EnCUE= PDU SessionEnCUE, PDU Session | (1) |
| --- | --- |

Where:

-	EnCUE, PDU Session,  is the energy consumption for active PDU Session of the UE;

The energy consumption per PDU session is the sum of the contribution all serving UPF (UPF PSA and I-UPF) for the PDU session and of the energy consumption of  of  gNb at which the UE has been connected during the measuring interval:

| EnCPDU Session= UPFEnCPDU Session, UPF+gNbEnCPDU session, gNb | (2) |
| --- | --- |

Where

-	EnCPDU Session, UPF is the energy consumption at the UPF (UPF PSA and I-UPF) for the PDU Session;

-	EnC PDU Session,gNB is the energy consumption at the gNB for the PDU Session;

Therefore, substituiting the equation (2) in (1) the  EnCUE   is the sum of the EnC of the active PDU session of the UE as follow

| EnCUE=PDU SessionUPFEnCUE,PDU Session, UPF+gNBEnCUE,PDU session, gNB = UE,PDU SessionUPFData VolumeUE,PDU Session, UPFData VolumeUPF×EnCUPF+RANData VolumeUE,PDU Session, RANData VolumeRAN×EnCRAN | (3) |
| --- | --- |

Where:

-	Data VolumePDU Session, UPF is the data volume of a PDU Session at the UPF;

-	Data VolumeUPF is the total data volume at the UPF;

-	EnCUPF is the total energy consumption of the UPF.

-	EnCUPFRatio is the energy consumption per bit at the UPF.

-	Data VolumePDU Session,gNB is the data volume of a PDU Session at the gNB;

-	Data VolumegNB is the total data volume at the gNB;

-	EnCgNB is the total energy consumption of the gNB.

Similarly, the EnCUE, PDU Session,QoS Flow can be evaluated as follows:

EnCUE,  PDU Session, QoS flow= EnCUE, PDU Session, QoS Flow,  UPF+EnCUE,  PDU Session, QoS Flow, RAN

Where EnCUE, PDU Session, QoS Flow,  UPF= Data VolumeUE,PDU Session, QoS flow UPFData VolumeUPF ×EnCUPF

And	EnCUE, PDU Session, QoS Flow,  RAN= Data VolumeUE,PDU Session, QoS flow RANData VolumeRAN ×EnCRAN

## X.3	Evaluation of Renewable Energy parameters

Editor’s note:	 Whether the Renewable Energy is provided as a ratio per NF of whether the EnC due to renewable EnCRen  per NF is provided by OAMwill be coordinated with SA5.

The total energy consumption due to renewable energy for a UE , EnCRUE .  is the sum of the EnC due to renewable energy of serving UPF plus the EnC due to renewable energy of the serving gNB.

If we assume that the OAM provides the ratio of renewable energy for the UPF, RenUPF,  and for gNB, RengNB,   the energy consumption due to renewable for UPF, EnCR,UPF, and RAN, EnCRgNBare respectively

| EnCRUPF=EnCUPF * RenUPF | (4) |
| --- | --- |
| EnCRRAN=EnCRAN * RenRAN | (5) |

Therefore the equation in clause x.2 are applicable to evaluate the EnC due to renewable per UE, replacing the EnCUPF and EnCRAN in (3) with the energy consumption due to renewable (4) and (5).

Therefore the  EnCRUE  is the following

| EnCR UE, = UE.PDU SessionUPFData VolumeUE,PDU Session, UPFData VolumeUPF*EnCRUPF+UE,RANData VolumeUE,PDU Session, RANData VolumegNb*EnCRgNB= UE.PDU SessionUPFData VolumeUE,PDU Session, UPFData VolumeUPF*EnCUPF*RenUPF+UE,RANData VolumeUE,PDU Session, RANData VolumeRAN*EnCgNB*RengNB | (6) |
| --- | --- |

The energy consumption due to non-renewable energy for a UE,  EnCNRUE  is estimated as follow

| EnCNRUE=EnCUE - EnCRUE | (7) |
| --- | --- |

The Renewable ratio for the UE is therefore

| RenUE=EnCRUEEnCNRUE=EnCRUEEnCUE - EnCRUE | (8) |
| --- | --- |

# * * * * End of changes * * * *