3GPP TSG-WG2 Meeting #166	S2-2412347

# Orlando, USA, November 18-22, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0493 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of section 6.3.2.5.3 “Procedure for Multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The associated step-by-step description for figure 6.3.2.5.3-1 does not match the numbering in the figure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligned the figure numbering and the step-by-step description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May cause some confusion for readers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

##### 6.3.2.5.3	Procedure for Multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B

Figure 6.3.2.5.3-1: General Procedures for 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model B

1.	The 5G ProSe Remote UE determines the Hop-Limit for discovery based on policy configuration (i.e. a mapping between maximum number of hops and RSC) or QoS requirements.

If the Hop-Limit is determined based on configuration associated with the RSC, the 5G ProSe Remote UE does not include the Hop-Limit in the Solicitation message. Otherwise, the 5G ProSe Remote UE includes the Hop-Limit in the Solicitation message.

2a.	The 5G ProSe Remote UE sends a 5G ProSe UE-to-Network Relay Discovery Solicitation message. The 5G ProSe UE-to-Network Relay Discovery Solicitation message additionally contains following IEs compared with that in clause 6.3.2.3.3: an indication that multi-hop relay is supported, hop count and Hop-Limit.

The Target Info may contain the User Info ID of the UE-to-Network Relay and Intermediate Relay(s).

3a.	If an indication that multi-hop relay is supported is contained in the received Solicitation message, the RSC contained in the Solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1a, of a 5G ProSe Intermediate Relay, and the Target Info matches the User Info ID of the 5G ProSe Intermediate Relay (if any), the 5G ProSe Intermediate Relay may decide to send a 5G ProSe UE-to-Network Relay Discovery Solicitation message.

The 5G ProSe Intermediate Relay shall drop the received Solicitation message if the hop count (corresponding to the number of Relays included in the message) has reached the Hop-Limit of the received Solicitation message or the (pre)configured maximum number of hops associated with the RSC.

The 5G ProSe Intermediate Relay may send a Response message when it has already found or established PC5 link with 5G ProSe UE-to-Network Relay(s), without sending Solicitation message. i.e., steps 4a-7a are skipped and step 8a is performed directly. The response message additionally contains the User Info ID of UE-to-Network Relay, path information to the UE-to-Network Relay which is an (ordered) list of User Info ID of intermediate Relay(s).

If the same information on User Info IDs of Remote UE and UE-to-Network Relay is received from different ProSe UEs, the 5G ProSe Intermediate UE-to- Network Relay may select a Solicitation message to be sent to the next hop based on various criteria (e.g., hop count, delay, channel quality of received messages, etc.).

NOTE:	If the 5G ProSe Remote UE does not receive any response after a pre-configured time, based on application requirement, it may increase the Hop-Limit and send the discovery message again.

4a.	A 5G ProSe Intermediate Relay sends a Solicitation message, it additionally includes its own User Info ID in the message. i.e., the message contains the path information which is an (ordered) list of User Info ID of Relays in the path that has relayed the Solicitation message. The hop count is increased by 1.

2b.-5b.	The Solicitation message from the same Remote UE goes through a different ordered list of 5G ProSe Intermediate Relays.

6-7. If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1, of the 5G ProSe UE-to-Network Relay, and the Target Info matches the User Info ID of the 5G ProSe UE-to-Network Relay (if any), then the 5G ProSe UE-to-Network Relay responds to the 5G ProSe Intermediate Relay with a 5G ProSe UE-to-Network Relay Discovery Response message. The 5G ProSe UE-to- Network Relay Discovery Response message additionally contains the path information compared with that in clause 6.3.2.3.3.

The 5G ProSe UE-to-Network Relay may choose the path based on e.g., the PC5 signal strength of each message received, hops to the Remote UE, the path information, etc.

Editor's note:	It is FFS how the 5G ProSe UE-to-Network Relay decides when to send the Relay Discovery Response message.

8-9.	A 5G ProSe Intermediate Relay forwards the 5G ProSe UE-to-Network Relay Discovery Response message. The Response message additionally contains the path information.

Editor's note:	It is FFS if and how the Relay Discovery Response message is forwarded along the path indicated in the path information.

10.	The Remote UE may perform relay path selection based on e.g., the PC5 signal strength and the number of hops to the 5G ProSe UE-to-Network Relay.

>>>> End of Changes <<<<