3GPP TSG-WG SA2 Meeting #165	S2-2411015

# Hyderabad, IN, 14th Oct – 18th Oct, 2024		(revision of S2-2410754)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5750 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of N3GPP device behind UE/5G-RGas concluded in TS 23.700-34 KI#4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The UIA_ARC WID includes that adding support of Non-3GPP device connecting behind a UE as concluded in the TR 23.700-32 KI#4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To support Non-3GPP device connecting behind the UE the following changes are needed: - Introduction of general part description and how identification of traffic from Non-3GPP device connecting behind a UEis supported. - Session management - QoS differentiation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusion of KI#4 would not be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (new), 5.x.1 (new), 5.x.2 (new), 5.x.3 (new), 5.x.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.x	Identification of traffic from Non-3GPP device connecting behind the UE

### 5.x.1	General

This clause specifies the scenario of a Non-3GPP device connecting through the UE that does not use NAS and that it is not authenticated by 5GC, but the 5GS is able to identify and manage traffic that originates from or is directed to the non-3GPP device, to provide differentiated QoS.

The support of identification of traffic for Non-3GPP devices connecting behind a 5G-RG is specified specified in TS 23.316 [84].

### 5.x.2 Traffic identification

When a Non-3GPP device is connecting to the UE , the UE  binds a Non-3GPP Device Identifier to the traffic form and to the non-3GPP device. This association enables the 5G System to distinguish between the traffic generated by different Non-3GPP devices connected through the same UE. How the UE identifies the Non-3GPP device, binds the traffic from the non-3GPP device to Non-3GPP Device Identifier is implementation specific.

At any point in time the Non-3GPP Device Identifier can be bound to only one single Non-3GPP device.

Non-3GPP Device Identifier Information is stored in the UDR and includes a Non-3GPP Device Identifier and QoS Information.

### 5.x.3	Session management enhancement

For Ethernet PDU Session Type, the UE send the Non-3GPP Device Identifier and may also send MAC address and/or the VLAN tag ID that is associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

For IPv4 PDU Session Type, the UE sends the Non-3GPP Device Identifier and it may send the IP Address and/ Port ranges associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

For IPv6 PDU Session Type, the UE sends the the Non-3GPP Device Identifier and it may send the IPv6 Address that is associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

Editor’s note: 	whether the the UE sends the Non-3GPP Device Identifier and user plane information (e.g. IP Address) in PDU session establishment is FFS.

### 5.x.4	QoS differentiation

QoS differentiation and policy control is defined in TS 23.503 [45] clause 6.1.3.xx

# * * * * End of changes * * * *