3GPP TSG-WG SA2 Meeting #166	S2-2412370

Orlando, USA, 18th Nov – 22nd Nov, 2024	(revision of S2-2411919)

# Title:	LS Reply on FS_XRM Ph2

# Response to:	LS (S2-2411282/R3-244844) on FS_XRM Ph2 from RAN3

# Release:	Release 19

# Work Item:	FS_XRM_Ph2

Source:	SA2

To:	RAN3

Cc:

Contact Person:

Name:	Mirko Schramm

Tel. Number:

E-mail Address:	mirko DOT schramm AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 thanks RAN3 for the LS on FS_XRM Ph2 (R3-244844/S2-2409598) and kindly asks RAN3 to consider the following SA2 feedback.

- From RAN3 perspective, the control plane based solution providing periodicity per QoS flow as part of the TSCAI is sufficient for semi-static periodicity case. Other potential cases (e.g. dynamic case) need to be clarified by SA2, if any.

Feedback from SA2:

SA2 has agreed to exclude the in-band signalling of the periodicity in Rel-19.

- For semi-static periodicity traffic, it is feasible for NG-RAN node to provide the available data rate to CN via UP. For dynamic traffic, some companies have concern that the reporting of available data rate may not be accurate.

RAN3 would like SA2 to further clarify the definition of “available data rate”, e.g., it is only below the GFBR, or between GFBR and MFBR, or even above MBFR; whether it should be periodic reporting, on demand reporting, or threshold(s) defined reporting.

Feedback from SA2:

From SA2 perspective, it is expected that the available data rate is properly reflecting the resources that are currently provided by the NG-RAN for the QoS Flow. While it is up to the application layer how to make use of it, we can however assume that the application will typically exchange data with a somewhat lower rate than what is reserved or measured at NG-RAN.

Regarding the definition of “available data rate”, it refers to the data rate that NG-RAN is currently able to provide for a QoS Flow with GFBR. The “available data rate” can be between the GFBR and the MFBR for a QoS Flow with MFBR>GFBR. The “available data rate” can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without AQP. The “available data rate” should however not be above the MFBR (as the NG-RAN is not required to provide more resources that what is required for the MFBR).

Periodic reporting should not be useful for the application layer (as the “available data rate” should often be identical to the GFBR).  Threshold based reporting is considered as useful with one or more threshold values provided by the application. When the data rate that NG-RAN is currently able to provide for a QoS Flow crosses one of the thresholds, the NG-RAN is expected to provide the reference to the respective threshold in the UL user plane packets and the application can then adjust its data rate accordingly. The usage of the first packets after a change in the “available data rate” should be sufficient to transfer the information to the UPF. Similar to the logic described in 23.501 clause 5.7.2.4.1a, the NG-RAN is expected to immediately report a change to a lower threshold value for the “available data rate” and a change to a higher threshold value with some hysteresis.

2. Actions:

To RAN3 group.

ACTION: 	SA2 asks RAN3 group to take the above into account and to provide feedback if necessary.

3. Dates of next TSG SA WG2 Meetings:

TSG SA2 Meeting #166-Adhoc-e	20th Jan – 24th Jan, 2025			Online

TSG-SA2 Meeting #167		17th Feb – 21st Feb, 2025			Athens, GR