3GPP TSG SA WG2 Meeting #166	S2-2412405

# 18 - 22 November, 2024, Orlando , US				    						(Revision of S2-2411606)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1509 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of DC exposure and standalone DC with IMS DC architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The architecture and procedures supporting DC event exposure and standalone DC has been agreed and captured in Rel-19 TS 23.228. The clause of architecture of the IMS DC needs update to align with the new added architecture and procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the limit to allow standalone DC establishment; - Add the reference to Annex for IMS DC event exposure and IMS DC capability exposure. - Add new clause of NEF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.1, AC.2.1, AC.2.2.4, AC.2.2.6, AC.2.2.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Annex XX and YY referrred to in this CR are numbered in CR1409 and CR1489. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

Annex AC (normative):
Support of Data Channel services in IMS

# AC.1	General

This annex describes IMS architecture enhancements to support data channel services.

Service based interfaces applicable to data channel services are specified in clauses AA.2.4 and AA.2.5.

The architecture in this annex supports separation of signalling function and media function supporting data channel services. A network function for data channel management signalling, Data Channel Signalling Function (DCSF), is specified. The network function to handle data channel media can be provided by introducing a new network function called Media Function (MF), which interacts with the IMS AS via the service-based interface DC2. MF provides media capabilities in support of IMS DC and Augmented Reality.

NOTE:	In this Release of the specification the IMS AS supporting data channel services is collocated with the TAS.

The IMS architecture supporting data channel services also supports exposure of IMS DC. The architecture and procedures of IMS DC event exposure are specified in Annex XX. The architecture and procedures of IMS DC capability exposure are specified in Annex YY.

In this Release of the specification IMS Data Channel is supported in the case of non-roaming and home routed roaming scenarios.

Multiplexing of multiple IMS Data Channel streams with different endpoints onto a single SCTP association is not supported in this Release of the specification.

# AC.2	Architecture and functions

## AC.2.1	Architecture

Figure AC.2.1-1 shows the data channel architecture when using the service-based DC media function.

Figure AC.2.1-1: Architecture option of IMS supporting DC usage with MF

NOTE 1:	MDC2 may connect different IMS security domains.

NOTE 2:	DC5 reference point is not specified in 3GPP.

NOTE 3:	MDC1, MDC2 and MDC3 reference points are not specified in 3GPP.

NOTE 4:	DC3 and DC4 reference points are not specified in this Release.

NOTE 5:	The reference points related to exposure of IMS DC events and capabilities are specified in Annex XX and Annex YY.

## AC.2.2	Functional entities

### AC.2.2.1	Data Channel Signalling Function (DCSF)

The DCSF is the signalling control function that provides data channel control logic. The DCSF is not involved in SIP signalling. The DCSF supports the following functionalities:

-	The DCSF receives event reports from the IMS AS and decides whether data channel service is allowed to be provided during the IMS session.

-	The DCSF manages bootstrap data channel and (if applicable) application data channel resources at the MF via the IMS AS.

-	The DCSF manages interworking between application data channel media and audio/video media when applied and instructs to the MF via the IMS AS;

NOTE 1:	Interworking between application data channel and voice/video refers to the change of transporting protocols which is specified in SA WG4.

-	The DCSF supports HTTP web server functionality to download data channel applications (bootstrapping) via MF to the UE based on UE subscription.

-	The DCSF downloads data channel applications from the Data Channel Application Repository.

NOTE 2:	The detailed procedures to download data channel applications to the UE are out of scope of this specification.

-	The DCSF interacts with NEF for data channel capability exposure via DC3.

NOTE 3:	Interaction with NEF is not specified in this release of the specification.

-	The DCSF interacts with the DC Application Server for DC resource control via DC4/DC3/N33 and for traffic forwarding via MDC3.

NOTE 4:	Interaction with the DC Application Server for resource control is not specified in this release of the specification.

-	The DCSF interacts with HSS for retrieving and storing DCSF service specific data via N72/Sc.

### AC.2.2.2	Data Channel Application Repository (DCAR)

The Data Channel Application Repository stores the verified data channel applications which are retrieved by the DCSF when required.

### AC.2.2.3	Data Channel Media Function (MF)

The MF provides the media resource management and forwarding of data channel media traffic. The MF provides the following functionalities:

-	The MF manages the data channel media resources (bootstrap and application data channel resources, if applicable) under the control of the IMS AS.

-	The MF terminates the bootstrap data channel from the UE and forward HTTP traffic between the UE and DCSF via MDC1.

-	The MF may anchor the application data channel in P2P scenarios, if required, and forward application data traffic from/to the UEs.

-	The MF relay traffic on the A2P/P2A application data channels between the UE and the DC Application Server via MDC2.

-	The MF handles transforming of data channel media following the media instructions from the DCSF.

### AC.2.2.4	IMS AS

The IMS AS is enhanced to support the following functionalities:

-	The IMS AS interacts with the DCSF via DC1 for event notifications.

-	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2 for data channel media resource management.

-	The IMS AS interacts with HSS for retrieving and storing DC enhanced IMS AS service specific data via N71/Sh.

-	The IMS AS receives data channel control request from NEF/DC AS for establishment, update or release of bootstrap data channel and/or application data channel.

-	The IMS AS supports IMS DC events exposure as specified in Annex XX.

-	The IMS AS supports IMS DC capability exposure  as specified in Annex YY.

### AC.2.2.5	S-CSCF

In addition to the functions defined in clause 4.6.3, the S-CSCF is enhanced to support the following functionalities:

-	The S-CSCF includes a Feature-Caps header field indicating its data channel capability in the 200 OK response to the initial and any subsequent REGISTER request from UE.

AC.2.2.6	HSS

In order to support data channel service, the HSS is additionally enhanced with the following functionalities:

-	The HSS stores IMS Data Channel subscription data as transparent data.

-	The HSS interacts with DCSF during service data retrieval by DCSF via N72/Sc.

-	The HSS interacts with IMS AS during service data retrieval by IMS AS via N71/Sh.

-	The HSS supports IMS DC event subscription as specified in Annex XX.

AC.2.2.Y	NEF

The NEF supports exposure of IMS DC events and IMS DC capability:

-	The IMS AS supports exposure of IMS DC events as specified in Annex XX;

-	The IMS AS supports exposure of IMS DC capabilities as specified in Annex YY;

**** End of Changes ****