3GPP TSG-WG SA2 Meeting #166	S2-2412492

# Orlando, USA, 18 – 22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5045 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Energy related information collection and exposure procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, China Mobile? CATT? ZTE, Samsung? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the conclusion for KI#1 of TR 23.700-66, the support of network energy related information exposure is agreed. The procedures and services for supporting collection and exposure of the energy related information need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedure for supporting network energy related information collection and exposure is added. The new services of the new network function are also added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network energy related information collection and exposure is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X.Y (new), 5.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.501 CR 5628 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | S2-2409980 |  |  |  |  |  |  |  |  |

# * * * * First change (all text new) * * * *

4.X.Y	Energy related information collection and exposure

This clause is used by the EIF to collect the energy related information and expose the energy related information to the Consumer NF.

4.X.Y.1 Energy related information exposure

The procedure is used by the AF or 5GC NF to subscribe to notifications and to explicitly cancel a previous subscription for Energy-Related Information exposure.

Figure 4.x.y.1-1: Energy-related Information exposure

0.	An AF sends a request to (un)subscribe to NEF in Nnef_EventExposure_Subscribe to request the energy related information exposure. Following information may be included:

-	Requested Energy related parameter(s), which indicates what Energy related parameters are requested for the required granularities, the Energy related parameters are as defined in clause 5.X.Y of TS 23.501 [2];

-	The granularity of the requested Energy related parameter(s), i.e., per UE, per PDU Session, per QoS flow, per application;

-    Time window for the energy collection and caculation.

1a. 	If the energy related information exposure subscription from AF is authorized by the NEF, the NEF subscribes the energy information exposure services of EIF by sending Neif_EnergyInfoExposure_Subscribe, including the information in step 0.

1b. Other 5GC NFs may also subscribes the energy information exposure services of EIF by sending Neif_EnergyInfoExposure_Subscribe.

2a. [Conditional] Based on the subscription in step 1a, after the calculation and determination of the requested energy related information in the EIF, the EIF notifies the requested energy related information to the NEF via Neif_EnergyInfoExposure_Notify.

2b. [Conditional] Based on the subscription in step 1b, after the calculation and determination of the requested energy related information in the EIF, the EIF notifies the requested energy related information to other 5GC NFs via Neif_EnergyInfoExposure_Notify.

3.	[Conditional] After step 2a, the NEF sends the notification received from the EIF to the AF via Nnef_EventExposure_Notify.

4.X.Y.2 Energy related information collection

Figure 4.X.Y.2-1 shows the procedure for EIF to collect the Energy related information. The EIF may trigger the following procedure after receiving energy related information exposure subscription as shown in step 1a and 1b in Figure 4.x.y.1-1.

Figure 4.X.Y.2-1: Energy related information collection

0. The EIF may receive the energy related information exposure subscription as shown in step 1a and 1b in Figure 4.x.y.1-1.

1.	The EIF invokes Nudm_UECM_Get service operation to retrieve the SMF ID for the Target of reporting (e.g., UE ID, PDU session ID).

The UDM provides a Nudm_UECM_Get response with the corresponding SMF ID.

2.  The EIF sends the Nsmf_EventExposure Subscription request to the SMF to request the data volume of the required granularity for the time window.

3.	The SMF selects the UPFs and requests the UPFs to report the data volume of the required granularity for the time window.

4. The SMF notifies the data volume to EIF by invoking Nsmf_EventExposure_Notify service operation, providing the data volume of the required granularities in each UPF, UPF ID(s) and RAN IDs.

NOTE 1:	The SMF can derive RAN IDs.

5.   The EIF sends the request to the OAM with providing RAN IDs and UPF ID(s) to retrieve the Node-level energy consumption information and Node-level data volume for the time window.

After the calculation and determination of the energy information in the EIF, the EIF may report the energy information required to the Consumer NF via Neif_EnergyInfoExposure_Notify as described in step 2a and step 2b in figure 4.X.Y.1-1.

# * * * * Second change (all text new) * * * *

5.2.X	EIF Services

5.2.X.1	General

The following table shows the EIF Services and EIF Service Operations.

Table 5.2.X.1-1: NF services provided by the EIF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Neif_EnergyInfoExposure | Subscribe | Subscribe/Notify | NEF, PCF, AF |
|  | Unsubscribe | Subscribe/Notify | NEF, PCF, AF |
|  | Notify | Subscribe/Notify | NEF, PCF, AF |

5.2.X.2	Neif_EnergyInfoExposure service

5.2.X.2.1	General

Service description: The service provides the energy related information to the consumer NFs. There are several operations for this service:

-	Notifying the energy related information to other NFs.

-	Allow consumer NFs to subscribe and unsubscribe for energy related information.

The following service operations are defined for the Neif_EnergyInfoExposure service:

-	Neif_EnergyInfoExposure_Subscribe.

-	Neif_EnergyInfoExposure_Unsubscribe.

-	Neif_EnergyInfoExposure_Notify.

Editor's note:	Whether and what other EIF service operations need to be defined is FFS.

5.2.X.2.2	Neif_EnergyInfoExposure_Subscribe service operation

Service operation name: Neif_EnergyInfoExposure _Subscribe

Description: the consumer subscribes to receive the energy related information from the network, or the subscription is updated if the same subscription is already existing in EIF.

Inputs, Required: Requested Energy related parameter(s) (e.g., Energy Consumption information) as defined in clause 5.X.Y of TS 23.501 [2], Reporting granularity (i.e., per UE, per PDU Session, per QoS flow, per application), Reporting mode (e.g., periodic reporting), Target of Reporting (e.g., SUPI, UE address(es), S-NSSAI, Flow description(s), Application Identifier), Notification Target Address (+ Notification Correlation ID), Time window.

Inputs, Optional: External Application Identifier(s), Reporting periodicity, Indication on supported policy control or actions for network energy saving, Subscription Correlation ID (in the case of modification of the event subscription), Expiry time.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding energy related information report is included, if available.

5.2.X.2.3	Neif_EnergyInfoExposure_Unsubscribe service operation

Service operation name: Neif_EnergyInfoExposure_Unsubscribe

Description: the NF consumer deletes a subscription if already defined in EIF.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

5.2.X.2.4	Neif_EnergyInfoExposure_Notify service operation

Service operation name: Neif_EnergyInfoExposure_Notify

Description: EIF reports the energy related information to the consumer that has previously subscribed.

Inputs, Required: Energy related information as required granularities, Notification Correlation Information, time stamp.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * End of changes * * *