3GPP TSG-WG SA2 Meeting #166	S2-2412514

Orlando, USA, November 18 - 22, 2024	(revision of S2-2411666)

Source:	Ericsson

Title:	KI#3 Conclusion Update on AIoT Services supported by 5GC

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: This pCR discuss and update KI#3 conclusion on the clause of AIoT services supported by 5GC, about enabling temporarily disabled devices.

1. Introduction

In SA2#165, there are some discussions about the Ambient IoT services supported by 5GC. The interim conclusions are agreed as below:

The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device.

-	Write: Request to write information to an AIoT Device.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently or temporarily disabled.

-	Enable: request to enable a temporarily disabled AIoT Device.

Editor's note:	It is FFS whether and how to support enabling temporarily disabled AIoT devices.

There are some comments on that for those disabled AIoT devices, they cannot be enabled again, as they are not expected to receive RF signals.

In clause 5.2.3 of TS 22.369, the management requirements for AIoT devices are specified:

The 5G system shall support a mechanism to:

- 	disable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently able to transmit RF signals

- 	enable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently disabled to transmit RF signals

Based on operator policy, the 5G system shall provide a suitable mechanism to permanently disable the capability of an Ambient IoT device or a group of Ambient IoT devices to transmit RF signals.

[Observation-1] 5GS needs to support enable and disable functions for AIoT Devices. 5GS needs to support permanently disable functions for AIoT Devices.

[Observation-2] When AIoT devices are temporarily disabled, their capabilities to transmit RF signals are disabled.

[Proposal-1] Temporarily disabled devices can be enabled again. Permanently disabled devices cannot be enabled again.

[Proposal-2] For temporarily disabled devices, their capabilities to receive RF signals are not disabled so that they can be enabled again. For permanently disabled devices, it is safe to disable their capabilities to receive RF signals, and it is also up to device implementation.

To enable those temporarily disabled devices, from SA2 point of view, AF can send the command towards CN, and CN delivers the command towards devices via readers.

In TR 38.769, according to the agreements from RAN2, the command from upper layer can be delivered in “inventory and command” and “command-only” manners from readers, as “inventory-only” is expected to be used for inventory use case.

According to our understanding, in command delivery requires the encryption to be performed in both DL (i.e. command message) and UL (i.e. execution result).

- The “command-only” manner requires the security context between the AIoT device and the CN to be pre-established (e.g., during inventory handling, security parameter exchange can be achieved after the AIoT device responds its device ID). It also requires the AIoT device needs to be able to store the security context in NVM.

- The “inventory-command” manner does not require the pre-establishment of the security context. The command delivery is split into inventory round and command round. In inventory round, there can be some security parameter exchange. And then in the command round, the command message and the execution result can be executed, using those security parameters.

According to those two command delivery manners, there is some feasibility analysis below:

- Command-only:

For command-only, there are no issues, if the Enable command is delivered together with the A-IoT paging message. The temporarily disabled device can identify itself as the target device of the Enable command, so that it can enable itself again by executing the Enable command.

The figure above depicts the high-level flows of the interactions among AIoT device, Reader and CN for the command-only case, which is for illustrative purpose:

- Device is temporarily disabled by a Disable command.

- CN receives the Enable request from AF and sends the Enable command towards Reader with the target device ID.

- Reader sends A-IoT Paging message with the target device ID and the Enable command.

- The temporarily disabled device identify itself as the target device for the A-IoT Paging, and then it can execute the Enable command.

- The AIoT device sends the result of Enable command to the Reader.

- The reader forwards the result towards the CN, so that CN is aware of the updated the device status.

[Conclusion-1] For command only case, the temporarily disabled devices can be enabled.

- Inventory and command:

In inventory round, the device cannot differentiate whether the inventory is caused by a normal inventory or Enable command delivery.

For a temporarily disabled device, even if it identifies itself as the target device from the A-IoT paging, it should not respond. According to requirements, its capability of transmitting RF signals is disabled at this moment. Without the device ID response, CN has no chance to trigger the command round to deliver the Enable command towards the device.

To enable the temporarily disabled device to respond the A-IoT paging for the Enable command, an indicator needs to be introduced to ask the temporarily disabled devices to respond in the inventory round.

The figure above depicts the high-level flows of the interactions among AIoT device, Reader and CN for the inventory and command case, which is for illustrative purpose:

- Device is temporarily disabled by a Disable command.

- CN receives the Enable request from AF and triggers Inventory Request towards the reader with the target device ID and a temporarily disabled device respond indicator.

- Reader sends A-IoT Paging message with the target device ID and the temporarily disabled device respond indicator.

- The temporarily disabled device identify itself as the target device for the A-IoT Paging and understands that temporarily disabled devices need to respond.

- The temporarily disabled device responds device ID for the A-IoT paging.

- The reader forwards the device ID towards the CN

- CN sends the Enable command towards the Reader for the responded device.

- The reader delivers the Enable command towards the device.

- The device executes the Enable command.

- The AIoT device sends the result of Enable command to the Reader.

- The reader forwards the result towards the CN, so that CN is aware of the updated the device status.

[Conclusion-2] For inventory and command case, with the introduction of an indicator to require those temporarily disabled device to respond A-IoT paging in inventory, the temporarily disabled devices can be enabled as well.

[Proposal-1] Resolve the EN.

[Proposal-2] For inventory and command case, an indicator to require those temporarily disabled device to respond A-IoT paging in inventory needs to be introduced and requires confirmation from RAN2/RAN3. Add an EN to highlight the dependency.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.1.0:

* * * Start of Change * * * *

### 8.3.2	AIoT services supported by the 5GC

The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device.

-	Write: Request to write information to an AIoT Device.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently or temporarily disabled.

-	Enable: request to enable a temporarily disabled AIoT Device. To enable temporarily disabled AIoT Devices when inventory and command delivery applies, an explicit indicator which enables temporarily disabled devices to respond in A-IoT paging, or an implicit indicator (i.e., the individual device ID is only presented in A-IoT paging when the temporarily disabled devices is to be enabled), is needed.

NOTE: 	The security aspect of Enable and Disable is studied and to be concluded by SA3.

Editor's note:	The explicit and implicit indicator require the confirmation from RAN2, and it is up to SA3 to determine what kind of individual device ID can be used as the implicit indicator which can address security/privacy requirements.

* * * End of Change * * * *