3GPP TSG-WG SA2 Meeting #166	S2-2412518

# Orlando, USA, November 18 – 22, 2024		(revision of S2-2412094)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5739 | rev | 8 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: UE subscription and policy control for energy efficiency and energy saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, [vivo, ZTE, Samsung, NEC, Huawei, China Mobile, MediaTek Inc., China Unicom, TOYOTA, China Telecom, CATT, Deutsche Telekom, LG Electronics] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the SA2#165 meeting the CR was agreed proposing a new “Indication for network energy saving”. For the Subscription data, there are following Editor’s Notes: Editor’s note: How the indication of network energy saving is used in the network is FFS. Editor’s note: Details and granularities of the indication (e.g. per UE, per slice) is FFS. For the policy control, there are the following Eitor’s Note: Editor’s note: The notification for the energy related information, if any, should not be provided from the UDM/UDR. The details of notification for the energy related information, e.g. what the notification would be, or which entity (e.g. EIF) provides the notification, is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to introduce the following subscription parameters and how they are used for policy control: - “Indication for network energy saving” is on per UE level and it is received by the AMF during the UE subscription data retrieval. The AMF forwards the parameter to the NG-RAN and to the PCF. The PCF may subscribe with the EIF for notifications. Upon notification by the EIF, the PCF updates the AM policies for the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The subscription and policy control for energy efficiency and energy saving is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y (new), 5.X.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | S2-2410192, S2-2410752, S2-2410801 |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.X.Y	Subscription information aspects

The Subscription data for a UE may include a Energy Sacing indicator that the UE is subject to network energy saving operation. The AMF receives this indication in the UE Subscription data. The AMF forwards the indication to the NG-RAN and to the PCF.

Editor’s note: Details and granularities of the indication (e.g. per UE, per slice) is FFS.

Editor’s note: How the indicatior of network energy saving is used in the RAN is FFS.

Editor’s note: The details of how the energy-related data is stored in the UDR and  whether additional parameters related with energy can be added in the UE subscription and the further details on the usage are FFS.

### 5.X.Z	Policy control aspects

During AM policy association establishment procedure, the AMF sends to the PCF the Energy Saving indicatoras described in clause 5.x.y. The PCF may subscribe with the EIF for energy-related notifications. Based on  this indicator and potential energy related notifications from the EIF, the PCF may make policy decisions reusing the existing parameters based on the operator policy. For example, the PCF may change the RFSP for the UE or the UE-AMBR for the UE in order to achieve anergy saving as described in 23.503 [45].

Editor’s note: Further details of possible policy decisions are FFS.

# * * * End of changes * * * *