3GPP TSG-WG SA2 Meeting #166	S2-2412548

Orlando, USA, November 18 - 22, 2024	(revision of S2-2412504, S2-2411665)

Source:	Ericsson, Samsung

Title:	KI#1 Conclusion Update on Architecture to Support Topology 2

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Perform evaluation of solutions for Ambient IoT Topology 2 (i.e., among RRC-based option, NAS-based option and user-plane option) and update the interim conclusions.

1. Introduction

In SA2#165, in interim conclusion for topology 2, user-plane option and RRC-based option are included, while NAS-based option is left as an EN as specified in clause 8.3.1.1:

Two options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

Editor's note:	Whether a NAS-based option is needed in addition is FFS.

[Proposal-1] In our view, SA2 needs to make a thorough evaluation among all three options. Update the clause to make RRC-based options complete.

In the protocol stack for the Indirect Path, there is an AIoT App layer between the AIoT device and the AF. According to the agreed clause 8.3.2 “AIoT services supported by the 5GC”, only inventory/read/write/enable/disable service operations are provided. There is no service operation defined for the transparent container delivery between the AF and the AIoT device. It also indicates there is no AIoT App layer between the AIoT device and the AF. Instead, AIoT Device NAS Layer is needed between the AIoT device and the AIOTF.

[Proposal-2] Update the protocol stack for the Indirect Path. Remove the App layer and add AIoT Device NAS layer.

In clause 6.4.2.1 “Solutions for Topology 2” of RAN TR 38.769, the three options are specified as below:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

[Observation-1] From RAN3 perspective, there are aspects of A-IoT CN to A-IoT-enabled UE communication that are transparent to the A-IoT enabled gNB; besides, the NAS- and UP-based options would not allow the A-IoT enabled gNB to recognise an ongoing communication between the A-IoT CN and the A-IoT-enabled UE.

RAN3 also agreed on the following architectural principle at RAN3#125bis:

The RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers, for both T1 and T2.

The definitions of the 3 solutions defined in RAN3 as outlined above and their respective depiction in terms of protocol stacks alone do not and cannot detail the way how RAN can enable such coordination function, as they only cover the aspect AIoT upper layer information exchange between the UE Reader and the 5GC.

[Observation-2] From SA2 perspective, the interim conclusions have to be complemented by yet to be determined RAN and SA3 aspects.

In interim conclusions for KI#3, AIoT services supported by 5GC include inventory, read, write, disable, enable. For these four AIoT services, the CN needs to receive the requests from AF, and further deliver them to AIoT devices via the readers. In this case, an AIoT Device NAS layer is needed between AIoT Device and CN (i.e., AIOTF).

[Proposal-3.1] The “A-IoT upper layer information” includes AIoT Device NAS message (between AIoT device and AIOTF).

RAN2 also agreed on the assistant information from CN to the reader and is checking with SA2 about it. The assistant information may include D2R message size, AIoT service type, targeted for one or more than one devices, approximate number of target devices. Depends on SA2 discussion, some of them will be provided by CN to the readers, while some of them may not.

It has to be noted that RAN2 hasn’t discussed which of the assistant information is mandatory, which is optional. Some assistant information is not absolutely necessary for a certain communication exchange scheme to work, but is rather provided for optimisation reasons.

[Proposal-3.2] The “A-IoT upper layer information” may be amended by assistant information, if available.

To deliver the “A-IoT upper layer information” from CN towards UE readers (A-IoT-Enabled UEs), the different options foresee “AIoT upper layer information” to take different paths:

- The User-plane option use the PDU session user plane to deliver the information (AIOTF -> UPF -> NG-RAN -> UE reader). UPF and NG-RAN are kept agnostic towards this information (it doesn’t even realize that such communication is ongoing). AIOTF would need to duplicate the information towards each involved UE reader.

- The NAS-based option delivers the information over control plane, i.e. information delivery is not bounded to specific PDU session (AIOTF -> AMF -> NG-RAN -> UE reader). NG-RAN is agnostic towards the information (it doesn’t even realize that such communication is ongoing). AIOTF would need to duplicate the information towards each involved UE reader.

- The RRC-based option delivers the information over NGAP (AIOTF -> NG-RAN -> UE reader, AMF is involved in indirect path option). NG-RAN is agnostic towards the A-IoT upper layer information as such but can be aware of the kind of AIoT transaction and also receives session specific assistance information. NG-RAN is responsible for duplicating respective information towards each involved UE reader.

Architecture wise, it can be stated that the RRC based solution aligned with topology 1. That is, CN interacts with AIoT RAN node function issuing inventory and command service requests, according to the system architecture definition of A-IoT RAN. From SA2 point of view, it appears as if the RRC based solution is more efficient and allows architectures for topology 1 and 2 to align.

Still, there are aspects that have to be studied and concluded together with RAN WGs.

[Proposal-3.3] Different options take different paths for “A-IoT upper layer information” delivery from CN to A-IoT-enabled UEs. There are RAN specific aspects that cannot be concluded by SA2 alone.

It has already been agreed in RAN2 that the network is in control of AIoT radio resources to be used by UE readers for topology 2.

For radio resource allocation, the A-IoT-enabled gNB cannot allocate radio resource without receiving timely information about the AIoT session causing the need for AIoT radio resources. The AIoT-enabled gNB needs some knowledge of the upcoming/ongoing A-IoT Service Request e.g., whether the A-IoT CN node (or AF) is interested in performing inventory or command, the area within which the AIoT session is to be executed, amended by assistance information e.g. approximate number of target devices (to allocate radio resources accordingly) if available, etc.

In this section, the protocol means by which the AIoT enabled gNB receives such information is referred as “A-IoT session resource control information”, which serves as a trigger to the AIoT RAN node functions for AIoT radio resource management towards A-IoT-enabled UEs.

It needs to be noted that the A-IoT session resource control information is not the same as the assistant information. To allocate radio resources in gNB and to perform A-IoT service request in UE readers, there are common information needed, e.g., service type, target to one or more devices, approximate numbers of target devices if available, etc. However, gNB needs further information from an A-IoT session perspective, instead only from an individual reader’s perspective, e.g., target area information, session (transaction) ID, number of readers determined, etc.

[Proposal-4] A-IoT session resource control information (to gNBs) is not the same as assistant information (to readers). gNB needs A-IoT session resource control information as a trigger to start radio resource preparation and further coordination of radio resources.

Overall system design for deciding how to trigger AIoT radio resource management is neither a pure RAN WGs’ task nor a pure SA2 task. It requires collaboration between SA2 and RAN WGs.

[Proposal-5] Proper design work for means to enable “A-IoT session resource control information” delivery towards A-IoT-enabled gNB is paramount for finalising the study especially on the UP and NAS option. This requires collaboration with RAN WGs for evaluation and conclusion.

NOTE: The figures in this section are for illustrative purpose, which focus on how the A-IoT upper layer info and A-IoT session resource control info are delivered. The response messages are omitted.

RRC-based option: A-IoT-enabled gNB receives A-IoT session resource control information from CN.

The AIOTF (for Indirect Path, via the AMF) triggers the setup of AIoT session resource to the A-IoT enabled gNB for a particular AIoT transaction, along with the A-IoT upper layer information which is to be forwarded A-IoT-enabled UEs.

Figure 1. RRC Option: CN -> A-IoT enabled gNB, per A-IoT session

NAS-based option: A-IoT-enabled gNB can receive A-IoT session resource control information CN.

The AIOTF delivers A-IoT upper layer information to the A-IoT-enabled UE over NAS, transparently to the A-IoT-enabled gNB. To trigger the setup of AIoT session resources some communication needs to be established towards the A-IoT enabled gNB in addition.

For the A-IoT session resource control information delivery from CN to A-IoT-enabled gNB, there are two options:

- It is straight forward to assume the network to trigger AIoT session resource control (in analogous way as e.g. PDU Session resources are setup). It can be triggered by a separate N2 message towards A-IoT-enabled gNB, to let gNB be prepared for the radio resource allocation. Such request is performed on a per AIoT session basis.

Figure 2-1. NAS-based Option: CN -> A-IoT enabled gNB, per A-IoT session

- The A-IoT session resource control information can also be delivered along with the NAS message containing A-IoT upper layer info. And this delivery will be per UE. And gNB should only setup radio resource when receiving the A-IoT session resource control info of A-IoT Session for the first time.

NAS-based option: A-IoT-enabled gNB receive radio resource request from A-IoT-enabled UE (no A-IoT session resource control information).

In the UE request option, A-IoT-enabled UE can derive the radio resource needed from the A-IoT upper layer information received. gNB can also allocate radio resource per UE request.

Figure 2-2. NAS-based Option: A-IoT-enabled UE -> A-IoT-enabled gNB, per UE

User-plane option: A-IoT-enabled gNB can receive A-IoT session resource control information from CN.

The AIOTF delivers A-IoT upper layer information to A-IoT-enabled gNB over PDU session user plane. The additional A-IoT session resource control information can be delivered over the control plane or the user plane as two options:

- In the CP option, the A-IoT session resource control information is delivered as a separate N2 message towards A-IoT-enabled gNB, to let gNB be prepared for the radio resource allocation, which can be per A-IoT session.

Figure 3-1. User-plane Option: CN -> A-IoT enabled gNB over CP, per A-IoT session

- In UP option, the A-IoT session resource control information is sent along with the A-IoT upper layer information. To enable gNB being able to process such information, the information can be included in extended GTP-U headers. Such delivery will be per UE.

Figure 3-2. User-plane Option: CN -> A-IoT enabled gNB over UP, per UE

User-plan option: A-IoT-enabled gNB receive radio resource request from A-IoT-enabled UE (no A-IoT session resource control information).

In the UE request option, A-IoT-enabled UE can derive the radio resource needed from the A-IoT upper layer information received. gNB can also allocate radio resource per UE request.

Figure 3-3. User-plane Option: A-IoT-enabled UE -> A-IoT-enabled gNB, per UE

[Proposal-6] Include those options in the interim conclusion for the three architecture options.

- RRC-based option: A-IoT session resource control information over NGAP from CN (per session)

- NAS-based option:

- A-IoT session resource control information in a separate N2 message from CN (per session)

- A-IoT session resource control information along with NAS message from CN (per UE)

- No A-IoT session resource control information, radio request from UE (per UE)

- User-plane option:

- A-IoT session resource control information in a separate N2 message from CN (per session)

- A-IoT session resource control information in extended GTP-U headers from CN (per UE)

- No A-IoT session resource control information, radio request from UE (per UE)

NOTE: In all options, it does not preclude that A-IoT-enabled UE can request A-IoT-enabled gNB to further adjust radio resources.

In user-plane option, the A-IoT session resource control information can be included in extended GTP-U headers, so that gNB can get the information from the GTP-U headers.

But GTP-U header extension is not flexible. It further requires the user plane path between the UPF and the AIOTF to be GTP-U based (similar as N3/N9), but not IP based (similar as N6), which brings much more complexities to the solution.

[Proposal-7] It is proposed not to pursue the extend GTP-U header option for user-plane option.

In both NAS-based option and user-plane option, A-IoT-enabled gNB can allocate radio resources based on UE request, without the A-IoT session resource control information.

But in this way, the gNB can only passively allocate radio resource based on the request from the UEs. It cannot coordination radio resource among the UEs effectively without the full picture of the needs of the A-IoT session.

Also, without the information from the CN, if the authorized UE sends a fake request towards gNB to allocate radio resource (e.g., the UE request the resource for a non-existent inventory), gNB cannot determine to reject the fake request.

Unlike the sidelink communication, for AIoT communications in Rel-19, the AIOTF triggers each A-IoT session. It is desired to let the AIOTF also inform the gNB via some means.

[Proposal-8] It is proposed not to pursue UE request option for NAS-based option and user-plane option.

[Proposal-9] Based on the analysis above, when determining architecture options, the delivery of the A-IoT session resource control information needs to be taken into account. As the radio resource allocation is under the remit of RAN WGs, SA2 needs to conclude together with RAN WGs.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.1.0:

* * * Start of Change * * * *

### 8.1.3	Architecture to Support Topology 2

#### 8.1.3.1	General

Two options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

A-IoT session resource control information contains the information needed by A-IoT-enabled gNB to enable the coordination of the usage of the A-IoT radio resources among readers, which is provided by CN. It includes the A-IoT session related information, e.g., target area information, session ID, service type, target to one or more devices, approximate number of target devices if available, etc.

Editor's note:	The details of A-IoT session resource control information are to be determined by RAN2 and RAN3.

For the A-IoT session resource control information delivery, the options for each architecture solution are listed as below:

-	RRC-based option: the A-IoT session resource control information is delivered over NGAP from the CN.

-	User-plane option: the A-IoT session resource control information can be delivered via a separate N2 request from CN, or in extended GTP-U headers along with the A-IoT upper layer information from CN. As an alternative, UE can request radio resource but gNB does not have A-IoT session resource control information.

NOTE:	In all options, it does not preclude that A-IoT-enabled UE can request A-IoT-enabled gNB to further adjust radio resources.

Editor's note:	The options are to be evaluated within the architecture option evaluation and conclusion.

* * * End of Change * * * *