3GPP TSG-WG SA2 Meeting #166 	S2-2412574

Orlando, USA, November 18 – 22, 2024	(revision of S2-2412041)

# Title:	[DRAFT] LS Reply on LP-WUS subgrouping

# Response to:	(S2-2411297/R2-2409226) LS on LP-WUS subgrouping

# Release:	Rel-19

# Work Item:	NR_LPWUS-Core

Source:	[Huawei to be] SA2

To:	RAN2, RAN3, CT1

Cc:

Contact Person:

Name:	Steven Wenham

Tel. Number:

E-mail Address:	steven DOT wenham AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR5824 (Rel-19 TS 23.501) and CR5131 (Rel-19 TS 23.502)

1. Overall Description:

SA2 thanks RAN2 on the progress and conclusion of using PEI subgrouping method as baseline for LP-WUS subgrouping as described in your LS in R2-2409226.

SA2 has agreed to apply the existing CN based PEI subgrouping mechanism for the CN based LP-WUS subgrouping in the attached CRs for TS 23.501 and TS 23.502.

Additionally, SA2 discussed the following aspects:

1) 	SA2 noted that the name “LP-WUS” can be mis-understood, as SA2 understands that the Wake Up Signal is transmitted with high power to enable a large coverage area, and low power refers to a low power wake up receiver used in the mobile.

2) 	Similar to RedCap and eRedcap, when Initial Registration is performed with a pre-Rel-19 gNB (or a later gNB that does not support LP-WUS) and which does not know how to process the UE Radio Access Capabilities in order to generate the LP-WUS, related paging information (i.e. number of RX, half duplex) in the TS 38.331 UERadioPagingInformation that is sent in TS 38.413 signalling messages.

An approach discussed in SA2 was LP-WUS (or RedCap/eRedCap as appropriate) capable gNB could analyse the UE Radio Capability for Paging Information that is sent in every NG-AP INITIAL CONTEXT SETUP REQUEST message to check in the UE Radio Access Capabilities that the UE Radio Capability for Paging Information contains the relevant information, and if it does not.

If a gNB determines the relevant information is missing then the gNB could upload updated information to the AMF (so that they can then be used in subsequent NG-AP Paging messages).

SA2 requests RAN3 to consider the above scenario and provide feedback.

2. Actions:

To RAN2, RAN3, CT1 group.

ACTION: 	SA2 asks RAN2, RAN3 and CT1 group to take the above into account and provide feedback if any.

ACTION: 	SA2 asks RAN3 to take the above into account and provide feedback on updating UE Radio Capability for Paging Information.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #166-Ad Hoc-e	Jan 20-24, 2025				Online

TSG-SA2 Meeting #167   	Feb 17 – 21, 2025				Athens, GR