3GPP TSG-WG2 Meeting #166	S2-2412884

# Orlando, USA, November 18-22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0489 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS Handling for multi hop U2N without N3IWF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, FirstNet, AT&T, Huawei, HiSilicon, China Telecom, InterDigital, Qualcomm, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Principles for QoS handling of multi-hop UE-to-Network Relays are agreed in the conclusions of the TR 23.700-03 as following: - End-to-end QoS management for multi-hop U2N Relays is done similarly to the end-to-end QoS management for single hop L3 U2N Relay as defined in TS 23.304 [4], with enhancement to handle QoS split over multiple legs of the PC5 interface. - The QoS flows setup can be initiated by the network or the 5G ProSe Layer-3 Remote UE during Layer-2 link establishment or link modification procedure. U2N Relay responsible for determining end to end QoS and PDB division. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause for QoS handling for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay without N3IWF is included. For Network initiated QoS Flow est.: - The U2N Relay determines the e2e PC5 QoS requirement based on Uu QoS parameters from SMF; - U2N Relay determines the max PC5 PDB; - U2N Relay determines the per hop PC5 QoS Flow parameters to be applied next hop. - U2N Relay provides e2e PC5 QoS, max PC5 PDB, per hop PC5 QoS Flow parameters to next hop IM Relay; - IM Relay determines per hop PC5 QoS Flow parameters based on e2e PC5 QoS and max PC5 PDB, and the per hop PC5 QoS Flow parameters cannot be higher than max PC5 PDB. - This goes on till the message reaches Remote UE. For Remote UE initiated QoS Flow: - Remote UE provides e2e PC5 QoS Info; - IM Relay forwards upstream to the U2N Relay. - U2N Relay performs the above parameter determinations and sends info in the Accept message downstream. - IM perform the same as above procedure, until reaching the Remote UE. - U2N Relay also initiates PDU session modification towards SMF to update/request Uu QoS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoS handling for Multi-hop Layer-3 UE-to-Network Relay without N3IWF to support conclusions defined in the TR 23.700-03 is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 5.6.2.x	QoS handling for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay without N3IWF

5.6.2.x.1	General

End-to-end QoS management for Multi-hop UE-to-Network Relays is done similarly as QoS management for single hop Layer-3 UE-to-Network Relay as defined in clause 5.6.2.1, with enhancement to handle QoS split over multiple legs of PC5 interface.

As shown in figure 5.6.2.x-1 below, the end-to-end QoS can be met only when the QoS requirements are properly translated and satisfied over the multiple legs respectively.

Figure 5.6.2.x-1: End-to-End QoS translation for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay operation

To achieve this, the QoS mapping can be pre-configured or provided to the 5G ProSe Layer-3 UE-to-Network Relay by the PCF using Prose Policy as specified in clause 5.1.4.1. When 5G ProSe Layer-3 UE-to-Network Relay conducts the mapping between 5QI and PQI, it may use the mapping information as specified in clause 5.6.2.1, with multi-hop adjustment taking into account the number of hops to the 5G ProSe Layer-3 Remote UE.

The 5G ProSe Layer-3 UE-to-Network to derive the maximum packet delay budget that applies to the PC5 link over each adjacent hop from the end-to-end QoS info based on a hop adjustment factor, e.g. 1/5 of the standardized PDB value of the PQI for hop count =5.

5.6.2.x.2	Network initiated QoS Flows

If the QoS Flows setup are initiated by the network, the SMF can base on the PCC rules or its local configuration to generates the QoS rules and QoS Flow level QoS parameters (e.g. 5QI, GFBR, MFBR) and signal to the 5G ProSe Layer-3 UE-to-Network Relay using PDU Session Establishment/Modification procedure. For the PDU sessions used for relaying, the SMF always provides the QoS Flow level QoS parameters to the 5G ProSe Layer-3 UE-to-Network Relay when the network triggers establishement of a QoS Flow.

Based on the received QoS parameters from SMF, the 5G ProSe Layer-3 UE-to-Network Relay decides the end-to-end PC5 QoS parameters for the corresponding PC5 QoS Flow based on the QoS mapping as defined in clause 5.6.2.1, taking into account the number of hops to the 5G ProSe Layer-3 Remote UE. The end-to-end PC5 QoS parameters are interpreted as the end-to-end QoS requirements for the traffic transmission between 5G ProSe Layer-3 Remote UE and the 5G ProSe Layer-3 UE-to-Network Relay. The 5G ProSe Layer-3 UE-to-Network Relay also decides the maximum PC5 PDB based on the end-to-end PC5 QoS parameters and the number of hops to the 5G ProSe Layer-3 Remote UE. The 5G ProSe Layer-3 UE-to-Network Relay determines the PC5 QoS Flow parameters, based on the end-to-end PC5 QoS parameters, hop adjustment factor and hop info (e.g. hop count or path info). The 5G ProSe Layer-3 UE-to-Network Relay provides the end-to-end PC5 QoS info, the maximum PC5 PDB, and the generated PC5 QoS Flow parameters to the next hop Iintermediate UE-to-Network Relay towards the 5G ProSe Layer-3 Remote UE, using layer-2 link modification procedure.

The Intermediate UE-to-Network Relay(s) applies the received PC5 QoS Flow parameters on the upstream PC5 link towards the parent relay. The Intermediate UE-to-Network Relay(s), based on its implementation, determines the PC5 Flow QoS parameters for next hop PC5 link towards the 5G ProSe Layer-3 Remote UE, based the received end-to-end PC5 QoS info, maximum PC5 PDB, hop adjustment factor and hop info (e.g. hop count or path info). The PDB for the PC5 QoS Flow parameters needs to be lower than the maximum PC5 PDB. The GFBR and MFBR values for the next hop PC5 GBR QoS Flow are set equal to the GFBR and MFBR values of the received end-to-end PC5 QoS info respectively.

Then the Intermediate UE-to-Network Relay(s) provides the end-to-end PC5 QoS info, the maximum PC5 PDB, and the generated PC5 QoS Flow parameters to the next hop Iintermediate UE-to-Network Relay towards the 5G ProSe Layer-3 Remote UE, using layer-2 link modification procedure, until it reaches the 5G ProSe Remote UE.

5.6.2.x.3	Remote UE initiated QoS Flows

If the 5G ProSe Layer-3 Remote UE initiates PC5 QoS Flows setup or modification during the Layer-2 link establishment or modification procedure, the 5G ProSe Layer-3 Remote UE provides the QoS Info as described in clause 6.4.3.6 to its connected Intermediate UE-to-Network Relay. The PC5 QoS parameters of the QoS Info (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.) sent by the 5G ProSe Layer-3 Remote UE are interpreted as the end-to-end QoS requirements for the traffic transmission between 5G ProSe Layer-3 Remote UE and UPF. The intermediate UE-to-Network Relay(s) forwards the received QoS info to the 5G ProSe Layer-3 UE-to-Network Relay, using Layer-2 link establishment or modification procedure.

Based on the received end-to-end QoS parameters from the 5G ProSe Layer-3 Remote UE via intermediate UE-to-Network Relay(s), the 5G ProSe Layer-3 UE-to-Network Relay determines the 5QI for the Uu QoS control and the PQI for end-to-end PC5 QoS control based on the QoS mapping as defined in clause 5.6.2.1 it may also do mapping adjustment by implementation taking into account the number of hops to the 5G ProSe Layer-3 Remote UE.

The 5G ProSe Layer-3 UE-to-Network Relay also determines the maximum PC5 PDB and PC5 QoS Flow parameters for the PC5 link with the next hop Intermediate UE-to-Network Relay towards the 5G ProSe Layer-3 Remote UE, as described in clause 5.6.2.x.2. The 5G ProSe Layer-3 UE-to-Network Relay provides the end-to-end PC5 QoS info and the determined maximum PC5 PDB and PC5 QoS Flow parameters to the next hop Intermediate UE-to-Network Relay towards the 5G ProSe Layer-3 Remote UE, as part of the Accept message for the Layer-2 Link establishment or modification procedure.

The Intermediate UE-to-Network Relay(s) applies the received PC5 QoS Flow parameters in the Accept message from the 5G ProSe Layer-3 UE-to-Network Relay or its parent Intermediate UE-to-Network Relay over the PC5 link.

The Intermediate UE-to-Network Relay(s) decides the PC5 QoS Flow parameters for the next hop PC5 link towards the 5G ProSe Layer-3 Remote UE as described in clause 5.6.2.x.2 and provides the end-to-end PC5 QoS info, maximum PC5 PDB,  and the PC5 QoS Flow parameters to its connected next hop Intermediate UE-to-Network Relay towards the 5G ProSe Layer-3 Remote UE as part of the Accept message for the Layer-2 Link establishment or modification procedure, until reaching the 5G ProSe Layer-3 Remote UE.

The 5G ProSe Layer-3 UE-to-Network Relay performs the UE requested PDU session Modification as defined in TS 23.502 [5], clause 4.3.3 for authorizing the requested QoS including the 5QI and the Packet Filters. If the PCF authorizes the requested QoS with a different 5QI value, the 5G ProSe Layer-3 UE-to-Network Relay may further update the PQI value based on the authorized 5QI value and the 5G ProSe Layer-3 UE-to-Network Relay performs the Layer-2 link modification procedure as defined above to update the corresponding PC5 QoS Flow parameters over all the PC5 links towards the 5G ProSe Layer-3 Remote UE.

* * * End of Changes * * *