3GPP TSG-SA2 Meeting #166	S2-2412939

14 - 18 November 2024, Orlando, USA                                 	(revision of S2-2412724)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1104 | rev | 13 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New analytics ID to support Signalling storm Mitigation and Prevention |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SK Telecom, Samsung, Verizon, AT&T, China Mobile, ETRI, KPN N.V., LG Uplus, Rakuten Mobile, ZTE, China Telecom, Nokia, Ericsson, Lenovo, OPPO, NTT DOCOMO, Huawei, HiSilicon, KDDI, vivo, Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 23.700-84 v1.0.0 has documented the conclusion for key issue#4 “NWDAF enhancements to support network abnormal behaviours (i.e. signalling storm) mitigation and prevention”. To solve the issue, the following conclusions are reached: “- By means of statistics and predictions, NWDAF supports assistance to signalling storm mitigation and prevention by statistically learning the expected normal level of signalling and identifying a significant deviation indicating a signalling storm caused by massive signalling of UEs and/or caused by NFs signalling. - A new Analytics ID to support Signalling storm Mitigation and Prevention caused by NFs signalling will be defined. - It will be determined in the normative phase whether to define a new Analytics ID or enhance an existing Analytics ID to support Signalling storm Mitigation and Prevention caused by massive signalling of UEs. - Analytics Filter includes information to assist the NWDAF for the signalling storm as defined above. NOTE 1: The consumers of the analytics ID and the parameters of the analytics request/subscribe (determined by the consumer) are to be specified in normative work.” This contribution provides the analytics detail of the prevent network abnormal behaviour use case based on the new NF’s information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new Analytics ID for the prevention of abnormal NF behaviour causing signalling storm and mitigation of its impact in the network use case in the TS 23.288. - improved SCP inputs and 6.x.4-1 Figure (step2) for clarity. - adjusted the explanation column of some of input data parameters as a result of offline discussion with Vz/SKT, clean up (reduce) SCP occurance in the procedure (as per E//’s request), and self correct heart-beat related information. - updated ‘abnormal operational event’ to ‘unexpected operational status indicator’, and changed descriptions. - NOTE 6 added for explanation of a posterior. - updated external/internal group identifier NOTE, changed NRF/OAM table to remove NRF with NOTE, updated SM/MM for the Output analytics for clarity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified that the procedure for the “Analytics-assisted prevention of abnormal NF behaviour”. It can mislead and result in inconsistent specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.x.1 (new), 6.x.2 (new), 6.x.3 (new), 6.x.4 (new), 7.1, 6.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Merge some contents from the 2410610 |  |  |  |  |  |  |  |  |

# * * * * First change (all new )* * * *

6.x	Signalling Storm Analytics

6.x.1	General

This clause specifies how NWDAF supports analytics for network abnormal behaviours (i.e. signalling storm) mitigation and prevention.

The signalling storm analytics provides analytics information (statistics or predictions) regarding the abnormal signalling. Abnormal signalling is a significant deviations from normal conditions indicating a potential signalling storm. The analytics of signalling storm requires the identification of whether the signalling storm is due to signalling from an NF or massive signalling from UE based on the requested analytics ID.

The NWDAF provides statistics and/or prediction analytics to consumer entity (e.g. SCP) or consumer NFs, e.g. AMF, SMF, UDM, PCF, NRF, OAM and AF.

NOTE 1: When the consumer is SCP, the target NF (i.e. The NF that signalling storm analytics should be provided for) might be sitting either in front of the SCP (egress) or behind the SCP (ingress).

The consumer of these analytics may indicate in the request or subscription:

-	Analytics ID = "Signalling Storm";

-	Target of Analytics Reporting: a UE or a group of UEs or several groups of UEs identified by internal Group ID or SUPI or SUPI range, a list of NFs which is causing the signalling storm;

NOTE:	When an untrusted AF provides a GPSI the NEF maps it into a SUPI and when the untrusted AF provides an External Group Identifier the NEF maps it into Internal Group Identifiers.

- 	Target Cause ID(s): Indicates specific Cause ID(s) as Analytics Output (e.g. signalling storm caused by UEs, signalling storm caused by NF, or all)

-	Analytics Filter Information:

-	NF ID List: instance IDs or NF Set IDs of target NF that signalling storm analytics should be provided for.

-	Area of Interest (AoI) and/or S-NSSAI which restricts the scope of signalling storm analytics to the specific area and/or slice;

-	Analytics Reporting Information:

-	Expected Report Time : indicates the time limit by which the analytics results need to be received., e.g. 10 seconds, 5 minutes, etc;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-	Signalling frequency threshold: indicates the frequency of the received signalling within the time period;

-	UE number threshold: indicates a threshold of the amount of UEs of which the number of one type of request messages meets the frequency of UE requests threshold. For example, if the number of UEs that initiate registration requests exceeds the pre-set rate threshold (10 times per hour) and the number of such UEs exceeding the pre-set rate threshold exceeds 100 (the UE number threshold is set to 100), the NWDAF will provide the analytics.

6.x.2	Input data

The NWDAF collects the signalling feature data, NF context, AF data and MDAF data as listed in Table 6.x.2-1, 6.x.2-2, 6.x.2-3, 6.x.2-4, 6.x.2-5 and 6.x.2-6.

The NWDAF can collect UE behavioural information per UE or group of UEs as specified in clause 6.7.5.2.

The NWDAF may use output of abnormal UE behaviour analytics and UE dispersion analytics as specified in clause 6.7.5 and 6.10.3 as input data for analytics on signalling storm. This information may be collected from a different NWDAF (e.g. distributed architecture).

The NWDAF may collect the subgroup of input data based on implementation.

Table 6.x.2-1: UE related Context Data collection

| Information | Source | Description |
| --- | --- | --- |
| UE ID |  | Identifies a single UE. |
| Signalling feature data |  | NF procedures containing signalling exchange information related to a particular UE or session from the Connection, Registration, Mobility and Session Managements procedures. (NOTE 1) |
| > Request type and number from UE/RAN (0..max) | AMF | Request type of N1 or N2 interface (such as received Initial Registration Request, Mobility and Periodic Registration Request, Service Request, etc) and the number of requests corresponding to the request type. |
| >> Time duration from receiving request from UE/RAN to response to UE/RAN | AMF | Time duration between the request from UE/RAN and response to UE/RAN. |
| >> Number of successful responses of UE/RAN | AMF | Number of successful responses associated to their initial requests, such as Registration Response, etc. |
| >> Number of failed responses of UE/RAN | AMF | Number of failed responses associated to their initial requests, such as Registration Reject, Service Reject, etc. |
| >> Reason of failed responses of UE/RAN | AMF | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| >> A posterior Request type of UE/RAN (0..max) | AMF | A posterior Request types triggered from UE/RAN, for NF Service request, or request to UE/RAN. (NOTE 6) |
| > Request type and number from NF (0..max) | NF | Request type received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc., as well as the number of requests received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc. (NOTE 2)(NOTE 3) |
| >> Time duration from receiving request from NF to response to NF | NF | Time duration between the request from NF and response to NF. |
| >> Time duration of transactions related to SCP ingress interface | SCP | Time duration between the received request from consumer NF and response forwarded by SCP to consumer NF. (NOTE 4) |
| >> Time duration of transactions related to SCP egress interface | SCP | Time duration between the routed/forwarded request to producer NF and response received from producer NF. (NOTE 4) |
| >> Number of successful responses of NF | NF | Number of successful responses associated to their initial requests. (NOTE 2) |
| >> Number of failed responses of NF | NF | Number of failed responses associated to their initial requests. (NOTE 2) |
| >> Number of successful responses related to SCP egress interface | SCP | Number of successful responses received over the egress interface associated to their initial requests. |
| >> Number of failed responses related to SCP egress interface | SCP | Number of failed responses received over the egress interface associated to their initial requests. |
| >> Reason of failed responses of NF | NF | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| >> Reason of failed responses related to SCP egress interface | SCP | Reasons of failed responses over the egress interface associated to their initial requests, e.g. reject, no-response, etc. (NOTE 5) |
| >> Number of redundant signalling of NF | NF | Number of received redundant signalling. The redundant signalling means the signalling which is transmitted multiple times. (NOTE 2) (NOTE 3) |
| >> A posterior Request type of NF (0..max) | NF | A posterior Request types triggered from NF, for NF Service request. (NOTE 3) (NOTE 6) |
| > Public Warning information | AMF | Public Warning information as defined in the TS 23.041[X], such as WRITE-REPLACE WARNING REQUEST, etc. |
| > Frequent Mobility Registration Update | AMF | The number of Mobility Registration Updates N within a period M may be an indication for abnormal ping-pong behaviour, where N and M are operator’s configurable parameters. |
| > Number of receiving Session Report from UPFs | SMF | Number of receive Session Report from UPF triggered by DL packet in case of PDU Session is in 5GCM-idle state. |
| UE Context in NF |  |  |
| > State transition information | AMF, SMF | UE related state transition information such as transition type, frequency of CM state changes etc. State transition identifier: - “Access Type change to 3GPP access”; - “Access Type change to non-3GPP access”; - “RM state change to RM-DEREGISTERED”; - “RM state change to RM-REGISTERED “; - “CM state change to CM-IDLE”; - “CM state change to CM-CONNECTED”; - “Handover”; or - “Mobility Registration Update”. - “Frequent Mobility Registration Update”(table 6.7.2.2-1) Or, PDU Session related state transition information such as transition type, frequency SM state changes, etc. State transition identifier: - “PDU Session Establishment”; - “PDU Session Release”; - “Communication failure”; or - “PLMN change”. It can be reported for a group of UEs(e.g., the number of total transitions or percentage of the group UEs who have transitions). |
| > timer information | AMF, SMF | Timer information which has been set for the UE, such as timer type, duration. |
| NOTE 1: NWDAF can optionally provide transaction dispersion analytic information for MM and SM transactions. NOTE 2: This data is provided per time interval that is indicated by the NWDAF. NOTE 3: The word “NF” refers here to SCP as well. As a centric routing/proxy entity, SCP might be involved with some signaling that may not be visible to the NF sending the SCP's ingress traffic, such a signaling could be associated with actions such as selection & discovery, retransmissions, re-directions, re-selection, throttling etc. NOTE 4: e.g. average time duration of a transaction (averaged over all of the transactions handled by the SCP within the requested time interval). NOTE 5: Distribution of the failed responses, e.g how many time-outs vs how many server errors vs how many consumer errors. NOTE 6: A posterior request is a request sent from NF1 to NF2 (or from NF2 to NF1) that was triggered by a previous request received by NF1. For example, in clause 4.3.2.2 of TS 23.502, the request sent by AMF in step 3 and the request sent by SMF in step 11 are both posterior requests of the request received by the AMF in step 1. These posterior requests can be collected by NF1 (the AMF in this example). An SCP can collect a posterior request between NF1 and NF2 only if it routes incoming requests to NF1 as well as route requests between NF1 and NF2. |  |  |

Table 6.x.2-2: NF Context Data collection

| Information | Source | Description |
| --- | --- | --- |
| NF ID |  | NF instance ID |
| > NF profile | NRF | NF Profile information such as allowed NF information for the NF and NF Service(s). |
| > NF load status information | NRF, SCP | Load information indicates the current load of the NF and NF Service(s), e.g. CPU, memory, and/or percentage of load information. (NOTE 1) |
| > Capacity and priority information of NFs and NF Services | NRF, SCP | Capacity and priority information of the registered NF and NF Service(s). (NOTE 1) |
| NF heart-beat related information | NF | NF heart-beat related information such as responding time, Number of retransmissions, heart-beat intervals. |
| heart-beat related information | SCP | heart-beat related information such as responding time, Number of retransmissions, heart-beat intervals between the NF and NRF and/or between SCP and NRF. |
| > Unexpected operational status indicator | NF, OAM | The parameter indicates unexpected operational status occurs. It is an indication of deviation from the normal operations, based on thresholds or rules configured by operator at the NF or OAM (e.g. for energy consumptions, loads, etc). |
| NOTE 1: If this information is not available in the NRF, it can be obtained from the OAM. |  |  |

Table 6.x.2-3: NF Specific Data collection

| Information | Source | Description |
| --- | --- | --- |
| NF ID |  | NF instance ID |
| > Usage information of UE IP address resources | SMF | Usage information of UE IP address resources (dynamic and static, V4, V6, etc.) for CP or UP allocation, such as number, usage, number of UE IPs, which prohibit allocation during certain time interval, etc. This parameter is valid for SMF only. |
| > Load information of connected UPFs | SMF | Load information of connected UPFs such as using PFCP Load Control Information. This parameter is valid for SMF only. |
| SCP Signalling statistics | SCP | The number of different types of signalling received and sent by SCP during a target time period, etc. This parameter is valid for SCP only. |

Table 6.x.2-4: Application activation time information

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | Identifies the application providing this information. |
| User activation time information (1…max) | AF | Information of activation time for the users (e.g. IoT users) per application. |
| > Number of UEs | AF | The total number of UEs |
| > Active Time | AF | The time stamp of the users per application switch to active, or the start and end time of the users activity per application. |
| > Inactive Time | AF | The time stamp of the users per application switch to inactive, or the start and end time of the users inactivity per application, if applicable. |
| > UE type ID | AF | Identifies a group of UEs, e.g. external group ID, or a list of UE IDs. |

Table 6.x.2-5: Data collection from NRF/OAM

| Information | Source | Description |
| --- | --- | --- |
| NRF ID |  | NRF instance ID. |
| Number of NF service registration requests | OAM | Number of registration request received at the NRF |
| > Number of successful NF service registrations | OAM | Number of successful registrations |
| > Number of failed NF service registrations due to encoding error of NF profile | OAM | Number of failed registrations |
| > Number of failed NF service registrations due to NRF internal error | OAM | Number of failed registrations |
| Number of NF service update requests | OAM | Number of update request received at the NRF |
| > Number of successful NF service updates | OAM | Number of successful updates |
| > Number of failed NF service updates due to encoding error of NF profile | OAM | Number of failed updates |
| > Number of failed NF service updates due to NRF internal error | OAM | Number of failed updates |
| Number of NF discovery requests | OAM | Number of discovery request received at the NRF |
| > Number of successful NF discoveries | OAM | Number of successful discovery attempts |
| > Number of failed NF service discoveries due to unauthorized NF Service consumer | OAM | Number of failed discovery attempts |
| > Number of failed NF discoveries due to input errors | OAM | Number of failed discovery attempts |
| > Number of failed NF discoveries due to NRF internal error | OAM | Number of failed discovery attempts |
| NOTE: If this information is not available in the OAM, it can be obtained from the NRF if available. |  |  |

Table 6.x.2-6: Data collection from MDAF/MDAS of control plane congestion analysis

| Information | Source | Description |
| --- | --- | --- |
| affectedObject | MDAF | Indication of 5GC NFs where congestion issues occurred or potentially may occur. |
| cPCongestionIssueID | MDAF | This field holds the ID of the control plane congestion issue which is reported. |

6.x.3	Output analytics

The output analytics of signalling storm provided by NWDAF is defined in Table 6.x.3-1, and Table 6.x.3-2. The Table 6.x.3-3 gives examples of NF/SCP actions for solving each risk. The final mitigation or prevention operations are based on operator's policy/configuration and NF/SCP implementation.

The statistics and predictions are provided with a Validity Period, as defined in clause 6.1.3.

Table 6.x.3-1: Signalling storm statistics

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm statistics. |
| > Target NF ID/SCP ID | A list of impacted NFs/SCPs of signalling storm detected by NWDAF. |
| > Cause of the signalling storm | The potential cause of abnormal level of signalling (i.e. massive signalling from UE or NF abnormal signalling). |
| > Source UE/NF | a UE or a group of UEs or several groups of UEs identified by internal Group ID or SUPI or SUPI range, or a list of NFs which cause the signalling storm. |
| > OPTIONAL Time slot entry (1..max) | List of time slots during the Analytics target period. |
| >> Time slot start | Time slot start within the Analytics target period. |
| >> Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| >> Received Signalling Analytics | Information of signalling received by the target NF(s). |
| >>> Total number of received signalling | Indicates the statistics on the number of signalling messages received by the target NF(s) in the time slot. |
| >>> Growth rate of received signalling | Difference between the number of signalling messages received by the NF in the time slot and the number of signalling messages received by the target NF(s) in the previous time slot. |
| >>> Signalling analytics of UE | Indicates the statistics on the number of signalling messages received from UEs within the time slot. NOTE1 |
| > OPTIONAL Timer List | The list of timer information per source UE(s) (NOTE 1) |
| >> Type of timer | The type of timer which has been set. |
| >> Timer duration | The timer duration that has be selected for the source UE(s). |
| NOTE 1: Only available when Cause of signalling storm is massive signalling from UEs, and there exists Source UE(s). |  |

Table 6.x.3-2: Signalling storm predictions

| Information | Description |
| --- | --- |
| Report (1..max) | List of predicted signalling storm analytics. |
| > Target NF ID/SCP ID | A list of impacted NFs/SCPs signalling storm predicted by NWDAF. |
| > Cause of the signalling storm | The potential cause of abnormal level of signalling (i.e. massive signalling from UE or NF abnormal signalling). |
| > Source UE/NF | a UE or a group of UEs or several groups of UEs identified by internal Group ID or SUPI or SUPI range, or a list of NFs which cause the signalling storm. |
| > OPTIONAL Time slot entry (1..max) | List of time slots during the Analytics target period. |
| >> Time slot start | Time slot start within the Analytics target period. |
| >> Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| >> Reference point | The information of the reference point impacted. (e.g. N1, N2) |
| >> Service operation(s) | The information of the service operation(s) impacted. (e.g. Namf_Communication(UEContextTransfer), Nsmf_PDUSession(UpdateSMContext)) |
| >> Received Signalling Analytics | Information of signalling received by the target NF(s). |
| >>> Received number of signalling | Received number of signalling of the specific signalling type. |
| >>> Growth rate of received signalling | Difference between the number of signalling messages received by the NF in the time slot and the number of signalling messages received by the target NF(s) in the previous time slot. |
| >>> Signalling analytics of UE | Indicates the statistics on the number of signalling messages received from UEs within the time slot. (NOTE 1) |
| > OPTIONAL Timer List | The list of timer information per Source UE(s) (NOTE 1) |
| >> Type of timer | The type of timer. |
| >> Timer duration | The timer duration that commonly is selected for the Source UE(s). |
| > Priority | Priority (relative to other NFs of the same type) of candidate NFs as defined in the TS 29.510 [18]. |
| > Capacity | Candidate NF capacity information, expressed as a weight relative to other NF instances of the same type as defined in the TS 29.510 [18]. |
| > Confidence | Confidence of this prediction. |
| NOTE 1 Only available when Cause of signalling storm is massive signalling from UEs, and there exists Source UE(s). |  |

Table 6.x.3-3: Example mechanisms to mitigate and prevent the signalling storm

| Cause of the signalling storm | Example actions of NFs |
| --- | --- |
| massive signalling from UE | AMF sets MM NAS related timer (e.g. back-off, T3512) for a selected set of UEs. |
| massive signalling from UE | SMF sets SM NAS related timer (e.g. back-off) for a selected set of Sessions. |
| massive signalling from UE | AMF/SMF sets suggested N1/N2 interface related ingress/egress threshold, or AMF triggers RAN to initiate overload control for a selected set of UEs in specific slice or priority as defined in clause 8.7.7 of TS 38.413 [8] to start overload control. The AMF may assign a slice under signalling storm conditions to the UEs and initiate overload control for the specific slice. |
| NF abnormal signalling | NRF configures the local policy (e.g. to prevent the source NF with abnormal signalling from being discovered or discovering others, to update the NF profile(s)(e.g. Priority, Capacity) of NF(s) based on the signalling storm statistics and/or predictions and send the updated NF profiles(s) to NF service consumer(s) of the NF(s)(e.g. subscriber of the NF profile as defined in clause in 4.17.7 of TS 23.502)). |
| NF abnormal signalling | Source NF configures to (re)select other NFs instead of NF with abnormal signalling and may unsubscribes the NF with abnormal signalling. |
| NF abnormal signalling | Source NF configures to deprioritize the NFs/Services with abnormal signalling from being selected. |
| NF abnormal signalling | Source NF triggers UE Reregistration and/or Session Reestablishments to avoid NF with abnormal signalling. |
| NF abnormal signalling | SCP blocks or throttles messges originating from an NF/service with abnormal signalling. |
| NF abnormal signalling | SCP redirects requests towards an NF service producer with abnormal signalling towards another service producer |
| NF abnormal signalling | SCP notifies communication peers of an NF with abnormal signalling about abnormal condition, e.g. high load |

6.x.4	Procedures

The NWDAF can provide information on network signalling storm as follows. (Figure 6.x.4-1).

Figure 6.x.4-1: Procedure for NWDAF-assisted Network Signalling Storm Mitigation and Prevention

1.	The consumer NF (SCP included) or other entities subscribes to or sends a request to NWDAF for the signalling storm analytics using either Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation. The request additionally includes thresholds such as confidence level and accuracy of detection.

The Analytics ID is set to "Signalling Storm analytics". The target for analytics reporting is set to be any NF/SCP or any UE. Analytic filters may be provided as shown in clause 6.x.1.

The consumer NF can request statistics or/and predictions for a given Analytics target period.

The consumer NF may also subscribe to "NF load analytics" as depicted in clause 6.5 and trigger the signalling storm analytics based on output of the NF load analytics, e.g. CPU usage is over 80%.

The consumer NF may also subscribe to "Abnormal behaviour" analytics as depicted in clause 6.7.5 and trigger the signaling storm analytics based on the suspicion of DDoS attack.

The consumer NF may also subscribe to "Dispersion Analytics" analytics as depicted in clause 6.10 and trigger the signaling storm analytics when the transaction dispersion is above the expected transaction dispersion configurable threshold.

2.	The NWDAF retrieves Input data from NFs and SCPs, using Nnf_EventExposure_Subscribe, or from NRF using Nnrf_NFManagement_NFStatusSubscribe service operation, such as UE and/or NF releated context data, NF context, AF data, MDAF data and UE behavioural information to derive the required analytics as depicted in clause 6.x.2. The NWDAF may optionally request additional data aligned with the analysis target identified in step 1.

The NWDAF may optionally subscribe the abnormal UE behaviour analytics and UE dispersion analytics as specified in clause 6.7.5 and 6.10.3 as input data from other NWDAFs, using Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation.

3.	The NWDAF derives the required analytics with the collected data described in clause 6.x.2.

4.	The NWDAF invokes Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response or response to the consumer NF/SCP for the Output data analytics as depicted in clause 6.x.3.

5.	The consumer upon receiving the detection and/or prediction and/or mitigation may execute based on the example mechanisms as depicted in clause 6.x.3.

# * * * * Second change * * * *

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, NRF, SCP |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, NRF, SCP |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, NRF, SCP |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, NRF, SCP |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-NWDAF, V-NWDAF |
|  | Unsubscribe |  | H-NWDAF, V-NWDAF |
|  | Notify |  | H-NWDAF, V-NWDAF |
|  | Request | Request / Response | H-NWDAF, V-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-NWDAF, V-NWDAF |
|  | Unsubscribe |  | H-NWDAF, V-NWDAF |
|  | Notify |  | H-NWDAF, V-NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Editor´s note:	It is FFS whether separate Roaming Analytics and Roaming data services for inbound and outbound roaming scenarios are required.

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| E2E data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behavior | Analytics ID: Movement Behavior | Statistics or predictions on movement behavior for an applicable area |
| Signalling Storm | Analytics ID: Signalling storm | Statistics or predictions on controlling signalling storm for network abnormal behaviour mitigation or prevention. |

# * * * * Third change * * * *

6.2.2.1	General

The Data Collection from NFs and SCP is used by NWDAF to subscribe/unsubscribe at any 5GC NF to be notified for data on a set of events.

The Data Collection from NFs and SCP is based on the services of AMF, SMF, UDM, PCF, NRF, NSACF, UPF and AF (possibly via NEF):

-	Event Exposure Service offered by each NF as defined in clause 4.15 and clause 5.2 of TS 23.502 [3].

-	other NF services (e.g. Nnrf_NFDiscovery and Nnrf_NFManagement in NRF as defined in clause 4.17 of TS 23.502 [3])

This data collection service is used directly in order to retrieve behaviour data for individual UEs or groups of UEs (e.g. UE reachability) and also to retrieve global UE information (e.g. Number of UEs present in a geographical area).

Table 6.2.2.1-1: NF Services consumed by NWDAF for data collection

| Service producer | Service | Reference in TS 23.502 [3] |
| --- | --- | --- |
| AMF | Namf_EventExposure (NOTE 2) | 5.2.2.3 5.2.3.5 |
| SMF | Nsmf_EventExposure (NOTE 2) | 5.2.8.3 5.2.3.5 |
| PCF | Npcf_EventExposure (for a group of UEs identified by an Internal-Group-Id or any UE) Npcf_PolicyAuthorization_Subscribe (for a specific UE) | 5.2.5.7 |
| UDM | Nudm_EventExposure | 5.2.3.5 |
| NEF | Nnef_EventExposure | 5.2.6.2 |
| AF | Naf_EventExposure | 5.2.19.2 |
| NRF | Nnrf_NFDiscovery | 5.2.7.3 |
|  | Nnrf_NFManagement | 5.2.7.2 |
| NSACF | Nnsacf_SliceEventExposure | 5.2.21.4 |
| UPF | Nsmf_EventExposure or Nupf_EventExposure | 5.2.8.3 5.2.26.2 |
| SCP | Nscp_EventExposure Service | 5.2.X.2 |

NOTE 1:	There is no data collected from the PCF by the NWDAF defined in this Release of the specification.

NOTE 2:	The Nudm_EventExposure can be used when NWDAF uses the procedures specified in clause 4.15.4.4 of TS 23.502 [3] to subscribe to AMF or SMF via UDM.

To retrieve data related to a specific UE, there are two cases:

-	If no Area of interest is indicated by the consumer, the NWDAF shall first determine which NF instances are serving this UE as stated in table 6.2.2.1-2 unless the NWDAF has already obtained this information due to recent operations related to this UE.

-	If an Area of interest is indicated, the NWDAF can:

-	First determine the AMF serving the UE and subscribe UE location from the AMF. Once the UE is in or moves into the Area of interest, the NWDAF determines which NF instances are serving this UE as stated in table 6.2.2.1-2 unless the NWDAF has already obtained this information due to recent operations related to this UE; or

-	Determine the NF instances of a given type of network function serving the Area of interest by querying NRF unless the NWDAF has already obtained this information due to recent operations related to this UE.

Table 6.2.2.1-2: NF Services consumed by NWDAF to determine which NF instances are serving a UE

| Type of NF instance (serving the UE) to determine | NF to be contacted by NWDAF | Service | Reference in TS 23.502 [3] |
| --- | --- | --- | --- |
| UDM | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| AMF | UDM | Nudm_UECM | 5.2.3.2 |
| SMF | UDM | Nudm_UECM | 5.2.3.2 |
| BSF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| PCF | BSF | Nbsf_Management | 5.2.13.2 |
| NEF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| NWDAF | NRF UDM | Nnrf_NFDiscovery Nudm_UECM | 5.2.7.3 5.2.3.2 |
| NSACF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| GMLC | NRF | Nnrf_NFDiscovery | 5.2.7.3 |

The UDM instance should be determined using NRF as described in clause 4.17.4 of TS 23.502 [3] and factors to determine as described in clause 6.3.8 of TS 23.501 [2].

The AMF, SMF instances should be determined using a request to UDM providing the SUPI. To determine the SMF serving a PDU session, the NWDAF should in addition provide the DNN and S-NSSAI of this PDU Session; otherwise the NWDAF will obtain a list of possibly multiple SMFs (e.g. one per PDU session).

The BSF instance is discovered and selected according to TS 23.503 [4], clause 6.1.1.2.2.

The PCF instance serving UE PDU Session(s) should be determined using a request to BSF with the allocated UE address, DNN and S-NSSAI.

To collect data (e.g. current number of UEs registered in a network slice or current number of PDU Sessions established in a network slice) from NSACF, the NSACF instance is discovered and selected as specified in clause 6.3.22 of TS 23.501 [2].

When NWDAF receives a request addressed to an Internal Group ID from a consumer, NWDAF may need to initiate data collection from several 5GC NFs, such as AMF, SMF, UDM, PCF, AF (e.g. via NEF), etc. If an Area of interest is indicated by the consumer, NWDAF may first discover the instances of the required 5GC NFs deployed in the network, e.g. by querying NRF, otherwise:

-	For discovering the UDM, NWDAF can query the NRF with the Internal Group ID as the target of the query.

-	For discovering AMF, SMF, PCF, NEF and AF, NWDAF may need to discover all instances in the network by using the Nnrf_NFDiscovery service.

NOTE 3:	It is assumed that all members of an Internal Group ID belong to the same UDM Group ID. NWDAF can select a UDM instance supporting the UDM Group ID of the Internal Group ID.

Then, if data needs to be collected from AMF, SMF, UDM and PCF, NWDAF may initiate the data collection with the Internal Group ID as the target, e.g. subscribing to the event exposure in all the instances of a given type of network function. This subscription to all the instances of required source of event exposure handles, e.g. mobility of UEs across AMFs, or initiation of new PDU sessions with different allocated SMFs.

For collecting data from AMF and SMF, NWDAF may collect the data directly from AMF and/or SMF, or indirectly via UDM, according to clause 4.15.4.4 of TS 23.502 [3]. The indirect method may be required if the event exposure subscription from NWDAF, for a UE identified by a SUPI or a group of UEs identified by an Internal-Group-Id, needs to survive the removal of UE context in the AMF including event exposure subscriptions, or upon the creation of new UE context in AMF or SMF serving the UE or group of UEs. In this case the UDM is responsible for (re)creating event exposure subscriptions in AMF and SMF, as specified in clause 4.15.4.4 of TS 23.502 [3].

The NWDAF determines to collect data from a trusted AF supporting specific Event ID(s) and serving specific application(s) based on internal configuration.

The NEF instance that is serving a specific network slices and/or applications of a UE should be determined using NRF using optional request parameters as defined in clause 6.3.14 of TS 23.501 [2]

If NWDAF needs to collect data from an AF deployed outside the operator's domain, the NWDAF shall contact NEF with a SUPI or Internal Group ID as the target of the data collection. NEF is responsible for translation of SUPI to GPSI, or internal to external group identifiers, by querying UDM, prior to contacting the AF.

NOTE 4:	It is assumed that an AF is provisioned with the list of UE IDs (GPSIs or SUPIs) belonging to an External or Internal Group ID.

NWDAF may collect data directly from UPF for a specific UE identified by a SUPI, a group of UEs identified by an Internal-Group-Id or for any UEs as defined in clause 5.8.2 of TS 23.501 [2] and in clause 4.15.4.5 of TS 23.502 [3]. NWDAF may subscribe indirectly via SMF or directly to UPF for UPF data collection. The direct subscription to UPF event exposure service is only for data collection for any UE e.g., to collect user data usage information for NWDAF NF Load analytic, and if the subscription is not including any of the parameters described in Table 4.15.4.5.1-1 of TS 23.502 [3].

NOTE 5:	To avoid causing high UPF load due to extensive reporting related to all traffic flows, the NWDAF can preferably subscribe for reporting for some UEs only.

To retrieve required data for any UE, the NWDAF may subscribe to events from the AMF and/or SMF instances or UPF instances it has determined, setting the target of event reporting to "any UE" and the event filter(s) according to the Analytics Filter Information. Alternatively, if the required data is communication related and for any UE within an Area of interest, the NWDAF can obtain from the AMF instances it has determined a list of UEs located within the Area of Interest. Based on the obtained UE list, for each UE in the list, the NWDAF retrieves the SMF serving the UE and the NWDAF subscribes to data from the relevant SMF per each specific UE. The indirect event exposure subscription to AMF or SMF via UDM is not available for "any UE" or "any UE within an Area of interest". If the required data is collected from UE via AF as described in clause 6.2.8 and the Target of Analytics Reporting received from consumer is "any UE", the NWDAF may either set the target of event reporting to "any UE" in the data collection request to the AF, or may determine a list of UEs from AMF and/or SMF based on the Analytics Filter Information and send the data collection request to the AF for the determined list of UEs.

NOTE 6:	If NWDAF requires collecting data from either AMF or SMF for "any UE" or "any UE within an Area of Interest", NWDAF can use the direct Event Exposure subscription to AMF or SMF, since subscriptions to "any UE" or "any UE within an Area of Interest" are persistent by nature in AMF or SMF, due to not being linked to a UE context.

To retrieve data related to "any UE" based on Analytics Filter Information, the NWDAF shall first determine which NF instances are matching the Analytics Filter Information (see clause 6.7.5.1) as stated in table 6.2.2.1-3 unless the NWDAF has already obtained this information due to recent operations related to this Analytics Filter Information.

Table 6.2.2.1-3: NF Services consumed by NWDAF to determine which NF instances are matching analytics filters

| Type of NF instance (matching analytics filters) to determine | NF to be contacted by NWDAF | Service | Reference in TS 23.502 [3] |
| --- | --- | --- | --- |
| AMF, SMF, UPF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |

To retrieve data related to Analytics IDs for "any UE" with Analytics Filter Information defining an area of interest in terms of TA or Cells and/or with specific S-NSSAIs, NWDAF requires the network slice association information to properly determine the AMFs to collect data from as well as the proper queries to OAM for data collection.

NOTE 7:	Examples of Analytics ID requiring NWDAF to use network slice association information for data retrieval are: network performance clause 6.6.1; user data congestion clause 6.8.1; QoS Sustainability clause 6.9; Dispersion Analytics clause 6.10.1; observed service experience for a Network Slice clause 6.4.1; and slice load analytics clause 6.3.

The network slice association information comprises the TAs associated with each AMF and for each TAI its associated access type, cells and list of supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF). Additionally, the mapping of cells per TAI and supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF) for TAI for each AMF can change and NWDAF shall obtain this accurate information in order to properly retrieve data for analytics generation.

In order to derive the network slice association information, NWDAF may be configured with the mapping of cells per TAI and the S-NSSAIs per TAI. NWDAF may subscribe to the "S-NSSAIs per TAI mapping" event exposed by AMF. The NWDAF may use the configured information (when the analytics subscription or request is at cell granularity) and the area of interest in the analytics subscription or request to retrieve from AMF the list of supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF) per TAI and access types per TAI for each AMF in the required area of interest. NWDAF consumes the "S-NSSAIs per TAI mapping" event exposed by AMF using, as target of event reporting, the list of TAIs based on the area of interest received in the Analytics Filter Information or identified by the mapping of the Cells per TAI matching to the Cell granularity included in the Analytics Filter Information. The AMF "S-NSSAIs per TAI mapping" event output contains, for each of the TAIs requested by NWDAF, its associated access type and the list of supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF).

To retrieve data from SMFs for Analytics IDs subscription or requests for "any UE" including Analytics Filter Information with specific Applications, DNNs, DNAIs and area of interest per TA granularity, NWDAF shall first discover the SMF serving the area of interest via NRF.

NOTE 8:	Examples of Analytics ID requiring NWDAF to collect data related to PDU sessions associated with an AoI with TA granularity are: network performance clause 6.6.1; user data congestion clause 6.8.1, QoS Sustainability clause 6.9.

NWDAF may directly consume events from the discovered and selected SMF using the event target set to "any PDU session" and event filters with the same parameters of the Analytics Filter Information, i.e. list of Application IDs and/or DNNs and/or DNAI and the area of interest related to the requested Analytics ID.

1.	When SMF supports the exchange of UE Location parameter when SMF interacts with AMF via Nsmf_PDUSession_Create/Update/CreateSMContext/UpdateSMContext due to session establishment, modification, or release, service request, or handover procedures (as defined in clause 5.2.8.2 of TS 23.502 [3]), SMF can directly map the PDU sessions to an AoI with TA granularity.

If there are any changes in PDU sessions in the area of interest, for the Application ID and/or DNN and/or DNAI subscribed by NWDAF, SMF notifies the detected changes to NWDAF via Nsmf_EventExposure_Notify service operation, enabling NWDAF to keep an updated map of SMF and PDU sessions associated with the Analytics Filter Information in an area of interest.

2.	When SMF does not support the exchange of UE Location parameter when SMF interacts with AMF but supports the mapping of PDU sessions per TA (as defined in clause 5.6.11 of TS 23.501 [2]), SMF may subscribe to UE mobility event notifications of AMF as described in clause 5.3.4.4 of TS 23.501 [2] using event ID "UE moving in or out of Area of Interest" and Event Filters as described in Table 5.2.2.3.1-1 from TS 23.502 [3] to retrieve the list of SUPIs (and GPSIs if available) in the area of interest. Based on the retrieved list of SUPIs in the area of interest, SMF identifies the PDU sessions in the area of interest.

3.	When SMF does not support the exchange of UE Location parameter when SMF interacts with AMF nor supports the mapping of PDU sessions per TA (as defined in clause 5.6.11 of TS 23.501 [2]), SMF rejects the request from NWDAF. Upon the reject, NWDAF identifies the need to create the mapping of PDU sessions per TA. NWDAF subscribes to UE mobility event notifications of AMF as described in clause 5.3.4.4 of TS 23.501 [2] using event ID "UE moving in or out of Area of Interest" and Event Filters as described in Table 5.2.2.3.1-1 from TS 23.502 [3] to retrieve the list of SUPIs (and GPSIs if available) in the area of interest. Based on the retrieved list of SUPIs in the area of interest, NWDAF subscribes to the SMFs serving the UEs in the area of interest and derives the mapping of PDU sessions per TA.

An NWDAF may require to discover and select other NWDAFs for UE related analytics. In this case, the NWDAF may discover from UDM if an NWDAF is already collecting data related to the UE, as specified in clauses 5.2 and 6.1C.

# * * * * End of changes * * * *