3GPP TSG-SA2 Meeting #166	S2-2412985

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024					   rev of S2-2412614

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0488 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for multihop L3 U2U relay reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | U2U relay reselection procedure is supported for L2 U2U relay and L3 U2U relay. But, it is missing for multihop L3 U2U relay. Rev: Simplify the description only refer to multihop U2U relay procedure. Update general subclause of 6.7.4.1 to include multihop U2U relay reselection procedure as additional procedure for multihop U2U relay for non-IP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Multihop L3 U2U Relay reselection procedure is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multihop L3 U2U Relay does not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.4.1, 6.7.4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of 1nd Changes * * *

#### 6.7.4.1	General

After being connected to the 5G ProSe UE-to-UE Relay, the 5G ProSe End UEs may trigger the 5G ProSe UE-to-UE Relay reselection based on conditions (e.g. the measured signal strength of PC5 unicast link with the 5G ProSe UE-to-UE Relay) as specified in TS 38.300 [12].

For 5G ProSe UE-to-UE Relay reselection, a 5G ProSe UE-to-UE Relay may be discovered by either the discovery procedures defined in clause 6.3.2.4 or by the negotiated 5G ProSe UE-to-UE Relay reselection procedure defined in clause 6.7.4.2 or clause 6.7.4.3. For 5G ProSe multi-hop Layer3 UE-to-UE Relay for non-IP, in addition, a 5G ProSe UE-to-UE Relay may be discovered by the discovery procedure defined in clause 6.3.2.6.3, by the negotiated 5G ProSe multi-hop Layer3-UE-to-UE Relay reselection for non-IP type PDU defined in clause 6.7.4.X.

In the negotiated UE-to-UE Relay reselection defined in clause 6.7.4.2 or clause 6.7.4.3, one 5G ProSe End UE initiates the UE-to-UE Relay reselection procedure, the 5G ProSe End UEs can negotiate a new 5G ProSe UE-to-UE Relay using the existing connection and to establish the communication via the reselected 5G ProSe UE-to-UE Relay prior to releasing the communication via the current 5G ProSe UE-to-UE Relay.

* * *Start of 2nd Changes * * *

#### 6.7.4.X Negotiated 5G ProSe multi-hop Layer-3 UE-to-UE Relay Reselection for Non-IP type PDU

Depicted in Figure 6.7.4.X-1 is the procedure for the negotiated 5G ProSe multi-hop Layer-3 UE-to-UE Relay reselection.

Figure 6.7.4.X.-1. Negotiated multihop Layer-3 UE-to-UE Relay reselection procedure.

- 5G ProSe End UEs have set up PC5 unicast links with one or multiple 5G ProSe UE-to-UE Relay, based on the procedure defined in clause 6.7.5.

- 5G ProSe End UEs are transferring data traffic e.g., Ethernet traffic, via 5G ProSe UE-to-UE Relay(s).

- The initiating 5G ProSe End UE determines, e.g. based on PC5 signal strength, to perform U2U Relay reselection.

- The initiating 5G ProSe End UE may discover a new multi-hop U2U relay path to the target 5G ProSe End UE.(refer to clause 6.3.2.6.3)

- The initiating 5G ProSe End UE sends a Link Modification Request to the responding 5G ProSe UE-to-UE Relay, which include a Relay re-selection indication and the newly discovered path information, and the User Info ID of the responding 5G ProSe End UEs.

- 5G ProSe UE-to-UE Relay determines the responding 5G ProSe End UE based on the received User Info ID of End UEs in step 5 and sends a Link Modification Request message to the responding End UEs (via other UE-to-UE Relay(s) if the responding End UE is connected via other UE-to-UE Relay(s)).

The Link Modification Request message includes a Relay re-selection indication, the newly discovered path information.

- The responding 5G ProSe End UE may select the newly discovered path in the received link modification as newly selected multi-hop path.

- The responding 5G ProSe End UE may initiate PC5 connection setup or modification procedure for communication with the initiating 5G ProSe End UE with path info for the selected multi-hop path in step 7. (refer to clause 6.7.5.2.2). For Ethernet traffic, MAC address of the initiating 5G ProSe End UE and MAC address of the responding 5G ProSe End UE are included.

- The responding 5G ProSe End UE sends a Link Modification Accept message via the existing multi-hop 5G ProSe UE-to-UE Relay(s) to the initiating 5G ProSe End UE. Link Modification Accept message includes the newly selected multi-hop path info, User Info ID of the iniating End UE, User Info ID of the responding End UE, and Relay re-selection indication.

- The responding 5G ProSe UE-to-UE Relay sends a Link Modification Accept message to the initiating 5G ProSe End UE. The Link Modification Accept message includes the newly selected multi-hop path, User Info ID of the initiating End UE, User Info ID of the responding End UE, and Relay re-selection indication.

11. The initiating 5G ProSe UE-to-UE Relay sends Link Modification Ack to the responding 5G ProSe UE-to-UE Relay.

12.	 The 5G ProSe UE-to-UE Relay sends Link Modification Ack to the responding 5G ProSe End UE. Link Modification Ack includes User Info ID of the iniating End UE, User Info ID of the responding End UE, and Relay re-selection indication.

13. 5G ProSe End UEs transfer traffic via the newly selected multi-hop path. For Ethernet traffic, existing MAC address of 5G ProSe End UEs are reused.

* * *End of Changes * * *