3GPP TSG-WG2 Meeting #166	S2-2412986

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024	(Revision of S2-2411746, S2-2411160)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0140 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing the ENs on message names |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | <Rev#2 - SA2#165> There is the EN on message names used in the pre-flight planning procedure, specifically, “Editor's note: The message names used in the procedures will be updated.” This EN now can be addressed, since the Nnef_UAVFlightAssistance services have been specified in clause 4.4.1.1.3. It is proposed to reuse them (and the newly proposed Nnef_RetrieveInfoUAVFlight_Get service) for the purpose of pre-flight planning and in-flight monitoring. <Rev#4 - SA2#166> - Incorrect operation semantics of the Nnef_UAVFlightAssistance_Notify. |  |  |  |  |  |  |  |  |
| Summary of change: |  | <Rev#2 - SA2#165> -Updating the message names (where required) using Nnef_UAVFlightAssistance and Nnef_RetrieveInfoUAVFlight_Get service operations, -correcting the description of some steps <Rev#4 - SA2#166> - The operation of Nnef_UAVFlightAssistance_Notify is corrected to use Subscribe/Notify model in cl. 5.12.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | <Rev#2 - SA2#165> The EN remains unresolved and the procedure misleading <Rev#4 - SA2#166> Incorrect use of the SBI operation may cause incorrect stage-3 work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.12.2, 5.12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.256 CR 0141 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

### 5.12.2	Procedure for NEF Assisted Pre-flight Planning

USS / UTM

GMLC

NFs, OAM

NWDAF

UAV UE

UAS NF / NEF

1. PDU session establishment as described in clause 5.2.3 of TS 23.256 (for communication between UAV/UAV-C and USS/UTM)

2. Pre-flight planning request (via the application layer)

3. Determining information for the pre-flight planning request towards the UAS NF/NEF

4. Nnef_RetrieveInfoUAVFlight_Get request

5. Determining NFs and services to invoke for the request

6. Nnef_RetrieveInfoUAVFlight_Get response

7. Response to the Pre-flight planning request (via the application layer)

8. Steps 2–5 from clause 5.3.4 of TS 23.256 to obtain a list of aerial UEs in a geographic area

9. Obtaining the Movement Behaviour analytics as described in clause 6.21.4 of TS 23.288 [6]

10. Obtaining the QoS Sustainability analytics as described in clause 6.9.1 of TS 23.288 [6]

11. Invoking GMLC service(s)

12. Generating the pre-flight assistance information

13. Nnef_RetrieveInfoUAVFlight_Get response

14. Planning a flight path(s)

15. Response to the Pre-flight planning request (via the application layer) containing the planned flight path(s)

Figure 5.12.2-1: Procedure for NEF Assisted Pre-flight Planning

1.	The UAV establishes a PDU Session for communication with the USS/UTM as described in clause 5.2.3.

2.	(via application layer) The UAV requests the pre-flight planning service from the USS/UTM.

3.	The USS/UTM determines to request a pre-flight assistance information from the 5GC and derives the information to provide to the NEF/UAS NF.

NOTE:	The content of pre-flight planning service information derived at USS is out of scope of 3GPP.

4.	The USS/UTM sends an Nnef_RetrieveInfoUAVFlight_Get request with the indication to provide a pre-flight planning assistance for a target UAV (i.e., aerial UE). The request message includes identifier of the UAV (e.g. GPSI), information about the starting and ending points (and any intermediate point, if any) for the flight. The request may also include requirements on the flight path (e.g. on time, fastest, shortest), candidate flight path(s) (either received from the UAV or locally derived at the USS/UTM), and accuracy level of predictions relevant to the flight planning.

5.	The NEF maps parameters included in the received Nnef_RetrieveInfoUAVFlight_Get request from the USS/UTM to information used by the 3GPP system (e.g. map the geographical area into an area of interest that is represented by a list of Cell IDs, gNB IDs or TAIs). The NEF determines services needed for the request and relevant NFs, e.g. NEF service on UAV tracking and mode (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), NWDAF analytics service (Movement Behaviour analytics, and/or QoS Sustainability Analytics), GMLC service (e.g. Ranging/Sidelink Positioning location).

6.	If the NEF cannot satisfy the requirements provided by the USS/UTM on flight planning (e.g. cannot get predictions relevant to the flight planning from NWDAF with the required accuracy level), the NEF rejects the pre-flight planning request from the USS/UTM, and the NEF may include the details about the reasons of the rejection inside the response.

7.	(via application layer) If the USS/UTM receives the response to the Nnef_RetrieveInfoUAVFlight_Get with the rejection of the pre-flight planning in step 6, the USS/UTM may respond to the UAV’s pre-flight planning request and include the received details rejection reason(s) if available.

8.	The NEF performs Steps 2-5 of clause 5.3.4 to determine a list of UAVs in the areas of interest.

9.	The NEF subscribes/requests for notifications on Movement Behaviour analytics from NWDAF as defined in clause 6.21.4 of TS 23.288 [20]. The request includes an identifier of the UAV (e.g. GPSI) obtained in step 4 and height information of the UAV; other parameters included in the request are described in clause 6.21.4 of TS 23.288 [20]. Additionally, if the list of UAVs in the areas of interest is obtained in Step 8, it may be used by the NEF to request Movement Behaviour analytics from NWDAF.

10.	The NEF subscribes/requests for notifications on QoS Sustainability analytics from NWDAF as defined in clause 6.9.1 of TS 23.288 [20]. The request includes an identifier of the UAV (e.g. GPSI) obtained in step 4 and the UAV’s flight path information; other parameters included in the request are described in clause 6.9.1 of TS 23.288 [20]. Additionally, if the list of UAVs in the areas of interest is obtained in Step 8, it may be used by the NEF to request QoS Sustainability analytics from NWDAF.

11.	The NEF may request GMLC service(s), e.g. Ranging/Sidelink positioning location as defined in clause 6.20.4 of TS 23.273 [8]. The list of UAVs in the areas of interest output from step 8 may be used by the NEF to request the GLMC service on Ranging/Sidelink Positioning location.

12.	The NEF generates the pre-flight assistance information based on the outputs from Steps 8-10. The pre-flight assistance information may contain the best matching path(s) among the ones provided by the USS/UTM in step 4 or potential flight path information if candidate flight path(s) is not provided in Step 4.

13.	The NEF responds to the USS/UTM with the pre-flight assistance information.

14.	The USS/UTM derives the flight path(s) using the retrieved pre-flight assistance information and flight planning mechanism(s); these mechanisms are outside the scope of 3GPP.

15.	(via application layer) The USS/UTM responds to the UAV with the planned flight path(s).

>>>> Next Change <<<<

### 5.12.3	Procedure for NEF Assisted In-flight Monitoring

Figure 5.12.3-1: Procedure for NEF Assisted In-flight Monitoring

NOTE:	The NFs in Figure 5.12.3-1 represents all the other NFs beside GMLC and NEF/UAS NF involved in the procedure.

1.	The USS/UTM sends an Nnef_UAVFlightAssistance_Create request with an indication to provide the in-flight monitoring of a target UAV to NEF. The request includes an identifier of the UAV (e.g. GPSI) and information about the flight path(s), monitoring mode (e.g. UAV location monitoring, flight trajectory monitoring, flight environment monitoring, Ranging/Sidelink positioning location monitoring, QoS Sustainability analytics), report requirements (e.g. report format (event triggered or periodically), assistance information (e.g. the monitoring results, information generated from the monitoring results)). Additionally, the request may include the planned flight path(s) if the flight trajectory is set for the monitoring mode, a list of UAVs in the areas of interest if the flight environment is set for the monitoring mode, preset value for distance if the monitoring mode is set  to Ranging/Sidelink positioning.

2.	The NEF maps the parameters in the Nnef_UAVFlightAssistance_Create request from the USS/UTM to information used by the 3GPP system. The NEF determines services needed for the message and relevant NFs, e.g. NEF service on UAV tracking (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), GMLC service (Ranging/Sidelink Positioning location), NWDAF analytics service (QoS Sustainability Analytics).

3.	If the NEF is not able to provide the requested information for in-flight monitoring, the NEF rejects the USS/UTM’s request for the in-flight monitoring, and the NEF may include in the Nnef_UAVFlightAssistance_Create response the details about the reason for rejecton if available.

4.	For UAV’s location monitoring, the NEF executes Steps 2-5 from clause 5.3.2 to obtain the location of the target UAVs.

5.	For flight trajectory monitoring, the NEF executes steps 2-4 from clause 5.3.3. Upon to the request message in step 1 for reporting, the NEF may compare the monitored flight path with the planned flight path (if provided in step 1) and generates a report about whether the UAV is following the assigned path/trajectory.

6.	For flight environment monitoring, the NEF executes steps 2-5 from clause 5.3.4 to obtain the information of aerial UEs presence in the same geographic area as the target UAV.

7.	For QoS Sustainability analytics, the NEF requests the NWDAF service as described in clause 6.9.1 of TS 23.288 [20]. The subscribe/request message include an identifier of the UAV (e.g. GPSI) obtained in step 4 and the UAV’s flight path information; other parameters included in the request are described in clause 6.9.1 of TS 23.288 [20]. The list of UAVs in the area of interest is obtained in Step 8 and it may be used by the NEF to request the NWDAF on QoS Sustainability analytics.

8.	For location monitoring via Ranging/Sidelink positioning service, the NEF requests the GLMC service for Ranging/Sidelink Positioning location results as described in clause 6.20.4 of TS 23.273 [8], the procedure of SL-MT-LR for periodic, triggered Location Events (steps 1-20 for initiation of the monitoring, steps 21-31 for periodic monitoring). If the NEF does not have a list of UAVs in the area of interest, the NEF obtains the list of UAVs in the area of interest in step 6 and use the obtained list as an input to request GLMC service for Ranging/Sidelink Positioning location.

9.	The NEF notifies the USS/UTM using an Nnef_UAVFlightAssistance_Notify about the monitoring results (e.g. the location of the target UAV, indication on whether the UAV follows the flight path, the target UAV’s presence in areas of interest, information about whether any other aerial UE/UAV is present in the same geographic area as the target UAV, distance to other UAVs and whether this distance is smaller than the preset value(s)).

Steps 4-9 may be repeated for report monitoring results periodically.

>>>> End of Changes <<<<