3GPP TSG-SA3 Meeting #119	S3-244624

Orlando, US, 11 -15 November 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Analysis on Solution2

# Document for:	Approval

Agenda Item:	4.12

# 1	Decision/action requested

SA3 is kindly asked to review and approve the proposed modifications on LTM Solution#2.

# 2	References

[1]	S3-244324 draftCR on NRMob

# 3	Rationale

The Solution#2 of [1] still holds editors notes and the evaluation is outstanding, ergo, this contribution is for solving the Editor’s Notes and it is providing the evaluation description.

# 4	Detailed proposal

******************* Start of Change ****************

## 3.2	Solution #2: Inter-CU LTM

### 3.2.1	Introduction

This solution describes the Option 4 as described by the RAN document [3] and is referring to the Key Issue 1.

### 3.2.2	Solution details

The inter-CU LTM procedure description will be split into two phases, i.e., first comes the LTM preparation phase and second is the LTM cell switch execution.

Figure 3.2.2-1: Inter-CU LTM Procedure (overview)

1. The UE has already connected to the source gNB and is in UE connected state. The AS and NAS security has been activated and are applicable to traffic that is going between UE and Network.

2. void

3. The UE is sending measurement report.

4. The source gNB is preparing the LTM cell switch candidates, i.e., is computing the keys per candidate target gNB.

Horizontal Key Preparation:

Compute KgNB* = KNG_RAN* = (targetPCI, ARFCN, KgNB)

Example: { horiz._KgNB*_1, KgNB}

{ horiz._KgNB*_2, KgNB}

In case of Vertical Key Preparation:

Vertical if unused {NH, NCC} is available: Compute KgNB* = KNG_RAN* = (targetPCI, ARFCN, NH)

Example: { vert._KgNB*_1, NCC=2 }

{ vert._KgNB*_2, NCC=3 }, { vert._KgNB*_3, NCC=4 }

5./6. The source gNB is triggering handover preparation procedure over XnAP. The source gNB is sending to every of the candidates the corresponding AS security information (i.e., Key NG_RAN* and NCC). The XnAP handover preparation procedure is according to [4]. The sharing of the AS security information will enable the target gNB to run horizontal key derivation at handover unless otherwise requested.

Rationale: The Handover Request message as specified by [4] is providing the capability for the transfer of the KgNB* = KNG-RAN* and the corresponding NCC value. This message can be used to transfer the computed keys independent whether there will be horizontal or vertical key derivation.

7./8. The source gNB is triggering RRC reconfiguration procedure and is includuding the next NCC value (e.g., in case of horizontal keying, NCC=1).

9. The UE is periodically sending measurements.

10. The source gNB is deciding for inter-CU LTM cell switch and is sending a MAC CE cell switch command to the UE (Step 11).

12. The UE is generating new keys based on NCC indication. Depending on the stored next NCC value UE will do vertical or horizontal key derivation (in this example with NCC=1 the UE will do horizontal keying.).

13. The UE is responding with a reconfiguration complete.

14. The target gNB is sending a NGAP path switch towards the AMF for the generation of a new { NH, NCC } pair.

15. void

16. void

17./18. The target gNB is now the new source gNB and is therefore computing new keys and is sharing those with the inter-CU candidates. The role and corresponding features of a source gNB is moving to the new source gNB. After the (former) source gNB has received the XnAP Handover Request Acknowledgement message, then all the computed keys (see Step4) are replaced with the new keys the target gNB has indicated.

19. The new target gNB (former source gNB) is discarding all unused NCC values which have been maintained because of the (former) source gNB role.

20./21. The target gNB is now sending a RRC reconfiguration message to the UE that includes the next NCC value (NCC=2). The UE is responding with related complete message.

22. The NGAP path switch procedure is triggering the computing of new { NH, NCC } pair at the AMF. The AMF indicates the pair to target node.

Editor’s Note: Evaluation is needed for the subsequent LTM in case of an L3 handover.

### 3.2.3	Evaluation

This solution addresses the security requirements of KI #1.

This solution is RRC-based and in addition is making reuse of NRMob means by creating the LTM cell switch candidates and the corresponding keying material. By doing this, each of the candidate target gNB is receiving their (individual) key material and would be ready in case the UE attempts to connect.

The solution is supporting horizontal and vertical key derivation.

The key-lifecycle management does foresee that the (former) source gNB must delete all computed keys after a certain retention period. The retention period can be configurable.

******************* End of Change ****************