3GPP TSG-SA WG3 Meeting #119	S3-244726

# Orlando, US; 11st – 15th November 2024

Source:	China Unicom, ZTE Corporation

Title:	Discussion on PLMN ID setting in the 3gpp-Sbi-Originating-Network-Id header in the Indirect Network Sharing case during UE authentication

Agenda item:	3.1

Document for:	Discussion

1.	Introduction

1.1 SA2 conclusion for Indirect Network Sharing

SA2 has completed the work of TEI19_NetShare (WID: SP-240488, UID: 1030011). This WID introduces the Indirect Network Sharing deployment between hosting operator (i.e. shared network operator) and participating operator (see clause 5.18.1 of TS 23.501). In this deployment, the communication between the shared RAN and the core network of the participating operator is routed through the core network of the hosting operator that connects to the shared RAN.

Indirect Network Sharing for 5G system, including UE, shared RAN and CP NFs of hosting operator, shall support each participating operator to use more than one PLMN ID. For Indirect Network Sharing, the shared RAN is broadcasting multiple PLMN IDs, including the PLMN ID which represents the hosting operator and the PLMN IDs which represent participating operators. A UE from a participating operator can select the PLMN ID representing the participating operator in the shared RAN area based on existing procedures.

Based on the conclusion of SA2 (see clause 5.18.2 and clause 5.18.5 of TS 23.501), three PLMN IDs are involved in this deployment as follows:

-	Selected PLMN ID selected by the UE from the broadcast information in the shared RAN area, which represents the participating operator;

-	PLMN ID of the participating operator (i.e. HPLMN ID).

-	PLMN ID of the hosting operator (i.e. VPLMN ID).

Specifically, the shared RAN will broadcast the PLMN ID representing the participating operator (i.e. selected PLMN ID) including three broadcast options:

- HPLMN ID of the UE, or

- EHPLMN ID of the UE, or

- the PLMN ID different from HPLMN ID and EHPLMN IDs (dedicated PLMN ID for Indirect Network Sharing).

Which broadcast option to be used is determined based on the coordination and agreement between the hosting operator and the participating operator. And based on SA2 conclusion (see clause 5.18.1 of TS 23.501 or below), during the UE authentication procedure, the selected PLMN ID is used to form the Serving Network Name.

The serving AMF determines the serving PLMN ID as follows:

-	For procedures involving the Network Function in the participating operator network (e.g. AUSF), the AMF sets the serving PLMN ID to the selected PLMN ID.

-	For procedures involving the Network Function only in the hosting operator network (e.g. PCF), the AMF sets the serving PLMN ID to the PLMN ID of the hosting operator.

1.2 CT4 discussion on 3gpp-Sbi-Originating-Network-Id header for Indirect Network Sharing

In CT4#125, based on SA2 conclusion, how to set the PLMN ID in the 3gpp-Sbi-Originating-Network-Id header in the Indirect Network Sharing case during the UE authentication has been discussed, three options were under discussion:

- Option #1: The PLMN ID set in the 3gpp-Sbi-Originating-Network-Id header by the source NF of the hosting operator network is the selected PLMN ID which represents the participating operator. Alignment with the Serving Network Name.

- Option #2: The PLMN ID set in the 3gpp-Sbi-Originating-Network-Id header by the source NF of the hosting operator network is the PLMN ID of the hosting operator (i.e. VPLMN ID).

- Option #3: Both Option #1 and Option #2 are ok based on the coordination and agreement between the hosting operator and the participating operator.

But no conclusion has been reached considering that this may involve the security aspect during CT4#125. Therefore, CT4 would like SA3 to check their specifications and give the feedback for guidance.

2.	Observation and conclusion

As specified in TS 33.501:

In the clause 5.9.3.2 of TS 33.501 The receiving SEPP shall be able to verify whether the sending SEPP is authorized to use the PLMN ID or SNPN ID in the received N32 message. Sending SEPP behavior for the 3gpp-Sbi-Originating-Network-Id header: - If the sending NF or the SCP has inserted the 3gpp-Sbi-Originating-Network-Id header in the signaling message (service/subscription request or notification message), the sending SEPP shall compare the PLMN ID or SNPN ID in the 3gpp-Sbi-Originating-Network-Id header in the received signaling message with the PLMN ID(s) or SNPN ID(s) that the sending SEPP represents by its certificate. - If the PLMN ID or SNPN ID does not match with any of the PLMN IDs that the sending SEPP represents, the sending SEPP shall discard the received signaling message. - If the PLMN ID or SNPN ID matches with any of the PLMN IDs that the sending SEPP represents, the sending SEPP shall forward the signaling message to the receiving SEPP. - If the sending NF and the SCP have not included the 3gpp-Sbi-Originating-Network-Id header in the signalling message, the sending SEPP shall include the 3gpp-Sbi-Originating-Network-Id header and send the updated signaling message to the receiving SEPP. - If the sending SEPP only represents one PLMN ID or SNPN ID, the sending SEPP shall insert the 3gpp-Sbi-Originating-Network-Id header with this ID. - If the sending SEPP represents multiple PLMN IDs or SNPN IDs, it is up to configuration and deployment to determine which PLMN ID or SNPN ID value should be included in the header. Receiving SEPP behavior for the 3gpp-Sbi-Originating-Network-Id header: - The receiving SEPP shall check whether the 3gpp-Sbi-Originating-Network-Id header included in the signalling message belongs to the sending SEPP’s own PLMN or SNPN. It does this by verifying that the asserted PLMN ID in the 3gpp-Sbi-Originating-Network-Id header matches one of the sending SEPP's own PLMN ID(s) or SNPN ID(s) either in the N32-f context, the sending SEPP's certificate, or a locally configured list of PLMN IDs or SNPN-IDs that the sending SEPP represents. - If the 3gpp-Sbi-Originating-Network-Id header does not match with any of the PLMN IDs or SNPN IDs belonging to the peer sending SEPP, the receiving SEPP shall discard the received signaling message. - If the 3gpp-Sbi-Originating-Network-Id header matches with any PLMN ID of the PLMN or SNPN IDs belonging to the peer sending SEPP, the header is successfully verified, and the receiving SEPP shall forward the received signaling message to the target NF. In the clause 5.9.3.5 of TS 33.501 The NF that sends a signalling message (service/subscription request or notification message) shall include its PLMN ID or SNPN ID in the 3gpp-Sbi-Originating-Network-Id header. If an NF supports multiple PLMN IDs or SNPN IDs, the sending NF shall include the PLMN ID or SNPN ID in the 3gpp-Sbi-Originating-Network-Id header on behalf of which the message is sent. The handling of the PLMN ID or SNPN ID in the 3gpp-Sbi-Originating-Network-Id header at the receiving NF is up to configuration and deployment. In the clause 6.1.2 of TS 33.501 Upon receiving the Nausf_UEAuthentication_Authenticate Request message, the AUSF shall check that the requesting SEAF in the serving network identified by the 3gpp-Sbi-Originating-Network-Id header specified in TS 29.500 [74] is entitled to use the serving network name in the Nausf_UEAuthentication_Authenticate Request. For the case that the 3gpp-Sbi-Originating-Network-Id header is not included, the AUSF may authorize the serving network based on its name available in the Nausf_UEAuthentication_Authenticate Request message.

The Indirect Network Sharing case is a inter PLMN case and needs the interconnection between the SEPPs. But differing to roaming case, the hosting operator and the participating operator have the tight coordination and agreement to deploy the Indirect Network Sharing which means that the PLMN ID check in the 3gpp-Sbi-Originating-Network-Id header can be based on the local configuration. For instance, the receiving SEPP from the participating operator can check the 3gpp-Sbi-Originating-Network-Id header included in the signalling message belongs to the sending SEPP’s own PLMN based on a locally configured list of PLMN IDs that the sending SEPP represents and the AUSF from the participating operator can check the serving AMF in the serving network identified by the 3gpp-Sbi-Originating-Network-Id header is entitled to use the serving network name in the Nausf_UEAuthentication_Authenticate Request during the UE authentication based on the local configuration.

Observation: In the SA3 specs, the receiving SEPP and AUSF can check the 3gpp-Sbi-Originating-Network-Id header based on the local configuration during the UE authentication. In the case of Indirect Network Sharing, which PLMN ID is included in the 3gpp-Sbi-Originating-Network-Id header can be determined based on the coordination and agreement between the hosting operator and the participating operator.

Conclusion: Therefore, the PLMN ID in the 3gpp-Sbi-Originating-Network-Id header set by the source NF of the hosting operator network can be the selected PLMN ID which represents the participating operator or the PLMN ID of the hosting operator (i.e. VPLMN ID) during the UE authentication.

3.	Proposal

It is proposed to discuss the observation and conclusion in clause 2. Based on the conclusion, the LS response (S3-244727) is proposed to clarify how to set the PLMN ID in the 3gpp-Sbi-Originating-Network-Id header in the Indirect Network Sharing case during the UE authentication.