3GPP TSG-SA3 Meeting #119	S3-244750

Orlando, USA 11 - 15 November 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2068 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for HONTRA procedure with respect to failure cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | HN_Auth |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This is the mirror of a change in an earlier release. In agreed CR C4-244376, CT4 has agreed new different types of failure causes for Reauth Notifications. This CR is to align HONTRA procedure accordingly. In received LS C4-244496/S3-244632, there is confusion about case#5. This CR is also to clean up the HONTRA procedure to address the misalignment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clean up HONTRA procedure in response to LS case#5 Update HONTRA procedure with new different types of failure causes for case#2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

6.1.5.2	Security mechanisms

The UDM may initiate primary authentication based on procedures initiated by the UE (e.g. UE registration in 5GC) or towards the UE (e.g. SoR/UPU) or events from other NFs, considering the local policy into account as well.

Figure 6.1.5.2-1 Home Network triggered primary authentication procedure

Step 0a and step 0b are the pre-requisites of the whole procedure.

0a.	The UDM may be pre-configured with an operator authentication  policy in order to determine when to trigger a primary authentication procedure.

0b.	The UE registers to the network. As part of the registration, the serving AMF registers the UE with the UDM via the Nudm_UECM_Registration as per TS 23.502 [8], clause 4.2.2.2.2. The AMF shall provide a callback URI within the AMF registration for the UDM to create an implicit subscription to later notify the AMF for potential home network triggered re-authentication using the Nudm_UECM_Re-AuthenticationNotification service operation as in step 2.

1a-c.	The UDM decides itself based on events (e.g., SoR/UPU or NF requests such as AAnF requests as defined in TS 33.535 [91]) or authentication policy and performs home network triggered primary authentication as described in the following steps. The NF such as the AAnF considers based on operator's local authentication policy described in TS 33.535 [91] to send Nudm_UECM_AuthTrigger request to the UDM for primary authentication using the UDM services as described in clause 14.2.6. The NF may send a Nudm_UECM_AuthTrigger Request message to the UDM with the SUPI of the target UE. The UDM may acknowledge the request with an Nudm_UECM_AuthTrigger Response to the NF.

NOTE A:	For the NF (e.g., AAnF) request event, the UDM can decide not to proceed with triggering the primary authentication based on the UDM’s local authentication policy. In case of AAnF being the NF, as AAnF sets the AF key expiry based on operator’s local authentication policy, no frequent AF key expiry can happen and there is no risk of signalling overload. Based on a received event and the local operator authentication policy, if there is no ongoing primary authentication for the UE, and if the UDM determines to trigger the primary authentication, the UDM determines the serving AMF/SEAF of the target UE.

If there are different AMFs registered in the UDM for different access,  the UDM shall select one AMF to perform the re-authentication. The criteria for selecting the AMF are dependent of the local UDM authentication policy.

NOTE 1: 	The reasons for the UDM determining that the UE needs to be authenticated can be different. For example, the UDM can determine to initiate a primary authentication when the AMF registers the UE upon the Registration procedure during the mobility from EPC or when SoR/UPU counters are about to wrap around, or when required based on authentication policy, or based on the request from AAnF. The UDM behaviour is determined by operator  policy which takes into account the support of certain features in the PLMN. For example, if the HPLMN does not support the SoR/UPU feature, then SoR/UPU counter wrap around will not happen, and primary authentication will not be required for this case.

2. 	The UDM sends a Nudm_UECM_Re-AuthenticationNotification message to the AMF/SEAF with the UE’s SUPI.

3. 	After receiving the Nudm_UECM_Re-AuthenticationNotification  message from the UDM, the AMF/SEAF shall decide whether to run the primary authentication procedure based on its own local authentication policy, and the UE state.

If the AMF/SEAF determines that it cannot run a primary authentication due to local policy or due to the UE being under handover or registration with AMF change, the AMF/SEAF sends the authentication response message to the UDM with a failure cause else it acknowledges the request.

Upon receiving a failure from the AMF, the UDM may wait before checking if another AMF is available over the other access in order to retry Step 2.

If the AMF/SEAF acknowledged the request but the AMF/SEAF is not able to initiate the primary authentication towards the UE (e.g. if UE is not reachable), the AMF/SEAF shall set the authentication pending flag. When UE re-attaches to the same AMF or becomes reachable, the AMF checks the authentication pending flag and performs the reauthentication if needed. Once UE reauthentication is done, the AMF resets the authentication pending flag.

NOTE 2:	In the case that the UE attaches to a new AMF, the new AMF will register to the UDM using the Nudm_UECM_Registration message. In this case, the UDM can determine again on whether to trigger the primary authentication as described in 1b.

4. 	The AMF/SEAF starts the primary authentication procedure as defined in clause 6.1.2 of the present document.

The UDM may execute other procedures (e.g. SoR/UPU) depending on the reason that motivated the UDM triggered (re-)authentication procedure in step 1.

*** END OF CHANGES ***