3GPP TSG-SA3 Meeting #119	S3-244936

# Orlando, USA  11 - 15 November 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Informative annex for N6 delay measurement protocols

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is required to approve this informative annex proposed for KI#1.1 in this contribution.

# 2	References

[1]	3GPP TR 33.749 v0.3.0 “Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3”

# 3	Rationale

The proposal is intended to clarify the security capabilities of the main protocols leveraged to measure the N6 delay between UPF and EAS. The protocols are defined in IETF RFCs corresponding to IP Performance Metrics (IPPM), such as OWAMP (RFC 4656), TWAMP (RFC 5357), STAMP (RFC 8762), or ICMP (RFC 792).

An informative annex that extracts the security capabilities and associated parameters available in the IETF RFCs in each protocol will facilitate the reading of the 3GPP technical report, and clarify how the protocols comply with the recommendations captured in the conclusion of KI#1.1.  It can also be referred for stage 3 implementation.

# 4	Detailed proposal

******************START of CHANGES****************************************

# Annex X (Informative): Security capabilities and procedures for IETF IP Performance Metrics defined protocols

The IETF RFCs corresponding to the IP Performance Metrics (IPPM) defined protocols, such as OWAMP (RFC 4656) [2], TWAMP (RFC 5357) [3] and STAMP (RFC 8762) [4], are leveraged for N6 delay measurement.

Each of those protocols support a set of security capabilities such as authentication, confidentiality and integrity protection, replay protection, etc. Table X-1 summarizes the security mechanisms specified by IETF for each protocol to support the most relevant security capabilities.

Table X-1: Security capabilities of IPPM protocols leveraged for N6 delay measurements

|  | OWAMP (RFC 4656) | TWAMP (RFC 5357) | STAMP (RFC 8762) |
| --- | --- | --- | --- |
| Authentication | HMAC | HMAC | HMAC |
| Confidentiality | AES | AES | Transport layer |
| Integrity | HMAC | HMAC | HMAC |
| Replay Protection | sequence numbers or timestamps | sequence numbers or timestamps | sequence numbers or timestamps |
| DoS prevention | With authentication | With authentication | With authentication |
| TLS support | TLS is not recommended in RFC | TLS is not recommended in RFC | DTLS/IPSec for encryption |

Table X-2 indicates the exchanged security parameters in control phase for OWAMP and TWAMP, and the required configuration parameters to enable the security mechanisms in each protocol. note that STAMP does not require a control phase.

Table X-2: Exchanged security and configuration parameters to enable security mechanisms

|  | OWAMP (RFC 4656) | TWAMP (RFC 5357) | STAMP (RFC 8762) |
| --- | --- | --- | --- |
| Exchanged Security Parameters | Security mode, KeyID, Session keys, Count, Salt, Challenge, IVs | Security mode, KeyID, Session keys, Count, Salt, Challenge, IVs | N/A |
| Configuration Parameters | Shared secret Encryption Algorithm (AES). HMAC Algorithm (SHA-256). Security Policies to limit resource use, such as network capacity and memory | Shared secret Encryption Algorithm (AES). HMAC Algorithm (SHA-256). Security Policies to limit resource use, such as network capacity and memory | Shared Secret Security mode HMAC Algorithm (SHA-256). |

The security procedures per protocol are represented in the following call flows.

Figure X-1: Security procedure for OWAMP

Figure X-2: Security procedure for TWAMP

Figure X-3: Security procedure for STAMP

NOTE: SeqNo and/or Timestamp may be encrypted based on the security mode.

******************END of CHANGES****************************************