3GPP TSG-SA3 Meeting #119	S3-245273

Orlando, US, 11 -15 November 2024	revision of S3-245053

# Source:	Ericsson

# Title:	Resolving ENs in solution #9

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is proposed to approve the following updates for solution #9 in TR 33.749.

# 2	References

[1]	3GPP TR 33.749: "Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3".

# 3	Rationale

This document resolves the ENs in solution #9 by providing more details.

Editor’s Note: Whether the UE ID server needs to authenticate the EEC is FFS.

The following text in the solution details already clarifies the point.

"The EEC does not need to authenticate with the server since the server issues the UE ID token, which does not include sensitive information, to the EEC running on the UE who has already executed the primary authentication."

Editor’s Note: How to deliver root certificate and/or certificate of UE ID server to EEC for authentication of UE ID server is FFS.

The EEC can be configured with the root certificate or certificate of the UE ID Server, or the Internet (public) PKI mechanism can be used.

Editor’s Note: It is FFS how the UE ID server gets the UE private IP address used for verification.

The UE ID Server can fetch the IP address from the IP packets received from the EEC. As stated in the solution details, "The UE ID Server is an AF function located inside the HPLMN trust domain and before the execution of NAT", so received IP address in the IP packets is the private IP address.

Editor’s Note: How the UE ID Server identifies the UE by using the private IP address of the PDU session is FFS.

The UE ID Server can use the Nbsf_Management_Discovery service operation with UE address and IP domain to retrieve the session binding information of the UE.

Editor’s Note: The UE ID token details (such as whether it is a signed token or random number) is FFS.

Since the issuer and resolver of the UE ID token is the UE ID Server, the details of the UE ID token (e.g., whether it is a signed token or a random number) can be left to implementation.

Editor’s Note: Further evaluation is FFS.

Further evaluation is provided and all the ENs in the solution are resolved, so we propose to remove the further evaluation EN.

# 4	Detailed proposal

It is proposed to approve the following change to TR 33.749 [1].

***START OF THE CHANGE***

## 6.9	Solution #9: Simplified UE ID token based solution

### 6.9.1	Solution overview

This solution proposes usage of UE ID token to address the security requirements of key issue #2.1. With the usage of UE ID token, there will be no need to share private IP address with EEC and other entities in the EC architecture. In section 6.9.2, the generic view of the solution is presented and how the solution can work for the following special cases of NEF API invocation by the EAS, NEF API invocation by the EES, and EES API invocation by the EAS is explained.

### 6.9.2	Solution details

Generic representation of the solution is presented in Figure 6.9.2-1.

Figure 6.9.2-1: Generic view of UE ID token based solution

Step 1: The client obtains a UE ID token from the UE ID server which is located inside the operator domain. UE ID server can identify the UE by using the private IP address of the PDU session. The UE ID Server can fetch the IP address from the IP packets received from the Client. The UE ID Server can use the Nbsf_Management_Discovery service operation with UE address and IP domain to retrieve the session binding information of the UE. For the UE ID Server service consumption by the client, the UE where the client is running on needs to execute the primary authentication successfully and have a PDU session. The client can authenticate the UE ID Server by using server certificate. The Client needs to be configured with the root certificate or certificate of the UE ID Server, or the Internet (public) PKI mechanism can be used. The client does not need to authenticate with the server since the server issues the UE ID token, which does not include cleartext sensitive information, to the client running on the UE who has already executed the primary authentication.

NOTE:	Since the issuer and resolver of the UE ID token is the UE ID Server, the details of the UE ID token (e.g., whether it is a signed token or a random number) can be left to implementation.

Step 2: The client forwards the token to the AF.

Step 3: The AF invokes a NEF service and use the UE ID token as UE identifier.

Step 4: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 5: The UE ID server returns the requested information.

Step 6: The NEF interacts with other NFs as per existing NEF service procedures as per TS 23.502.

Step 7: The NEF returns the result of the NEF service execution.

Note that if the AF is a trusted AF, it can directly interact with the UE ID Server to identify the UE and the obtain required information such as IP address, and then invoke the NF/NEF directly with the required inputs.

Application of the solution to the case of NEF API invocation by the EAS/EES is presented in Figure 6.9.2-2.

Figure 6.9.2-2: The EAS/EES invokes NEF API

Step 1: The AC/EEC obtains a UE ID token from the UE ID server.

Step 2: The AC or the EEC respectively forward the token to the EAS or the EES.

Step 3: The EAS/EES invokes a NEF service and use the UE ID token as UE identifier.

Step 4: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 5: The UE ID server returns the requested information.

Step 6: The NEF interacts with other NFs as per existing NEF service procedures as per TS 23.502.

Step 7: The NEF returns the result of the NEF service execution.

Application of the solution to the case of EES API invocation by the EAS is presented in Figure 6.9.2-3.

Figure 6.9.2-3: The EAS invokes the EES API

Step 1: The AC obtains a UE ID token from the UE ID server.

Step 2: The AC forwards the token to the EAS.

Step 3: The EAS invokes a EES service and use the UE ID token as UE identifier.

Step 4: The EES invokes the NEF API by sending the UE ID token.

Step 5: The NEF interacts with the UE ID server to resolve the UE ID token into UE information (e.g., private IP address, MSISDN, SUPI) needed for the operation.

Step 6: The UE ID server returns the requested information.

Step 7: The NEF sends the UE ID to the EES.

Step 8: The EES returns the result of the EES service execution.

### 6.9.3	Solution evaluation

Solution applicability evaluation:

- The solution addresses both the privacy and prevention against spoofing attacks requirements of key issue #2.1.

- The solution works also for the case that the NAT operation is performed after the UPF.

- The solution does not require interaction between AC and EEC.

- The EAS only learns the UE ID token, so there is no need to assign AF specific GPSI for the EAS.

Solution impact evaluation:

- The solution does not have any impact on the ME and the USIM. There is impact on the Client (the EEC and the AC) such that the Client needs to be able to access the UE ID Server, obtain the UE ID token and provide the token to the EAS/EES.

- The solution has impact on NEF services. To reduce the impact the UE ID token can be provided as input in any NEF Service that takes GPSI as input (as NAI/External Id format).

- The procedure in TS 23.558 clause 8.6.5.2 needs minor update to allow the EEC to use the UE ID token instead of UE IP address.

- A new AF (UE ID Server) needs to be introduced.

- A new interface between the UE ID Server and the NEF for the NEF to be able to consume the UE ID Server service is required.

- EES Eees_UEIdentifier API and NEF's Nnef_UEId API needs minor update to allow the use of UE ID token as an input parameter.

***END OF THE CHANGE***