3GPP TSG-SA3 Meeting #119	draft_S3-244769-r1

Orlando, USA 11 - 15 November 2024

- Source:	ZTE Corporation

- Title:	Remove the EN in stable conclusions

- Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

This contribution proposes to update conclusions in TR 33.745.

# 2	References

# 3	Rationale

The conclusions of KI#2, KI#4, KI#7 and KI#8 are stable, remove the EN “Further conclusions are FFS” in these conclusions.

# 4	Detailed proposal

************* Start of 1st Changes *************

## 7.2	Key Issue #2: Authentication aspect of 5G NR Femto connecting to the operator network

It is agreed to consider the following principles for the normative work:

-	The SeGW network element can be deployed at the border of operator’s security domain and the security features for device mutual authentication specified in TS 33.320 [2] can be inherited with the following changes:

-	Replace H(e)NB with 5G NR Femto.

-	IKEv2 EAP-AKA-based authentication can be as an option for authentication between 5G NR Femto and SeGW.

************* End of 1st Changes *************

************* Start of 2nd Changes *************

## 7.4	Key Issue #4: UE access control

It is agreed to consider the following principles for the normative work:

-	The CAG based access control mechanism for a CAG capable UE accessing the 5G NR Femto is performed in the core network.

-	CAG verification for UE access control defined in TS 23.502[11], TS 38.413[12] is reused.

-	No solution is needed for normative work for interworking between CAG and CSG cells in this release.

************* End of 2nd Changes *************

*********** Start of 3rd Changes *************

## 7.7	Key Issue #7: Direct link between 5G NR Femtos

It is agreed to consider the following principles for the normative work:

-	The security aspects for direct links between 5G NR Femtos specified in TS 33.320 [2] can be inherited with the following changes:

-	Replace HNB/H(e)NB with 5G NR Femto.

-	Replace Iurh/X2 interface with Xn interface.

************ End of 3rd Changes *************

************ Start of 4th Changes *************

## 7.8	Key Issue #8: 5G NR Femto management system accessible on the public internet

It is agreed to consider the following principles for the normative work:

-	The security features for H(e)MS accessible on public Internet specified in TS 33.320 [2] can be inherited with the following changes:

-	Replace H(e)NB with 5G NR Femto.

-	Replace H(e)MS with 5G NR Femto MS.

************* End of 4th Changes *************