3GPP TSG-SA3 Meeting #119	draft_S3-244982-r3

Orlando, US, 11 -15 November 2024

- Source:	Xiaomi, vivo, Huawei

- Title:	LTM: Partial Conclusion on Key Issue 1

- Document for:	Approval

Agenda Item:	4.12

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed partial conclusion on key issue #1 for Inter-CU LTM security.

# 2	References

[1]	S3-244981		LTM: Further Update to Solution 8

[2]	S3-244324		Living document on NR mobility enhancement

# 3	Rationale

Based on S3-244981 [1], the pCR proposes partial conclusion on the change of security algorithms and UP security policy for key issue #1 in S3-244324 [2].

# 4	Detailed proposal

*************** Start of the Change ****************

# 4	Conclusions

4.X	Conclusion for key issue #1

For the case where CU is acting as MN and DC is not configured, it is agreed that the normative work is based on the following principles:

-	For the parameters in AS security context, which are not changing per inter-CU LTM handover (including UE’s 5G security capabilities, selected AS security algorithms, UE’s UP security policy, UE’s UP security activation status):

-	If the gNB receives mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) in the Path-Switch Acknowledge message after LTM switch,

-	it initiates an intra-cell handover procedure which includes RRC Connection Reconfiguration procedure indicating the change of selected algorithms (as defined in clause 6.7.3.1) and/or UE’s UP activation status (as defined in clause 6.6.1) to the UE.

-	it initiates an LTM candidate preparation procedure to update the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs.

Editor’s Note: The conclusion for the parameters in AS security context, which are changing per each inter-CU LTM handover (including NCC value, keySetChangeIndicator) is FFS.

*************** End of the Change ****************