3GPP TSG-SA3 Meeting #119	S3-245175

Orlando, USA, 11 – 15 November 2024

- Source:	Xidian, OPPO

- Title:	Mutual Authentication Using AEAD for Inventory and Command case

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This document proposes a solution to address KI#5.

# 2	References

N/A

# 3	Rationale

It is proposed to add a mutual Authentication solution using AEAD for Inventory and Command case for KI#5 of TR 33.713.

# 4	Detailed proposal

Start of Change 1

## 6.Y	Solution #Y:  Mutual Authentication Using AEAD for Inventory and Command case

### 6.Y.1	Introduction

The solution addresses KI#5. It proposes a mutual authentication using AEAD between AIoT devices and the network. According to the conclusions in TR 23.700-13, the solution makes the following assumptions:

- An AIoT device shares a permanent identity (i.e., AIoT Device ID) and root key (i.e., K) with the network side or third party (e.g., UDM or Credential Holder's AAA server).

### 6.Y.2	Solution details

### The message flow of this solution is described below:

Figure 6.Y.2-1 Mutual authentication for security credentials stored in the UDM

  - Third-party AF sends inventory request to the AIoT function through NEF. The request can include an AIoT Device ID to page the device.

3-4. AIoT function generates a fresh nonce and sends the nonce to the AIoT devices through a Reader.

- AIoT device calculate KAF and authentication parameters.

  - AIoT device computes KAF using key derived function. The input parameters include the K, the Device ID and the nonce.

  - The device uses f2 or AES algorithm to compute RES. The input parameters include the K, the identity and the nonce.

  - Alternatively, the devices can use AEAD algorithm to compute a message authentication code to protect the authenticity of uplink data (i.e. Device ID). The input parameters include the KAF and uplink data (i.e. Device ID).

- The device sends inventory response (authentication request) including the identity, RES or message authentication code to the Reader.

- The Reader forwards the response to the AIoT function, including authentication request.

- The AIoT function sends the authentication request to the UDM, including nonce.

- The UDM computes KAF and verifies the RES or message authentication code.

  - computes KAF using key derived function. The input parameters include the K, the Device ID and the nonce.

  - retrieves the SQN and the K according to the ID.

  - computes the anonymity key (AK) using f5 or AES algorithm. The input parameters include the K and the fresh nonce.

  - computes MAC using f1 or AES function. The input parameters include the K, the SQN fresh nonce and the command message.

- The UDM sends KAF, AK⊕SQN and MAC to the AIoT function and increases the SQN by one.

- The AIoT function stores the KAF corresponding to the identity and sends the Command request to the Reader, the message includes AK⊕SQN and MAC.

- The Reader forwards the Command request to the AIoT device.

- The AIoT device computes the AK in the same way as the UDM and decrypts the concealed SQN. If the SQN is valid, then it verifies MAC.

  - Optionally AIoT device sends command response message.

Editor’s Note: How SQN is validated is FFS.

Editor’s Note: Use of Device ID in Inventory (i.e., paging) and whether it applies to a single or a group of AIoT devices is FFS.

Editor’s Note: Impact of using UDM on the network for credential storage and key generation is FFS.

Editor’s Note: How AES AEAD mode is used in the AKA procedure is FFS.

### 6.Y.3	Evaluation

End of Change 1