3GPP TSG-SA3 Meeting #119	draft_S3-245184-r4

Orlando, USA 11-15 November 2024								revision of S3-244723, merger of S3-244610, S3-245081

- Source:	US National Security Agency, OTD_US, Johns Hopkins University APL, MITRE-FFRDC, Ericsson, Motorola Mobility

- Title:	General Recommendations for Conclusion 2 Policy Enforcement

- Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

It is proposed to approve the proposed changes to TR 33.794 clause 8.2 conclusions for KI 2

# 2	References

[8] 	NIST Special Publication 800-207: "Zero Trust Architecture".

# 3	Rationale

This submission provides additions to conclusion 2 including some general recommendations.

# 4	Detailed proposal

*****Start Changes*****

- 8.2	Key Issue #2: Security mechanisms for policy enforcement at the 5G SBA

No normative work is needed for KI#2.

The informative guidance includes the following aspects:

Aspects on security policy enforcement (e.g., NF profile updates) for improved access control decisions considering security evaluation and monitoring results are described as follows:

- A policy enforcement point in SBA performs actions based on security evaluation results/outcome (e.g., in case of NF being identified/suspected to be compromised) during NF service requests (e.g., NF service discovery, access token requests, NF service update) and there may be more than one policy enforcement point in SBA as applicable.

- NF (i.e., service producer or service consumer) acts as policy enforcement point (i.e., in case of deployment A when no NRF and SCP involved as well as in general case of SBA service request/response as applicable)

- NRF acts as policy enforcement point (i.e., in case of deployment B where NRF is involved).

- NRF and SCP acts as policy enforcement points (i.e., in case of deployment C first NRF is involved in Service discovery and then SCP is involved during service request).

- SCP acts as a policy enforcement point (i.e., in case of deployment D).

NOTE 1: Deployment models i.e., Communication models for NF/NF services interaction are described in TS 23.501 [18].

- If SBA NF performs PEP functions (e.g. as described in NIST [8]) then following actions are needed:

- The PDP instructs the PEP i.e., actions to the NFs functioning as a PEP.

NOTE 2: PDP decides on the security issues/incidents and how the related responses were decided is left to MNO’s Implementation. It is upto the MNO to decide the security issues and the corresponding responses.

PEP takes actions according to (a).

*****End Changes*****