3GPP TSG-SA3 Meeting #119 	draft_S3-245206-r11

- Orlando, US, 11 -15 November 2024                     Merger of S3-244796, S3-244865, S3-245028, S3-245119

- Source:	China Telecom, ZTE, China Unicom, CATT, Nokia??, Nokia Shanghai Bell??, Ericsson??, Samsung??, Xiaomi communications??US National Security Agency??JHU/APL??Huawei??China Mobile??

- Title:	Conclusion to KI#2

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to add conclusion to KI#2.

# 2	References

# 3	Rationale

The contribution proposes to add conclusion to KI#2.

# 4	Detailed proposal

It is suggested to approve the following change.

*************** Start of the change ****************

## 8.2	Conclusion for KI#2: Dedicated NFs interacting with PLMN through SBA interface

- It is proposed to use one or more proxy entities at the border between PLMN and PNI-NPN operational domains. These proxy entities and the security capabilities of these proxy entities listed below can serve as a foundation for normative work.

- Transport layer protection solution (as defined in TS 33.501 clause 13.3) is supported for mutual authentication between the proxy entities and between each proxy entity and the NFs they serve.

- SBA authorization framework (as defined in TS 33.501 clause 13.4) is reused for authorization between NFs in the PLMN and PNI-NPN operational domains.

- Topology hiding to IP level (e.g., IP address or FQDN) and application level (e.g., target NFs in the discovery response, Callback URI in the payload of the messages) is supported.

- Message inspection and filtering, malformed message handling are supported.

NOTE: Specific method for above security capabilities c) and d) can be left to implementation.

Besides the security capabilities listed above, the part of the routing functionalities listed below also needs to be supported:

- Indirect Communication (see TS 23.501 clause 7.1.1 for details).

- Delegated Discovery (see TS 23.501 clauses 7.1.1 and 6.3.1 for details).

- Message forwarding and routing to destination NF/NF service.

- Message forwarding and routing to a next hop. (Required for bi-directional protection)

The function supporting above security functionalities c) and d) can be co-located with SCP or the above routing capabilities and security capabilities a) and b)in a new proxy entity. The new proxy entity should support the above routing capabilities and security capabilities a) and b) additionally.

- Security functionalities in the border entity in the PLMN operational domain is enforced, by configuring the border entity, based on operator policy, for serving the NF in the PNI-NPN operational domain.

*************** End of the change ****************