3GPP TSG-SA3 Meeting #118 	S3-243904

Hyderabad, India, 14 – 18 Oct 2024

Source:	Google

Title:	Evaluation update for solution #5 (Using ACME protocol for certificate enrolment)

Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approve the pCR to TR 33.776

# 2	References

[1] 3GPP TR 33.776 Study of Automatic Certificate Management Environment (ACME) for the Service Based Architecture (SBA) (Release 19)

# 3	Rationale

This contribution proposes a solution evaluation update for solution#5 in TR 33.776.

# 4	Detailed proposal

*** Start of 1st Change ***

## 6.5	Solution #5: Using ACME protocol for certificate enrolment

### 6.5.1	Introduction

This solution proposes to use the ACME protocol to address the requirements in key issue KI#4 (Certificate enrolment).

### 6.5.2	Solution details

#### 6.5.2.1 	Initial Trust

This solution can assume that the initial trust has already been established via the initial trust schema defined in TS 33.310 [3], which is briefly described as follows.

Figure 6.5.2.1.1 Initial trust schema

As depicted in Figure 6.5.2.1.1, Operation, administration and maintenance (OAM) system has a preestablished trust with Operator CA/RA. An operator CA/RA can be a trusted third-party CA/RA, with which the 5GS has a pre-established trust. The OAM can configure the 5G Core NF with a list of trust anchors and with a private/public key pair to be used for ACME account creation. Alternatively, the 5G Core NF can generate its own key pair.

Note that the Operator CA/RA behaves as an ACME server and the 5G Core NF acts as an ACME client.

#### 6.5.2.2	Certificate enrolment

Figure 6.5.2.2.1 describes the ACME certificate enrolment procedure for a 5G NF. Note that 5G Core NF can also be referred to as 5G NF.

Figure 6.5.2.2.1 – ACME certificate enrolment

1. 	The ACME client requests a certificate by sending a new order request for 5G SBA ACME Identifier to the CA’s newOrder resource using a POST request. 5G SBA ACME Identifier can be any ACME identifier shown to work with 5G SBA, e.g., Solution #1, Solution #2, Solution #3.

2.	The ACME server responds with a 201 (Created) response that includes authorization objects with challenges to be satisfied as described in RFC 8555[2].

3.	The ACME client checks the authorization objects within the response and completes the listed challenges before requesting the ACME server to sign the certificate as described in RFC 8555 [2]. Any challenge validation methods shown to work for 5G SBA can be included in this list.

4.	After the ACME client successfully completes the challenge validation procedure, it sends a Certificate Signing Request (CSR) to the ACME server.

5.	The ACME server issues the certificate and publishes it in the corresponding resource directory to the URL provided in the order object.

6.	The ACME client downloads the certificate by sending a POST-as-GET request to the certificate URL provided.

NOTE:	The 5G NF/client may proactively obtain authorization and may not have to perform challenge-response procedure in Steps 2 – Step 4 based on pre-authorization procedure described in clause 7.4.1 of RFC 8555 [2].

### 6.5.3	Evaluation

This solution addresses KI#4.

This solution impacts core network function (NF), OAM and service protocols in the 5G core network.

The solution outlines how certificate enrolment in 5G SBA may be performed using the ACME protocol [2] with any combination of ACME identifier type and       ACME challenge validation type that is      suited for use in 5G SBA deployments.

The 5G NF downloads the signed certificates using a TLS protected connection from a trusted URL the ACME server provides during the request.

If pre-authorized, the 5G NF does not need to perform additional authorization via a challenge-response process during certificate enrolment.

*** End of 1st Change ***