3GPP TSG-SA3 Meeting #118 	S3-243905
Hyderabad, India, 14 – 18 Oct 2024

Source:	Google

Title:	Evaluation update for solution #7 (Using ACME protocol for secure transport of messages)

Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approve the pCR to TR 33.776

# 2	References

[1]	3GPP TR 33.776 V0.4.0 Study of Automatic Certificate Management Environment (ACME) for the Service Based Architecture (SBA) (Release 19)

[2]	IETF RFC 8555: Automatic Certificate Management Environment (ACME), 2019

[3]	IETF Automated Certificate Management Environment (ACME) Renewal Information (ARI) Extension, 		Aug, 2024

# 3	Rationale

This contribution proposes a update in the solution evaluation for solution#7 in TR 33.776.

# 4	Detailed proposal

*** Start of 1st Change ***

## 6.7	Solution #7: Using ACME protocol for secure transport of messages

### 6.7.1	Introduction

This contribution addresses key issue #2.

### 6.7.2	Solution details

The solution assumes that the 5G NF is issued with the operator CA’s root certificate, which is used to validate the ACME server’s TLS certificate.

This solution is based on RFC 8555 [2] wherein the communication between ACME client and the ACME server are done over HTTPS for authentication  and confidentiality.

When an ACME client fetches a resource from an ACME server, the server authenticates the requester and verifies       any access control as described in RFC 8555 [2].

ACME for 5G SBA uses JWS based integrity protection as described in RFC 8555 [2].

ACME for 5G SBA uses nonces to protect messages against replay-attacks. An ACME server maintains a list of nonces that it has issued and requires any signed request from the client to carry such a nonce as described in RFC 8555 [2].

### 6.7.3 	Evaluation

This solution addresses KI#2.

This solution impacts 5G core network function and 5G OAM system.

All exchanges initiated by the ACME client meet the requirement for confidentiality,  integrity protection  and replay protection. Once the client has established initial trust, messages can be considered mutually authenticated.  Depending on the method of initial trust establishment, all messages could be considered mutually authenticated.

The server is always authenticated  to the client prior to sending any data from the client, and no certificates are issued until mutual authentication is established.

This solution only applies to the client-initiated exchanges which are necessary for ACME. Not all solutions will require exchanges initiated by the ACME server to another endpoint. Any solution which introduces such an exchange will need to provide further analysis of the security of the transport mechanism.

*** End of 1st Change ***