3GPP TSG-SA3 Meeting #118															           S3-244703

# Orlando, USA  11 - 15 November 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2062 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Kamf definition alignment for NAS count |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per Annex A.13, Kamf generation will use only 3GPP access related NAS count. However, it is not true. In Rel16, non 3GPP access was added and section 6.9.3 says If the source AMF determines to perform horizontal KAMF derivation, the source AMF shall derive a new key KAMF from the currently active KAMF and the uplink NAS COUNT value in the received Registration Request message However, A.13 says: Derivation of KAMF' from KAMF during mobility shall use the following input parameters. - FC = 0x72 - P0 = DIRECTION - L0 = length of DIRECTION (i.e. 0x00 0x01) - P1 = COUNT, - L1 = length of COUNT (i.e. 0x00 0x04) The input key KEY shall be KAMF. When KAMF' is derived in handover, DIRECTION shall be 0x01 and COUNT shall be the downlink NAS COUNT of the 3GPP access. When KAMF' is derived in idle mode mobility (i.e., mobility registration update), DIRECTION shall be 0x00 and COUNT shall be the uplink NAS COUNT of the 3GPP access used in the Registration Request. Therefore, section A.13 needs to be aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text is updated for A.13 for non-3GPP access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification, causing a confusion which access should be used for Kamf generation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES**********

# A.13	KAMF to KAMF'  derivation in mobility

Derivation of KAMF'  from KAMF during mobility shall use the following input parameters.

-	FC = 0x72

-	P0 =  DIRECTION

-	L0 = length of DIRECTION (i.e. 0x00 0x01)

-	P1 = COUNT,

-	L1 = length of COUNT (i.e. 0x00 0x04)

The input key KEY shall be KAMF.

When KAMF' is derived in handover, DIRECTION shall be 0x01 and COUNT shall be the downlink NAS COUNT of the 3GPP/non-3GPP access used in the Registration Request.

When KAMF' is derived in idle mode mobility (i.e., mobility registration update), DIRECTION shall be 0x00 and COUNT shall be the uplink NAS COUNT of the 3GPP/non-3GPP access used in the Registration Request.

************ END OF CHANGES*****************