3GPP TSG-SA3 Meeting #119	S3-244708

Orlando, USA  11 - 15 November 2024	(revision of S3-24xxxx)

# Source:	Nokia

# Title:	New (mini) WID on SUCI calculation failure alignment

# Document for:	Approval

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: WID on Security for SUCIErr

Acronym: SUCIErr_Sec

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  |  |  |  |
| Don't know |  |  | X |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

SUCI computation failure could happen because of multiple reasons. It could be wrong or missing parameter values, either due to misconfiguration or erasure of the parameter from the configuration file in the USIM. For whatever reason, when SUCI computation failure happens in the 5G USIM  an ‘error status word’ is returned to ME. What is expected behaviour of Mobile Equipment in this scenario is not defined in any of the specifications. Once this error happens, leaving the UE in the same state will cause,

- UE (and user) to not get any service,  (if UE is not able to generate the SUCI, then authentication cannot be performed, and UE will not be able to connect to network  to report this error to the network.)

- Revenue loss to operators

This kind of error can occur in multiple scenarios for the UE.

| Scenario | Consequence |
| --- | --- |
| Scenario#1: Home network updates the configuration related to SUCI but after that the UE moves to unregistered state. Therefore, no security context in NAS or in AS is available. | UE tries to trigger the registration request towards the 5GC but fails because SUCI calculation fails. So, UE cannot get the 5GC service. |
| Scenario #2:UE is registered and authenticated in 5GC. NAS and AS has the security context. Home network updates the configuration related to SUCI and now the SUCI cannot be calculated. | UE is already connected to the 5GC and NAS/AS security is in place. When AMF request for Identity request for SUCI, it will fail. |
| Scenario#3: Home network updates the configuration related to SUCI and then UE moves to EPC/4G. UE comes back in 5GC but registration to 5GC fails because SUCI calculation fails | UE can still use the EPC service, not 5GC |

Somehow UE should be able to come out of the error and inform to 4G/5G network of the exact error or missing parameter so that network can take corrective actions so that UE services in 5G can be resumed.

# 4	Objective

The main objective is:

- Specify the requirements for UE and 5GC to handle the SUCI calculation error in different scenarios.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 33.501 | Specify the requirements for UE and 5GC to handle the SUCI calculation error. | TSG #108 | This TS covers Stage 2 |

1 TU

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA3

# 8	Aspects that involve other WGs

CT WGs for protocol development

# 9	Supporting Individual Members

Supporting IM name

Nokia