3GPP TSG-SA3 Meeting #119	S3-244835

Orlando, US, 11 -15 November 2024

# Source:	Huawei, HiSilicon

# Title:	Adding evaluation to solution #4

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

Approve the pCR proposal for TR 33.700-22

# 2	References

[1]	3GPP TR 33.700-22

# 3	Rationale

This contribution provides evaluation to the solution #4 and addresses one evaluation EN.

# 4	Detailed proposal

pCR

***  START OF CHANGES  ***

## 6.4	Solution #4: resource owner authorized revocation

### 6.4.1	Introduction

This solution addresses revocation part of the "Key Issue #1.2: Resource owner authorization management ".

In TS 33.122 [4], it is stated

CCF can receive a revocation request message from the resource owner via the UE, resource owner function, web page etc.

In this solution, this revocation request message is described to complete the revocation procedure, given the CAPIF-8 reference point and relevant procedure is specified in the present document.

### 6.4.2	Solution details

The procedure for revoking API invoker authorization initiated by the resource owner through the resource owner function (ROF) is given below:

Editor’s Note: how authorization data is provided to CCF/AEF is ffs

1.  The resource owner may trigger token revocation through the ROF. The ROF sends an authorization revocation request message to the CCF. The message shall include the service API ID, the GPSI, and other information related to the revoked token (e.g., the scope info).

2.	With reference to step 2 in clause 8.23.4 of TS 23.222 [3], the request message includes in addition the GPSI of the UE, on which the ROF resides.

3.  With reference to step 3 in clause 8.23.4 of TS 23.222 [3], the AEF may additionally determine whether to update the resource due to revocation, e.g., for the QoS service API, the AEF may inform PCF/SMF to modify the QoS level of corresponding PDU sessions after revocation.

Editor’s Note: the need for resource update is ffs.

4.  The same as the step 4 in clause 8.23.4 of TS 23.222 [3].

5.  Similar to the step 5 in clause 8.23.4 of TS 23.222 [3], the difference is invalidated authorization here is for the resource owner/UE corresponding to the GPSI.

6.  Similar to the step 6 in clause 8.23.4 of TS 23.222 [3], the difference is that the message is sent to the ROF.

Figure 6.4.2-1: Revocation procedure for initiated through ROF

### 6.4.3	Evaluation

This solution addresses the requirements of Key Issue #1.2.

This solution reuses the authorization procedure specified in clause 6.5.3 in the TS 33.122 [4] and extends the revocation procedure in clause 6.5.3.4 in TS 33.122 [4] to include steps related to CAPIF-8 interface.

The revocation procedure is to prevent token being misused by an attacker. Although a short-lived token can potentially mitigate the issue, it has restrictions for its applicability or use cases. For example, setting token lifetime may be challenging in many scenarios. On the one hand, a large number of tokens or frequent token refreshing may be needed if the token lifetime is short, which constrains the system with complexity and overhead. On the other hand, if the token lifetime is short, the token may be misused or compromised.

***  END OF CHANGES  ***