3GPP TSG-SA3 Meeting #119	S3-244843

Orlando, US, 11 -15 November 2024

# Source:	Huawei, HiSilicon, China Telecom

# Title:	Additional conclusions for KI#3

# Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Approve the pCR proposal for TR 33.700-32

# 2	References

[1]	3GPP TR 33.700-32 Study on security aspects of User Identities and Authentication

# 3	Rationale

This contribution proposes additional conclusions for KI#3 of TR 33.700-32 [1], i.e., to address the editor notes in the conclusion clause 7.3.

The normative work in SA2 has started recently focusing on the case where non-3GPP Devices are connecting behind a UE or 5G-RG. The objective is to allow the 5GC to associate the traffic from each individual non-3GPP device with a Device Identifier and use the Device Identifier to obtain QoS Settings for the traffic. Specifically, for non-3GPP devices requiring QoS differentiation, the following is being specified in SA2:

- A UE/5G-RG binds the Device Identifier to a non-3GPP device

- QoS differentiation is performed within a PDU Session (using different QoS Flows) or between PDU Sessions.

- The UE/5G-RG and/or the AF/DN may decide to request for change of QoS Flows

It is expected that the authorization part is handled by SA3. Specifically, the UE/5G-RG request of change of QoS flows requires authorization by 5GC. Since the Non-3GPP Device belongs to the DN/AF or its subscription details are with DN/AF, 5GC authorization should be based on the interactions with the DN/AF and 5GC grants the corresponding QoS/Policies after receiving confirmation from the DN/AF. Authorization process based on the PDU session establishment or modification procedure is in line with the normative work in SA2. Therefore, it is proposed to specify the following during normative phase in SA3:

- 5GC authorization of traffic association/traffic change request from a non-3GPP device with a Device Identifier behind a UE/5G-RG.

- Authorization is based on the PDU session establishment or modification procedure. The details can be specified in the normative phase.

# 4	Detailed proposal

pCR

***  START OF CHANGES  ***

## 7.3	Key issue #3: Authentication and Authorization of one or more non-3GPP devices behind one gateway UE or 5G-RG

Based on the architecture assumption, authentication of non-3GPP device is not done by 5GC. Authentication for non-3GPP devices performed by UE or 5G-RG is not in the scope of 3GPP. Thus, no normative work is needed.

Normative work is recommended for 5GC authorization of traffic change request from a non-3GPP device with a Device Identifier behind a UE/5G-RG. Authorization is based on the PDU session establishment/modification procedure. The details will be specified in the normative phase.

***  END OF CHANGES  ***