3GPP TSG-SA3 Meeting #119	S3-244876

# Orlando, US  11 - 15 November 2024

# Source:	Huawei, HiSilicon

# Title:	Discussion Paper for using MPQUIC in ATSSS scenario

# Document for:	Approval

Agenda Item:	4.1.15

1	Decision/action requested

It is proposed to endorse the proposal raised by this discussion paper.

2	References

[1]					IETF RFC 8446: " The Transport Layer Security (TLS) Protocol Version 1.3".

[2]	IETF RFC 9001: "Using TLS to Secure QUIC".

3	Rationale

3.1	Background

During the SA3#117 meeting, it was decided to add an annex to describe certificated based authentication for MPQUIC in ATSSS. Thus, the current version of Annex AA of TS 33.501 only allows the UE to use X.509 certificate to authenticate the UPF. However, TLS 1.3 also supports other options (e.g. pre-shared keys). This contribution tries to analyse different options for server authentication used in R18 ATSSS scenario and proposes to add the pre-shared key option.

3.2	Analysis

Observation # 1: Existing hop-by-hop security mechanism for UE and UPF already provides protection for data transmission in ATSSS scenario.

As we know, hop-by-hop security mechanism has already been provided for UE and UPF data transmission. For ATSSS scenario, other high layer steering functionalities (e.g. MPTCP, ATSSS-LL) than MPQUIC do not have additional security features. From this point of view, this high-level intergrated TLS security is not essential, and the overhead of this function can be minimized.

Observation # 2: Pre-shared key (PSK) is the basis for Early Data (0-RTT) in QUIC protocol.

As we know, 0-RTT (i.e. early data) is an important feature provided by QUIC which enables the clients to send data immediately and save a round trip at connection setup for some application data. As described in IETF RFC 8446 [1] and RFC 9001 [2], the client and server should share a PSK in order to use 0-RTT feature:

“When clients and servers share a PSK (either obtained externally or via a previous handshake), TLS 1.3 allows clients to send data on the first flight ("early data"). The client uses the PSK to authenticate the server and to encrypt the early data.”

In cases UE and UPF adopt the 0-RTT feature to send data immediately, they already share a PSK, using the same PSK for authentication can avoid the trouble of issuing additional TLS certificate for UPF.

Observation # 3: Using the PSK mode can save computation and communication cost.

As a matter of fact, certicficate-based authentication for TLS needs the server to compute digital signatures and transport the signature and X.509 certificates to UE.  The UE needs to verify the digital signature using the certificate. While PSK mode only needs to transport a key identifier which is much shorter than certificate. Also  deriving a secret from the PSK does not need any asymmetric algorithm computations. Since the computation cost of asymmetric cryptographic algorithm is much larger than that of symmetric cryptographic algorithm, the PSK mode is more efficient.

Observation # 4: The PSK can be flexibly deployed and is aligned with the existing specification.

The PSK used for UPF identity authentication may be flexibly deployed based on the actual situation. For example, the PSK may be pre-configured by out of band mechanisms or obtained from the previous  security contexts.

It is worth mentioning that the exisitng specification exclude the hard-coded pre-shared key option. Hard-coded pre-shared key means that the PSK is directly written into the code and cannot be changed unless the software is upgraded. Pre-configured PSKs can be changed at any time through configuration means and are not conflicting with the existing specification.

The pre-configured PSK can be the same for different UPFs in the same PLMN. In that case, the UEs only needs to be pre-configured with one PSK to authenticate different UPFs which will further reduces deployment complexity.

To sum up, we should not limit the use of various approaches for building security, especially when other approaches have additional benefits, such as reducing the cost of communication and computation, and making deployment more flexible.

4	Detailed proposal

According to analysis above, we have the following observations:

Observation # 1: Existing hop-by-hop security mechanism for UE and UPF already provide protection for data transmission in ATSSS scenario.

Observation # 2: Pre-shared key (PSK) is the basis for Early Data (0-RTT) in QUIC protocol.

Observation # 3: Using the PSK mode can save computation and communication cost.

Observation # 4: The PSK can be flexibly deployed and is aligned with the existing specification.

Based on the observatios, we propose:

Proposal 1: Adding PSK based authentication option to perform the server authentication for MPQUIC in ATSSS scenario.