3GPP TSG-SA3 Meeting #119	S3-244918

Orlando, US 11 – 15 November 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2077 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authorization of a service request when the discovery is delegated to the target PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, TEI19_NFsel_by_tPLMN |  |  |  |  |  | Date: |  |  | 2024-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In scenarios where NFc and NFp pertain to different domains (e.g. different PLMNs, NPNs, or different regional organizations in a same PLMN) and using Indirect Communication with Delegated Discovery, the operator (or organization) of the target domain may prefer to perform the discovery and selection of the NFp in the target domain, e.g. for the following reasons: - to avoid disclosing information about candidate NFp that may be sensitive or that changes frequently (e.g. load and capacity info); - to enable the operator of the target domain to deploy its own discovery/selection policies, independently from NF implementations in other domains; - because SCPs in the target domain have the best knowledge about candidate NFp instances and sets, incl. load and capacity info, NF service status, etc. SA#103 approved a Rel-19 WID on NF discovery and selection by target PLMN (TEI19_NFsel_by_tPLMN), see SP-240490, with the objective to enable, for 5GC, the target PLMN/domain to perform the target NF producer selection based on target operator's policy, using indirect communication with or without delegated discovery (in the latter case, when the request indicates the NF set, the selection of the target NF instance in the set is delegated to the SCP of the target domain). Corresponding stage 2 requirements have been approved at SA#104 (S2-2407125, S2-2407356) and resulted in the following stage 3 CRs. C4-243186; Indirect communication with or without delegated discovery between different PLMNs with possible NF selection at target PLMN; Rel-19 29.500. C4-243617; NF discovery and selection by target PLMN; Rel-19 29.510. 29.573 holds a place holder section on authorization of NF service access when for a service request the discovery is delegated to the target PLMN. Details will be captured in 33.501, while 29.573 can then reference them. The CR aims to provide the missing parts in 33.501. |
| Summary of change: |  | Providing the details of access token request and service request for the different cases when the target PLMN has discovered selected the NF instance/service instance or NF Set. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security details are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.1.2.0 (new), 13.4.1.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** START OF CHANGES

#### 13.4.1.2	Service access authorization in roaming scenarios

##### 13.4.1.2.0	General

The clause describes service access authorization in roaming scenarios without SCP involvement. The service access authorization in roaming to support indirect communication between different PLMNs with or without delegated discovery with possible NF selection at the target PLMN is described in clause 13.4.1.X.

***** NEXT CHANGE

#### 13.4.1.5	Service access authorization in roaming scenarios in indirect communication scenarios with possible NF selection at target PLMN

##### 13.4.1.5.1		General

This clause specifies specific requirements to support indirect communication with or without delegated discovery between different PLMNs with possible NF selection at target PLMN, as specified in clause 6.3.1 of 3GPP TS 23.501 [X] and clause 4.17.10a of 3GPP TS 23.502 [4].

The terms "source PLMN" and "target PLMN" throughout this clause refer respectively to the PLMN originating the request (i.e. the PLMN of the NF service consumer) and the PLMN receiving/serving the request (i.e. the PLMN of the NF service producer). vNRF refers to the NRF in the source PLMN and hNRF refers to the NRF in the target PLMN.

The OAuth 2.0 roles for roaming are described in clause 13.4.1.2.1.

##### 13.4.1.5.2		Service access authorization for indirect communication with delegated discovery

With NF type token or NF Set/instance specific token

If the discovery is delegated to the target PLMN, the NF Service Consumer or SCP in the source PLMN (visited) sends a service request via vSEPP and hSEPP to the SCP in the target PLMN, optionally including CCA if cross-certification between PLMNs is enabled, and an access token if received in a previous service response. The NF Service Consumer and SCP in the source PLMN shall include the necessary information in the service request allowing to identify the target PLMN (or SNPN).

The SCP in the target PLMN (or SNPN) discovers the NF Service Producer and requests from hNRF the access token including the information received from NF Service Consumer and SCP of the source PLMN.

If the information received is successfully verified, hNRF issues an access token. If an access token was provided by the NF Service Consumer, the access token request may be skipped.

The SCP in the target PLMN (or SNPN) initiates the service request towards the NF Service Producer in the target PLMN with the received input parameters including the access token.

After successful access token validation, the NF Service Producer provides the service to SCP, which forwards the service response to the NF Service Consumer.

See also clause 13.4.1.3.2.

With NF type token

For NF type tokens, the NF Service Consumer or SCP in the source PLMN may also request the access token via vNRF from hNRF first and then insert the access token into the service request towards the target PLMN.

##### 13.4.1.5.3		Service access authorization for indirect communication without delegated discovery

###### 13.4.1.5.3.1	With mutual authentication between NF and NRF at the transport layer

In indirect communication without delegated discovery between different PLMNs, if NF Service Consumer and vNRF have been mutually authenticated, the NF Service Consumer selects an NF Set or a specific NF instance / NF Service instance after discovery and requests an access token (e.g. with NF type scope) from vNRF.

vNRF verifies the input parameters. If successful, vNRF identifies hNRF based on the target PLMN (or SNPN) information. After vNRF and hNF have been mutually authenticated via vSEPP and hSEPP, vNRF forwards the access token request to hNRF in the target PLMN (or SNPN). hNRF issues the access token and sends it back to the NF Service Consumer in the source PLMN.

After receiving the access token, the NF Service Consumer sends a service request to the SCP in the source PLMN including the access token and optional the CCA of the NF Service Consumer. If the CCA is included, the NF type of the expected audience in the CCA shall contain the NF type of the NF Service Producer.

The SCP in the source PLMN initiates the service request towards the target PLMN via vSEPP and hSEPP.

If the access token is successfully validated by the Service Producer in the target PLMN, the Service Producer provides back to SCP in the source PLMN the service response, which forwards it to the NF Service Consumer.

See also clause 13.4.1.3.1.1.

###### 13.4.1.5.3.2	Without mutual authentication between NF and NRF at the transport layer

In indirect communication without delegated discovery between different PLMN, if there is no mutual authentication between NF Service Consumer and NRF at the transport layer in the source PLMN, the NF Service Consumer or SCP selects an NF Set or a specific NF instance and requests the access token (e.g. with NF type scope) from SCP in the source PLMN. SCP initiates the access token request to vNRF.

vNRF verifies the input parameters. If successful, it identifies hNRF based on the target PLMN (or SNPN) information, mutually authenticates and forwards via vSEPP and hSEPP the access token request to hNRF in the target PLMN (or SNPN).

After receiving the access token from hNRF, the SCP in the source PLMN forwards the access token response including the access token to the  NF Service Consumer.

The NF Service Consumer sends a service request to the SCP in the source PLMN including the access token and optional the CCA of the NF Service Consumer.

The SCP in the source PLMN initiates the service request towards the NF Service Producer in the target PLMN, the service request including the access token and optional CCA.

If the access token is successfully validated by the Service Producer in the target PLMN, the Service Producer provides back to SCP in the source PLMN the service response, which forwards it to the NF Service Consumer.

See also clause 13.4.1.3.1.3.

***** END OF CHANGES