3GPP TSG-SA3 Meeting #119	S3-244942

# Orlando, USA  11 - 15 November 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Conclusion for KI#2.1

# Document for:	Approval, Information, Discussion

Agenda Item:	5.14

# 1	Decision/action requested

It is required to approve the conclusion proposed for KI#2.1 in this contribution.

# 2	References

[1]	3GPP TR 33.749 v0.3.0 “Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3”

# 3	Rationale

Nine solutions have been proprosed to addressed KI#2.1 in [1]. This contribution is intended to propose a way forward in the conclusion of that KI to proceed in normative phase.

Camp 1: Solutions #1, #2, #5, and #7 have been focused on the first requirement (clause 5.3.1.3.1 of [1]), by proposing different mechanisms to make the IP address of the UE (private and/or public depending on the scenario) a reliable identifier in API invocations intended to retrieve 5G system UE Ids and privacy related information.

Camp 2: Solutions #3, #4, #6, #8 and #9 have been focused on the first and second requirements (clauses 5.3.1.3.1 and 5.3.1.3.2 of [1]), by proposing the use of alternative short-term lived UE related information, i.e., random numbers assigned by the core (#3), A-KID derived from AKMA (#4), and UE ID tokens provided by application functions (#6, #8 and #9) with the purpose of avoiding potential privacy issues.

With the aim to provide a comprehensive approach that covers both security requirements, i.e, the verification of the user information provided by the EEC and the protection of the UE privacy, the recommendation is to proceed in normative work with the solution principles of the Camp 2, allowing a “secure” use of IP addresses (Camp 1) as implementation option.

The solution principles of Camp 2 are summarized as follows:

- Short-term lived information is used in NEF API call invocations, preventing the use of permanent or semi-permanent UE identifiers subject to be abused/compromised by malicious applications.

- The entity responsible for the generation of the short-term lived information is part of the 5G system, either an application function or an embedded functionality in an existing 5G core NF. Note that in case of using an AF, this entity requires to store locally information of the UE, that already resides in the 5G Core NF (e.g., UDM/UDR), to be mapped to that short-term lived information (e.g., token).

- The concrete format of the referred short-term lived information, i.e., temporary ID, token, A-KID, etc., is to be decided in normative phase, and to be agreed jointly with CT4.

- Short-term lived information may be used for the AF invoking the NEF APIs for UE information without retrieving 5G UE Id, hence the privacy of the UE is not compromised. For example, if AKMA feature is supported in the 5G system and it is used for the AF invoking the NEF APIs, there is no need to retrieve 5G UE Id, but instead A-KID can be used w/o compromising the privacy of the UE.

# 4	Detailed proposal

******************START of CHANGES****************************************

# 7	Conclusions

## 7.X	KI#2.1: Secure retrieval of 5G system UE Ids and privacy related information

The following solution design principles are to be followed in normative phase:

- Short-term lived information is used in NEF API call invocations, preventing the use of permanent or semi-permanent UE identifiers subject to be abused/compromised by malicious applications.

- The entity responsible for the generation of the short-term lived information is part of the 5G system, either an application function or an embedded functionality in an existing 5G core NF. Note that in case of using an AF, this entity requires to store locally information of the UE, that already resides in the 5G Core NF (e.g., UDM/UDR), to be mapped to that short-term lived information (e.g., token). Thus, a synchronization between these entities will be needed.

- The concrete format of the referred short-term lived information, i.e., temporary ID, token, etc., is to be decided in normative phase, and to be agreed jointly with CT4. If the referred short-term lived information is A-KID, TS 33.535 is referred to the format.

- Short-term lived information may be used for the AF invoking the NEF APIs for UE information without retrieving 5G UE Id, hence the privacy of the UE is not compromised. For example, if AKMA feature is supported in the 5G system and it is used for the AF invoking the NEF APIs, there is no need to retrieve 5G UE Id, but instead A-KID can be used w/o compromising the privacy of the UE.

Solutions using potentially long-term and/or easily inferred identifiers can be implementation options. The verification of those identifiers should be part of those implementations.

******************END of CHANGES****************************************