3GPP TSG-SA3 Meeting #119	S3-245006

Orlando, US, 11 -15 November 2024

# Source:	Ericsson

# Title:	Discussion on the NF consumer PLMN ID check

# Document for:	Endorsement

Agenda Item:	4.1.2

# 1	Decision/action requested

Endorse the proposal on NF consumer PLMN ID check

# 2	References

[1]	https://www.3gpp.org/ftp/TSG_SA/WG3_Security/TSGS3_93_Spokane/Docs/S3-183789.zip

[2]	https://www.3gpp.org/ftp/tsg_sa/WG3_Security/TSGS3_94_Kochi/docs/S3-190559.zip

[3]	3GPP TS 29.510 "5G System; Network function repository services; Stage 3"

[4]	3GPP TS 33.501 "Security architecture and procedures for 5G system"

[5]	SA3 CRs: CR S3-231606

[6]	CT4 CRs: C4-191528, C4-225627, C4-203256

# 3	Rationale

## 3.1	History

SA3 identified a security requirement, described in an LS sent to CT3 and CT4 [1], to prevent an attack by an NF of one network impersonating an NF of another network:

"It is necessary to enable the receiving SEPP to reliably verify the source of the request and response message, before forwarding the message including the PLMN information to the target NF for the subsequent authorization verification procedure." [1]

To fulfill the security requirement, SA3 proposed, in a subsequent LS to CT3 and CT4 [2], including PLMN ID of NF consumer in the access token in plaintext:

"It is proposed to enable verification of the PLMN-ID of the NF consumer by including the consumer's PLMN-ID unencrypted into the access token specified in TS 29.510, e.g. as part of existing claims." [2]

In Rel-15, as a result of the above LSes, CT4 added NF consumer's PLMN ID in the Access Token Claims (CR C4-191528).

In Rel-16, to consistently define authorization parameters for different services/procedures in the NRF, in the Access Token Request message, CT4 included a list of PLMN IDs instead of a single PLMN ID when NF supports multiple PLMN IDs (CR C4-203256).

In Rel-17, SA3 introduced the verification of NF consumer’s PLMN ID based on 3gpp-Sbi-Originating-Network-Id header, and clarified the requirement of SCP/SEPP to include the header (CR S3-231606). Accordingly, CT4 updated the 3gpp-Sbi-Originating-Network-Id header to identify the entity inserting the header (CR C4-225627).

## 3.2	Observations

Observation 1: Starting from Rel-16, NF consumer PLMN ID check based on access token claim may not work when NF consumer network support multiple PLMN IDs

In Rel-16, to consistently define authorization parameters for different services/procedures in the NRF, in the Access Token Request message, CT4 included a list of PLMN IDs instead of a single PLMN ID when NF supports multiple PLMN IDs (CR C4-203256).

However, access tokens can include only one PLMN ID of the NF consumer. In the current specification, it is unclear how the NRF chooses one PLMN ID of the NF consumer to provide in the access token claim when it receives an access token request with a list of PLMN IDs of the NF consumer.

Therefore, it may happen that the NF consumer obtains an access token from the NRF with a PLMN ID of the NF consumer that does not match with the PLMN ID that the NF consumer intends to use in the service request API message sent to the target NF producer.

According to TS 33.501, the NF producer shall check the PLMN ID of the NF consumer in the access token with the PLMN ID contained of the NF consumer in the API request. And when the access token verification fails, NF producer shall generate error response.

13.4.1.2.2           Service Request Process

In addition to the steps described in the non-roaming scenario in 13.4.1.1, the NF Service Producer shall verify that the PLMN-ID (or SNPN ID) contained in the API request is equal to the one inside the access token.

13.4.1.1.2           Service Request Process

3. If the verification is successful, the NF Service Producer shall execute the requested service and responds back to the NF Service Consumer. Otherwise, it shall reply based on Oauth 2.0 error response defined in RFC 6749 [43].

Observation 2: Starting from Rel-17, there are two solutions to fulfill the security requirement regarding NF consumer PLMN ID check.

In Rel-15, NF consumer PLMN ID check is based on the access token claim.

TS 33.501 clause 13.4.1.2.2	Service Request Process:

In addition to the steps described in the non-roaming scenario in 13.4.1.1, the NF Service Producer shall verify that the PLMN-ID (or SNPN ID) contained in the API request is equal to the one inside the access token.

The pSEPP shall check that the serving PLMN ID of subject claim in the access token matches the remote PLMN ID. If PRINS is used, this can be achieved by the pSEPP checking the PLMN ID of the serving network in the access token against the PLMN ID(s) in the N32-f context.

And starting from Rel-17, NF consumer PLMN ID check based on 3gpp-Sbi-Originating-Network-Id header is introduced.

TS 33.501 clause 5.9.3.2	 Requirements for Security Edge Protection Proxy (SEPP):

Receiving SEPP behavior for the 3gpp-Sbi-Originating-Network-Id header:

- 	The receiving SEPP shall check whether the 3gpp-Sbi-Originating-Network-Id header included in the signalling message belongs to the sending SEPP’s own PLMN or SNPN. It does this by verifying that the asserted PLMN ID in the 3gpp-Sbi-Originating-Network-Id header matches one of the sending SEPP's own PLMN ID(s) or SNPN ID(s) either in the N32-f context, the sending SEPP's certificate, or a locally configured list of PLMN IDs or SNPN-IDs that the sending SEPP represents.

TS 33.501 clause 5.9.3.5	 Requirements for Network Functions (NF):

The NF that sends a signalling message (service/subscription request or notification message) shall include its PLMN ID or SNPN ID in the 3gpp-Sbi-Originating-Network-Id header.

If an NF supports multiple PLMN IDs or SNPN IDs, the sending NF shall include the PLMN ID or SNPN ID in the 3gpp-Sbi-Originating-Network-Id header on behalf of which the message is sent.

The handling of the PLMN ID or SNPN ID in the 3gpp-Sbi-Originating-Network-Id header at the receiving NF is up to configuration and deployment.

NOTE:	A misconfigured PLMN ID or SNPN ID in the 3gpp-Sbi-Originating-Network-Id header can lead to service interruption.

# 4	Discussion

Based on current specification, the verification of PLMN ID based on access token at NF producer may fail when multiple PLMN IDs are deployed in the NF consumer's network.

It may happen that the NF consumer does not have an access token with a PLMN ID that matches with the PLMN ID the NF consumer intends to include in the service request API message. As a result, there could be three scenarios:

a) NF consumer does not send the service request,

b) NF consumer sends the service request without access token, and

c) NF consumer sends the service request with an access token which includes an NF consumer PLMN ID mismatched with the NF consumer PLMN ID contained in the service request API message.

When the NF producer receives a service request for scenario b) and c), NF producer will generate error response since access token verification fails.

# 5 	Detailed proposal

Based on the observations made in this paper, we propose to remove the access token-based check of NF consumer PLMN ID at NF producer and SEPP starting from Rel-17 (CR S3-24xxxx).

For Rel-16 deployment, we propose to send a LS to CT4 to take the observations and SA3 changes on Rel-17 into account.