3GPP TSG-SA3 Meeting #119	S3-245015

Orlando, US, 11 -15 November 2024

# Source:	Ericsson

# Title:	New key issue for Authenticated and authorized access to devices in Ambient IoT via 3GPP core

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This document proposes to add a new key issue for authenticated and authorized access to AIoT in TR 33.713.

# 2	References

[1]	3GPP TR 22.369

# 3	Rationale

The SA1 TR 22.369 [1] contain the following requirement:

The 5G system shall provide suitable mechanisms to support communication between a trusted and authorized 3rd party and an Ambient IoT device or group of Ambient devices.

Downlink. Authorization is insufficient by itself and needs to be coupled with mutual authentication to protect devices from unauthorized access by malicious parties, and to protect AFs from revealing sensitive information to someone impersonating an NEF towards the AF.

Uplink. Additionally, AIoT devices can regularly report data to an AF via the NEF without receiving a request initiated by the AF for each report. In this situation, the NEF needs to authenticate AF before delivering the report to ensure that it is delivered to the appropriate AF.

Potential interactions between over-the-top (OTT) security and access security needs to be be clarified.

# 4	Detailed proposal

**** START OF CHANGE ****

## 5.X	Key issue #X: Authenticated and authorized access of AF to devices in Ambient IoT via 5G System

### 5.X.1	Key issue details

The SA1 TR 22.369 [1] contain the following requirement:

The 5G system shall provide suitable mechanisms to support communication between a trusted and authorized 3rd party and an Ambient IoT device or group of Ambient devices.

Downlink. Authorization is insufficient by itself and needs to be coupled with mutual authentication to protect devices from unauthorized access by malicious parties, and to protect AFs from revealing sensitive information to someone impersonating an NEF towards the AF.

Uplink. Additionally, AIoT devices can regularly report data to an AF via the NEF without receiving a request initiated by the AF for each report. In this situation, the NEF needs to authenticate AF before delivering the report to ensure that it is delivered to the appropriate AF.

Potential interactions between over-the-top (OTT) security and access security needs to be clarified.

### 5.X.2	Security threats

The 5G system accepts unauthenticated traffic that appears to be coming from an authorized third party but is actually sent by an adversary.

A 3rd party AF transmits sensitive data to a malicious 5G system that is impersonating a legitimate 5G system.

An unauthorized 3rd party AF sends traffic towards a large set of AIoT devices and induces high traffic loads in downlink. If the device application responds to the downlink traffic, there is also a high load of uplink traffic.

### 5.X.3	Potential security requirements

The 5G system shall authenticate all requests from authorized 3rd party AFs before initiating interaction with AIoT devices.

The 5G system shall authenticate itself to the 3rd party AF before accepting input. This enables the 3rd party AF to verify that the 5G system is acceptable for receiving the data sent and expected responses from AIoT devices.

**** END OF CHANGE ****