3GPP TSG-SA3 Meeting #119	S3-245018

Orlando, US, 11 -15 November 2024

# Source:	Ericsson

# Title:	Evaluation of Solution#29 to KI#3

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This document provides evaluation of Solution #29 in TR 33.713

# 2	References

None

# 3	Rationale

Editor’s notes in the solution details are addressed in a separate pCR.

AICI is not stored in the network. Instead, the network decrypts AICI. A device accepts an AICI only if it is computed by the legitimate home network --- hence a device cannot have an AICI that the network won’t recognize. Therefore, the question about AICI synchronization in the network is not relevant.

The solution requires AIoT devices to have the capability to update and store AICI.

# 4	Detailed proposal

**** START OF CHANGE ****

## 6.29	Solution #29: Providing a network-computed AIoT concealed device identifier (AICI) to an AIoT device

### 6.29.1	Introduction

This solution addresses KI#3: Privacy by protecting AIoT device identifiers.

### 6.29.2	Solution details

The solution proposes a method for the 5G network to compute a AIoT Concealed Device Identifier (AICI) and provide the SUCI to the AIoT device in a command message. Once an AIoT device is identified, e.g., after a successful completion of inventory procedure, the network can send a command message on the downlink channel to the AIoT device. In the downlink command message, the network includes an AICI, which is computed based on the long-term identifier of the AIoT device using the public key of the network. The downlink command message is both confidentiality and integrity protected using keys derived from a shared key between the network and the AIoT device. Figure 6.29.2-1 presents a high-level message flow of the solution.

Figure 6.29.2-1: Procedure for delivering a AICI to an AIoT Device

The figure is self-explanatory, therefore, the steps are not explained step-by-step. Exact content of the messages exchanged, and details about authentication challenge, computing response to the challenge, and deriving the keys Kenc and Kint are not described because these details have to be adjusted with the authentication protocol that is finally agreed.

NOTE: If the AIoT device does not have a network-computed AICI, for example, in the very first time of the device’s life cycle, then the AIoT device computes AICI using null scheme. This happens only in the beginning. To avoid using null scheme in the first time, a network can choose to provision every AIoT device with a network-computed AICI before they are handed out to their users.

Editor’s Note: The content of the paging message and how the device decides to respond to the paging message based on the content of the paging message is FFS.

Editor’s Note: Synchronization of AICI is FFS

Editor’s Note: Whether AICI is to be computed and decrypted in SIDF/UDM is FFS

Editor’s Note: Whether AIoT device can update and store AICI is FFS

### 6.29.3	Evaluation

The solution fulfills the potential security requirement in KI#3 in the context of the uplink messages.

The solution requires an NF in the 5GC to compute AICI on behalf of the AIoT device.

The solution requires AIoT devices to have the capability to update and store AICI.

Editor’s Note: Further evaluation is FFS.

**** END OF CHANGE ****