3GPP TSG-SA3 Meeting #119	S3-245097

Orlando, USA  11 – 15 November 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2086 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MOBIKE for NTN 3GPP backhaul over feeder link |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | gNB onboard satellite may change its IP address as a consequence of it connecting to different NTN gateways on the ground along satellite’s orbital trajectory. IP addresses associated with tunnel mode IPsec needs to reflect the change in gNB IP address. gNB IKEv2 negotiation with SEG needs to include MOBIKE_SUPPORTED in notify payload. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce MOBIKE support in gNB for NTN Generic Regenerative architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unknown anomalous state of the IPsec tunnel between gNB and SEG. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start Change ***

Annex XX (normative): 
Security aspects of the 5G Satellite Access

# XX.1	General

This Annex specifies the Security aspects of the 5G Satellite Access.

# XX.2	Generic Regenerative Backhaul over Feeder Link

An NG-RAN consisting of a gNB embarked on an airborne or space-borne NTN vehicle communicates with the ground-based infrastructure via a feeder link (as defined in TS 38.300 [52]) of a ground-based NTN Gateway (as defined in TS 38.300 [52]).

The backhaul segment connecting the gNB on an NTN vehicle and the ground based SEG is carried over a feeder link. The orbiting satellite movement results into frequent switches of a feeder link from a source NTN Gateway to a target NTN Gateway.

It may result into a change in IP address of a gNB. In case IPsec tunnel mode is used for backhaul protection, the gNB onboard orbiting satellite and SEG shall support MOBIKE_SUPPORTED for IKE_AUTH (IETF RFC 4555 [111]) to migrate the IPsec tunnel to the new outer IP addresses. The security associations shall be realized according to TS 33.210 [3] and TS 33.310 [5].

According to clause 16.14.4.2 of TS 38.300 [52], in a soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, so a temporary overlap is observed during the transition between the feeder links where the gNB has active connectivity to both the source NTN Gateway and target NTN Gateway. The gNB can use MOBIKE to notify SEG the alternate IP address for IPsec in the event if gNB is not reachable on its previous IP address.

*** End of Change ***