3GPP TSG-SA3 Meeting #119	S3-245106

Orlando, US, 11 – 15 November 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Discussion on Impact Analysis for Inter-CU LTM

# Document for:	Discussion

Agenda Item:	4.12

# 1	Decision/action requested

This document is for the presentation and discussion of the security impacts of the Inter-CU LTM options.

# 2	References

(Reference - in list form - should be made to previous related SA5/3GPP/etc. documents.)

(For changes against a draft TS/TR, a pseudo CR - a.k.a. pCR - will be provided using this Tdoc template. In this case, the number, name and version of the draft TS/TR used as base must be provided and the version must be the latest available version of the draft TS/TR.)

[1]	S3-241773 (R2-2404037) LS on security handling for inter-CU LTM in non-DC cases

[2]	TS 38.331

[3]	TS 37.340

# 3	Rationale

## 3.1 Background and Motivation

For the WI on the inter-CU LTM with security key change, the RAN2 has discussed and collected options as possible directions for handling the key change as part of inter-CU LTM cell switch. The options have been documented and shared with SA3 by means of a LS [1]. Now, as part of the LS, the SA3 is requested to study the different options from security perspective and to give feedback if any of the listed options is not feasible or not acceptable from security perspective.

## 3.2 Methodology and Working Assumptions

This discussion document is taking the current RRC handover procedures into consideration and is analysing from LTM perspective.

For this LTM it is assumed that the MAC CE must carry in addition LTM related information elements, e.g., the NCC value.

The MAC CE frame could be extended by the NCC value, as shown by the following Figure 3.3-1.

Figure 3.2-1: MAC CE Frame includes NCC value placement

No new security mechanisms will be taken into consideration, i.e., the MAC CE frame is being transferred in the clear, i.e., is not protected.

## 3.3 Cell Switch Command Procedure - overview

Below Figure 3.3-1 depicts the general principle of LTM execution.

Figure 3.3-1: Cell Switch Command Procedure Flow (overview)

Altering of MAC CE Cell Switch Command Message

In Step 6 the source gNB decides for a LTM and is sending an MAC CE Cell Switch Command, that includes the NCC value (see working assumption). The UE is receiving this message with potential altered NCC value, then there exist two alternatives, first (Alt#1) the NCC value is assumed by the UE to be valid, and the UE is responding with RRC reconfiguration complete message, or second (Alt#2) the NCC value is not as part of that list, and the UE is sending RRC connection reestablishment request.

Observation#1: The UE does not have any means to identify whether the MAC CE Cell Switch Command has been altered.

Observation#2: The RRC error recovery procedure is always triggered on a timer basis (see [2] and [3]), i.e. there is no RRC reconfiguration failure/error procedure, and thus the RRC recovery procedure is triggered only after the expiry of the corresponding timer (i.e., T304). Now if we assume that the UE has accepted a received NCC value, this may result in out-of-sync as long the timer has not expired.

Observation#3: The Time-based recovery in a non-synchronised method relies on the expiry of the corresponding timer. While the timer value could be optimized. This value (T304) is used to also detect the RACH failure for the UE, so it cannot be shortened too much.

## 3.4 Abbreviations

LTM			L1/L2 Triggered Mobility

MAC			Medium Access Control

MAC CE		MAC Control Element

NCC			Next Counter Check

NH			Next Hop

RRC			Radio Resource Control

# 4	Detailed proposal

Proposal#1: It is proposed to analyse how the UE can detect any unauthenticated altering/modification of the MAC CE Cell Switch Command.

Proposal#2: It is proposed to analyse potential fast UE recovery procedures.