3GPP TSG-SA3 Meeting #119	S3-245291

# Orlando, USA  11 - 15 November 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Solution for KI#3 to authorize VAL-S or VAL_C to access digital asset

Document for:	Approval

Agenda Item:	5.18

# 1	Decision/action requested

Approve the solution added to TR 33.721

# 2	References

[1]	3GPP TR 23.700-21 Study on Application enablement architecture for mobile metaverse services

[2]	3GPP TR 33.721 Study on security aspects of 5G Mobile Metaverse services

[3]	3GPP TS 33.501 Security architecture and procedures for 5G system

[4]	3GPP TS 23.438 Service Enabler Architecture Layer for Verticals (SEAL); Digital Assets

[5]	3GPP TS 33.122 Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs

# 3	Rationale

Avatar and digital asset support, including digital asset avatar management and discovery, is discussed in Key issue #3 in TR 23.700-21, which requires to create, update, retrieve, delete and discover digital assets securely.

Correspondingly, KI#3 on Security aspects of digital asset container in 5G was approved and added in TR 33.721.

Architecture, procedures and information flows for digital asset service to support mobile metaverse services are specified in TS 23.438. Users can be associated with one or more digital assets like Avatars, software licenses, files, etc. Applications like mobile metaverse services can utilize the digital assets related to users, and the users can benefit from having the use of their digital assets between the various metaverse applications/platforms in an interoperable way.

This solution aims to authenticate and authorize a VAL client (a.k.a DA client) or VAL server to access digital asset(s) based on CAPIF.

# 4	Detailed proposals

# * * * * First change* * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[2]	3GPP TR 23.700-21: "Study on Application enablement architecture for mobile metaverse services".

[3]	3GPP TS 22.156: "Mobile Metaverse Services; Stage 1".

[4]	3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

[5]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs"

[6]	3GPP TS 33.501: "Security architecture and procedures for 5G System"

[7]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows"

[x]	3GPP TS 23.438: “Service Enabler Architecture Layer for Verticals (SEAL); Digital Assets”

# * * * * second change* * * *

## 6.x	Solution #x: authenticate and authorize VAL server/client to access digital asset

### 6.x.1	Introduction

This solution addresses Key Issue #3 on Security aspects of digital asset container in 5G.

Avatar and digital asset support, including digital asset avatar management and discovery, is discussed in Key issue #3 in TR 23.700-21[2]. The corresponding requirements were defined in clause 5.4 of the TR 23.700-21[2] which include, for example, subject to operator policy, regulatory requirements and user consent, the metaverse enablement service shall provide digital asset management mechanisms as follows:

-	to create, update, retrieve, delete and discover digital assets securely.

-	to manage associations between digital assets and user identifiers.

-	to allow an authorized third party to manage digital asset(s) associated with a user.

Correspondingly, KI#3 on Security aspects of digital asset container in 5G was described in TR 33.721 which requires 5G system to authenticate and authorize a digital asset service consumer to access the digital asset(s) in a digital asset container.

Architecture, procedures and information flows for digital asset service to support mobile metaverse services are specified in TS 23.438 [x]. Especially, figure 6.2-2 of TS 23.438 [x] illustrates the DA architecture to support Metaverse services using CAPIF architecture.

Users can be associated with one or more digital assets like Avatars, software licenses, files, etc. Applications like mobile metaverse services can utilize the digital assets related to users, and the users can benefit from having the use of their digital assets between the various metaverse applications/platforms in an interoperable way.

This solution aims to authenticate and authorize a VAL client (a.k.a DA client) or VAL server to access digital asset(s) based on CAPIF.

### 6.x.2	Solution details

Figure 6.x.2-1 Procedure to authorize VAL_C/VAL_S to access digital asset

Precondition:

- Digital asset client (DA client/APIInvoker in UE), CAPIF core function (CCF), digital asset DA server (DA server/AEF) are preconfigured with certificates and trust anchor for TLS based mutual authentication.

- CAPIF core function (CCF) is preconfigured with authorization policies for API Invokers, which may define authorization policies that what AEF services can be accessed by the API Invokers.

- Digital asset is created and stored in DA server which may include authorization information provisioned by the owner of the digital asset, refer to clause 7.2 of TS 23.438 [x] for the detail information in DA profile.

Procedure:

1. VAL server/client (or API invoker) is onboarded successfully and CAPIF-1E authentication is performed with CCF. The authorization polices for the API invoker may be updated based on the API invoker profile.

2. VAL server/client sends Access Token Request to CCF to update/retrieve/delete digital asset, the request includes client id, UE id such as GPSI, digital asset identifier and optionally digital asset profile for update operation.

3. CCF checks if the VAL server/client is allowed to access digital asset services based on preconfigured authorization policies for the API invoker or updated policies in step 1.

4. If so, based on the deployment option, CCF may check if authorization information associated to the digital asset identifier existed locally. If not, CCF locates corresponding DA server based on API type, location, UE information, digital asset identifier, etc., and performs CAPIF-3 authentication with the DA server.

5. Optionally, CCF sends request to the DA server to get authorization information of the digital asset.

6. DA server returned authorization information of the digital asset based on the DA profile created in precondition step.

7. CCF stores the authorization information of the digital asset, and checks if the VAL server/client, on behalf of the subscriber identified by GPSI, is allowed to update/retrieve/delete the digital asset based on allowed user/application list or owner list .

Editor’s Note: The mapping between subscriber and allowed user is not defined in SA6 or SA2. Therefore, how the CCF uses GPSI to check against the allowed user list is FFS.

8. If the subscriber in VAL server/client is not the owner of the digital asset, based on the policy, CCF may ask authorization from the resource owner based on RNAA framework.

Editorial Note: Whether RNAA framework is used for authorization from the resource owner is FFS.

Editor’s Note: The mapping between subscriber and owner is not defined in SA6 or SA2. Therefore, how the CCF uses GPSI to check against the owner of digital asset is FFS.

Editor’s Note: As the user and owner of a digital asset may not be the same person, how the CCF uses one GPSI to check against both the user and owner is FFS.

9. If the VAL server/client is allowed to update/retrieve/delete digital asset, CCF sends access token to the VAL server/client which includes authorized operations and parameters.

10. After received access token for accessing of digital asset, the VAL server/client performs CAPIF-2E authentication with DA server.

11. VAL server/client sends DA update/retrieve/delete request to DA server which includes access token besides other parameters.

12-13. DA server verifies the access token and proceeds the request accordingly and returns the result to the VAL server/client.

Note: Instead of retrieving authorization information of the DA from DA server in step 5 and 6, the CCF may only authorize the VAL server/client based on preconfigured/updated local authorization policies and include related permissions in the scope of the access token, and DA server may further authorized the access based on authorization information of the required DA.

### 6.x.3	Evaluation

TBA.

# * * * * End of changes * * * *