3GPP TSG-SA WG4 Meeting #130	S4-241857

# USA, Orlando, 18 – 22 November 2024

Source:	InterDigital Europe, Tencent

Title:	Pseudo-CR on Haptics integration in 5GMS services and architecture

Spec:	3GPP TR 26.854

Agenda item:	10.8

Document for:	Agreement

1. Introduction

This contribution describes the integration of haptic media in both the generalized media delivery architecture and the IMS architecture to support the agreed use-cases and address part of item (5) of the SID:

“Identify 3GPP streaming and communication services that may benefit from the haptics media type and identify the gaps and requirements need to integrate haptics experiences.”

2. Reason for Change

Introduction of haptic media types in 3GPP 5GMS and IMS services and architectures.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854

* * * First Change * * * *

# 8	3GPP services

## 8.1 	Haptics media integration in the Generalized Media Delivery architecture

### 8.1.1	overview

The Haptics media functionalities in the generalized 5G Media Delivery architecture defined in 3GPP TS [26.501], and [26.506] are shown in figure 8.1.1-1. Existing interfaces and functions are reused to support Haptic media type and Haptic media transport in 5G services (e.g. 5GMS, RTC) to address the use-cases of section 5.

Figure 8.1.1.-1: Haptics media in the Generalized Media Delivery architecture

### 8.1.2	Network functions and UE entities

As Haptics media can be integrated in 3GPP services as a companion media type to Video and Audio, the media related definitions described in [TS 26.506] and [TS 26.501] are extended to describe the haptics media related functions highlighted in the figure 8.1.1-1 as follow:

-	Media AF: An Application Function as defined in clause 6.2.10 of [TS 23.501] dedicated to Media Delivery, including haptics media delivery.

-	Media AS: An Application Server dedicated to Media Delivery including haptics media delivery. The depicted Haptics media engine support access, coding and delivery function, of user plane haptic media data

-	Media Client: A UE internal function dedicated to Media Delivery comprising:

-	Media Session Handler: An entity on the UE that communicates with the Media AF in order to establish, control and support the delivery of a media session. The media session includes delivery of Haptics media.

-	Media Access Function: An entity on the UE that communicates with the Media AS in order to access and deliver media content, including haptics media. The media access function for example may be further sub-divided into content delivery protocols, codecs, media types and metadata representation, including those related to Haptic media data.

-	Media-aware Application: An application entity on the UE that makes use of 3GPP-defined APIs to invoke the Media Session Handler and/or the Media Access Function in order to support Media Delivery, including haptics media delivery.

### 8.1.3. 	Haptics media integration in XR Split rendering architecture

#### 8.1.3.1 Overview

As expressed in [TS.26.565], typical use cases for split rendering includes immersive gaming and immersive communication. These use cases, haptic enabled, are also described in section 5 of this document.

Figure 8.1.3.1-1 is based on figure 6.1.4-1 of [26.565] and illustrates the haptics media functions as part of the SRS, providing support for haptic media coding and delivery, similarly to audio and video media.

Media function: Haptics media engine

Figure 8.1.3.1-1: Haptic media functions in the User Plane Architecture for Split management architecture

On the UE, Figure 8.1.3.1-2, based on Figure 5.1.2-1[26.565] illustrates the haptics media entities in the XR baseline client.

Haptics codec

Haptics renderer

* * * End of Changes * * * *

Figure 8.2.1-2 Haptic media entities in the XR Baseline Client architecture

### 8.2.2	 Network functions and UE entities

The haptics media related network functions in the XR end to end split rendering architecture are similar to the one described in section 8.1.2

The haptics media entities on the XR baseline Client consist of:

-   the haptic media codec, handling and decompressing a compressed haptics media bitstream is illustrated in the MAF function of the SRC along with AV codecs.

-   the haptic renderer, handling the rendering of haptics effects using the targeted actuators is illustrated in the light presentation engine function of the SRC.

* * * Next Change all new* * * *

## 8.2 Haptics media integration in IMS architecture.

### 8.2.1 Overview.

To support the use-cases in section 5 for IMS based services (MTSI, and IBACS), the existing IMS architecture is not modified. Existing functions and API are reused to enable haptic enhanced IMS based real time communication.

Figure 8.2.1-1, illustrates the haptic media functionalities in the MTSI client defined in figure 4.2 of [26.114] which is similar to figure 4.2.1 of the AR-MTSI client of [26.264]

Sensors

Haptic media encoder

Haptics media decoder

Actuators

Figure 8.2.1-1: Functional components of an MTSI client with haptic media support.

Editor’s note: redo figures with editable source

### 8.2.2 Network functions and UE entities

For MTSI, [26.114] the network functions such as the MRFP and MGW may handle haptics media similarly to other media for supplemental services.

Referring to Figure 4.3.1 of [26. 264], Generalized IMS DC Architecture to support AR communication, the AR AS, the IMS AD and the MF/MRD handles haptics media similarly to video or speech media.

* * * Next Change  Add to reference section* * * *

[26.114]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[26.264]	3GPP TS 26.264: “IMS-based AR Real-Time Communication”

[26.565]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[23.501]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”