3GPP SA4 RTC #130	S4-241903

Orlando, FL, US, November 18-22, 2024	Revision of S4aR240086

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | pseudo | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_RTP_Ph2] AL-FEC awareness at RAN for the Downlink and Uplink Data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 11/18/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This paper addresses Note 3 for Solution #10 to Key Issue #4: AL-FEC awareness for PDU Set handling A difficulty in resolving Note 3 is packet loss estiation for the gNB to AS segment (5G core network and N6 segment). To address comments received at the SA4 RTC Telco on Oct 23, 2024. Additional comments from Huawei: [Qi] For 0086, I copied our inline discussion as following for convenience. - For RLC UM, the NDI design is not that accurate since the gNB may decide to give up the TB transmission due to retransmission times limits, which means the calculation is still not accurate. And this also needs cross-layer implementations, e.g. mapping between TB and the PDCP PDU in order to derive the packet loss rate. Besides, is there any relationship between the packet loss rate in Uu interface between UE and RAN (i.e. puu) and the packet loss rate for E2E (i.e. pe2e)? The only thing we can say is pe2e≥pu2u. How to determine the packet loss rate between RAN and AS? [Liangping] There is no simple relationship. This is what the proposal tries to solve. [Qi] In addition, the exchanges between UE and RAN is in the Modem under the Operating System (OS) while the E2E packet loss rate is obtained above the OS layer, e.g. RTP/RTCP layer. In practice, it is quite difficult to do this. [Liangping] This belongs to cross-layer signaling, which is already supported in certain handsets. To support the proposal, we just expand the signaling to include the packet loss events. [Liangping]: Regarding NDI, yes, but I think the probability the gNB gives up due to reaching the max RTX limit should be very small. The proposal is not to look at the loss rate, rather to look at loss events, from which the UE can estimate the packet loss rate between RAN and AS. Not sure if this address your concerns. [Qi]: Is there any reference to such statement “the probability the gNB gives up due to reaching the max RTX limit should be very small”? Seems I got different views from my RAN colleagues😊. Please also consider my other comments. [Liangping] If the probability is large, then the system is ill configured. For a proper configuration, the probability should be small, because gNB knows the status of decoding and can configure the PHY parameters of the UE to adapt to UL channel conditions, i.e., through adaptive modulation and coding (AMC) for each retransmission (redundancy version). - This is my main concern actually. The packet loss rate (let’s assume you can derive this) between the RAN and the AS is not a static value. AFAIK, there could still be a "micro-burst" in the transport network. Even we can get the statistical packet loss rate, personally I am still not sure whether that is beneficial to enable the packet dropping in the UE side at the first hop. [Liangping]: I think RTP sender faces the same problem, i.e., it sets the redundancy based on e2e packet loss rate estimation. This step in here is not new. [Qi]: The difference is who should be responsible for such problem. If the RTP sender estimates the E2E loss rate wrongly, then that’s the fault of the application layer itself. Otherwise, the 5G system should be responsible for this. From application perspective, if the optimization solution may bring harm to the user experience since we know the measurement cannot be that accurate, why does it need to pay to enable such a feature in 5GS? [Liangping] I think it will be OK because the UE derives the loss rate estimate, and it is up to the UE to decide whether to use the estimate to set a lower redundancy ratio. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a practical method on packet loss estimation for the gNB to AS segment (5G core network and N6 segment). Addressed the gNB feedback on uplink reception, how to figure out the RTP packet losses from the gNB feedback, and how to set the redundancy for the future transmissions based on past observed packet loss rate. Addressed the additional comments from Hauwei. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Note 3 is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 6.10	Solution #10: AL-FEC awareness at RAN while considering upstream and downstream packet losses

### 6.10.1	Key Issue mapping

This maps to Key Issue #4.

### 6.10.2	Description

### 6.10.2.1	Motivation

As explained in clause 5.4.2.2, there is an upstream network segment before the RAN and there may be a downstream network segment after the RAN (e.g., a tethered link). For DL data originated from the AS, when the RAN drops extra PDUs of a PDU Set encoded with AL-FEC, it needs to consider how many of the PDUs have already been lost in the upstream network segment. The RAN also needs to consider how many packets may get lost in the downstream network segment. Otherwise, the application receiver may not receive enough PDUs for reconstructing all the source PDUs.

NOTE 1:	Although the solution below is on how the RAN drops extra PDUs, it applies to a generic network device if the network device receives the downstream packet loss probability.

### 6.10.2.2	Solution

Figure 6.10.2.2-1: The RAN drops extra PDUs by considering packet losses in the upstream network segment and downstream network segment.

For DL data, the upstream is from the AS to the RAN, and the downsteam is from the UE to the XR tethered endopoint.

The RAN derives the PDUs that have been lost in the upstream network segment, e.g., by taking the difference between the number of PDUs in the PDU set (NPDS) in the RTP header extension for PDU Set marking for a PDU Set and the number of PDUs of the PDU Set that it has received.

The downlink network segment informs the RAN of the packet loss rate over the tethered link. This is shown in Figure 6.10.2.2-1.

NOTE 2:	Although in Figure 6.10.2.2-1 it is shown that the UE provides the packet loss feedback, the XR Tethered Endpoint can also provide the packet loss feedback.

The RAN then decides on how many PDUs are extra PDUs given (1) the upstream packet losses, (2) the packet losses at the RAN (not dropped as extra PDUs), and (3) the downstream packet loss rate.

The following example shows how the RAN decides on the number of extra PDUs that it can drop while still ensuring that the receiver can reconstruct all the source packets with a high probability. Denote the following:

(1)	the upstream packet losses: N1

(2)	the packet losses for the network segement from the ingress of RAN to the ingress of the UE (not dropped as extra PDUs): N2

(3)	the downstream packet loss rate: p

It is assumed that the packet losses in the downsteam are independent. Consider a PDU Set with be K=30 source PDUs, and 15 repair PDUs, hence a total of N=45 PDUs. Let the packet losses at for the network segment from the ingress of RAN to the ingress of the UE (not dropped as extra PDUs) be N2. Let the AL-FEC code be a MDS code. For simplicity each PDU is assumed to be sent in a separate transport block over the RAN air interface. Let N1=2, p=2%. Let the minimum reconstruction success probability be 0.995. Then the allowed values for the number of extra PDUs N3 that the RAN can drop must satisfy the following inequality:

n=Km-N3m-N3n(1-p)n pm-N3-n> 0.995, where m=N-N1-N2.

In the above, the summand gives the probability that n packets out of m-N3 packets are a success and the remaining m-N3-n packets are a failure. The probabilities are summed across all values of n which allow the application receiver to reconstruct all the source packets.

Figure 6.10.2.2-2: The RAN can drop up to N3=6 extra PDUs while still meeting the reconstruction probability threshold 0.995 with the packet losses in the upsteam, at the RAN (due to wireless link error, not dropped as extra PDUs), and in the downstream.

NOTE 3:	Although the solution presented above is for DL traffic, a similar solution can be devised for UL traffic, in which case the tethered link becomes upstream and the gNB to the AS segment (5G core network and N6 segment) becomes downstream.

Note: For UL data, the downstream is from the AS to the RAN, and the upstream is from the XR tethered endopoint to the UE.

For the uplink data traffic, the UE needs to determine the number of PDUs that it needs to successfully deliver over the air between the UE and the gNB. The number depends on the packet loss rate for the gNB to the AS segment. It is difficult to directly measure the packet loss rate. One possible method is to send probing packets (e.g., ICMP ping messages), but the AS may not recognize the IP address of the gNB and may reject the probing packet for security reasons. Even if transmitting probing packets is enabled, the probing packets may be treated differently in the network from the media packets, resulting inaccurate estimation of the packet loss rate. As an alternative, the packet loss rate may be estimated indirectly: the gNB provides feedback on the uplink packet reception, the AS provides feedback on the end-to-end packet reception, and the UE estimates the packet loss rate for the gNB to the AS segment, denoted as pr2a, which is the ratio of the number of packets lost as observed by the AS but not observed as lost by the gNB to the number of packets successfully received by the gNB. This is illustraged with an example in Figure 6.10.2.2-3.

Figure 6.10.2.2-3:  Indirect method for calculating the packet loss rate for the gNB to AS segment, where an X sign means the corresponding packet is lost.

NOTE 4: The feedback that the gNB provides on the uplink packet reception may be in the form of HARQ feedback (using New Data Indicator (NDI)). RLC Status PDU (in the case of RLC AM), or PDCP Control PDU.

Once the packet loss rate for the gNB to the AS segment is estimated, a similar formulation can be used to derive the number of PDUs that the UE can drop to meet a minimum reconstruction success probability at the AS.

To count the RTP packet losses from the gNB feedback, the UE needs to keep a mapping between the RTP packets and the lower-layer packets whose status is conveyed in the gNB feedback.

The future packet loss rate for the network segment from the gNB to the AS may be greater than the estimate. To minimizes the probability of failing to reconstruct the source PDUs, the UE may add a margin in the estimate, i.e., using a larger packet loss rate, in the same approach taken by a traffic source that sets the redundancy for AL-FEC based on end-to-end packet loss rate estimation.

The above assumes that the extra PDUs dropped at the RAN are not taken as signals of network congestion. Network congestion can occur in the upstream network segment, at the RAN or in the downstream network segment.

NOTE 5:	It is FFS under what conditions the RAN can drop extra PDUs considering network congestion in the upstream network segment, at the RAN or in the downstream network segment.

NOTE 6: For RLC UM, the NDI design may not that accurate since the gNB may decide to give up the TB transmission due to retransmission time limits, which means the calculation is still not accurate. This may need cross-layer support, e.g. mapping between TB and the PDCP PDU may be needed in order to derive the lost application layer packets. The probabilty of such events is FFS

NOTE 7: There is no clear relationship between the packet loss rate in Uu interface between UE and RAN (i.e. puu) and the E2E packet loss rate, but, pe2e should be larger than puu. Additional details of this relationship are FFS.

Cons: One drawback of the solution is that the underlying system may take some decisions that are detrimental to the application. If the RTP sender estimates the E2E loss rate wrongly, then that’s the fault of the application layer itself. Otherwise, if the 5G system will be responsible. Therefore, when implemented in 5GS the estimates need to be accurate.

# * * * * end of 1st change * * * *