3GPP SA4 #130	S4-241904

Orlando, FL, US, November 18 - 22, 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | pseudo | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_RTP_Ph2] RTP header extension for conveying AL-FEC information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 11/18/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | One aspect of Key issue #4: AL-FEC awareness for PDU Set handling This paper identifies normative work needed to expose AL-FEC information to enable AL-FEC aware PDU Set handling |  |  |  |  |  |  |  |  |
| Summary of change: |  | The design options are outlined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of not being able to support AL-FEC awareness for PDU Set handling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 6.x	Solution #x: Conveying AL-FEC information to the network

### 6.x.1	Key Issue mapping

This maps to Key Issue #4.

### 6.x.2	Description

As observed in Solution #8 (clause 6.8), the network complexity is reduced if the source packets and the repair packets of the same ADU are grouped into the same PDU Set as opposed to different PDU Sets.

As explained in clause 5.4, exposing AL-FEC information is needed for AL-FEC aware network resource allocation.

There are two approaches.

User-plane approach

It is noted in Solution #5 (clause 6.5) that for the packet formats considered there, the AL-FEC information for FlexFEC codes and ULPFEC codes is invisible to an intermediate network entity such as the UPF when the RTP payload is encrypted. Similarly, if the RTP/UDP is the transport protocol for Reed-Solomon FEC codes, the same invisibility issue arises.

Therefore, it is important to define new RTP packet formats to make the AL-FEC information visible to the network.

For MDS codes (including approximate MDS codes), although successful decoding only depends on how many PDUs are received, there is still a benefit to know whether a PDU is a source packet or a repair packet because in the case where all source PDUs are received, the decoding is trivial.

For the FlexFEC and ULPFEC codes, the dependence information between the source packets and the repair packets is also important for decoding.

Given that the PDU Set information is currently conveyed by the RTP header extension for PDU Set Marking, the same RTP header extension can be enhanced to expose AL-FEC information.

Control-plane approach

The control-plane approach may be an efficient approach to conveying relatively static information and can be considered for conveying the AL-FEC information for MDS (including approximate MDS) codes. However, it is not suitable for carrying highly dynamic information such as the dependence between source packets and repair packets in the FlexFEC codes.

Therefore, the following conclusion is made.

Conclusion:

Consider normative work for conveying AL-FEC information to the network to both the user-plane approach and the control-plane approach.

For the user-plane approach, consider enhancing the RTP header extension for PDU Set marking to convey the following information:

- The type of AL-FEC, whether it belongs to MDS (including approximate MDS) codes, or the other type of codes (including FlexFEC and ULPFEC) where the dependence between the source packets and the repair packets is important for decoding

- For MDS codes

  - redundancy ratio

- For FlexFEC type of codes:

  - An indication whether a PDU is a source PDU or a repair PDU

  - The dependence information between source PDUs and repair PDUs

For the control-plane approach, consider what information needs (e.g., redundancy ratio) to be conveyed for MDS (including approximate MDS) codes.

NOTE: Coordination with SA2/RAN2/RAN3 is needed.

# * * * * End of 1st change * * * *