3GPP TSG-SA4 Meeting #130	S4-241926

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.510 | CR | 0011 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [iRTCW] Clarifications on consumption reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications, BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | iRTCW |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 26.510 describes the consumption reporting procedure by a Media Session Handler for a downlink media delivery session but does not yet specify the consumption reporting procedure by a Media Session Handler and a Media AS for an RTC session, which is in scope for the indicated Work Item in this release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describes the details of metrics reporting and consumption reporting procedure by a Media Session Handler and a Media AS (both acting as consumption reporting entity) for an RTC session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not fully address the consumption reporting feature for an RTC session in this release. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.12.1, 5.3.5.1, 5.3.5.2, 5.3.6.1, 5.3.6.2 and 9.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.2.12	Consumption Reporting provisioning

#### 5.2.12.1	General

These operations are used by the Media Application Provider at reference point M1 to activate and to configure consumption reporting functionality associated with media delivery for downlink media streaming sessions or RTC sessions. The Media Application Provider may provision a single Consumption Reporting Configuration within the scope of a Provisioning Session which determines whether and how often the Media Session Handler and/or the Media AS submits consumption reports to the Media AF. This clause defines the basic operations. More details are provided in clause 8.12.

NOTE:	The Consumption Reporting feature is not applicable to uplink media streaming.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.1.6 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.1.7 shall be included in the response message.

Next change

### 5.3.5	Metrics reporting

#### 5.3.5.1	Procedures

These procedures are used by the Media AS at reference point M3 or else by the Metrics Reporting functions of the Media Client and subsequently by the Media Session Handler at reference point M5 to submit a metrics report to one of the Media AF instances listed in the client metrics reporting configuration of the Service Access Information resource previously retrieved using the procedure in clause 5.3.2.3.

-	When the metrics collection and reporting feature is provisioned for media delivery sessions using the operations specified in clause 5.2.11, one or more client metrics reporting configurations, each associated with a provisioned Metrics Reporting Configuration, shall be provided to the Media Session Handler (acting in this case as metrics reporting entity) in the Service Access Information.

-	When the metrics collection and reporting feature is provisioned for RTC sessions using the operations specified in clause 5.2.11, one or more client metrics reporting configurations, each associated with a provisioned Metrics Reporting Configuration, shall be provided to the Media Session Handler and the Media AS (both acting in this case as metrics reporting entity) in the Service Access Information.

Furthermore:

-	Before a media delivery session is started, the metrics reporting entity shall check if the Service Access Information includes any Client Metrics Reporting Configurations. If any such configurations are present, the metrics reporting entity shall initiate the metrics reporting procedure for the media delivery session based on these configurations.

-	During the course of the media delivery session, the metrics reporting entity shall periodically check if any Metrics Reporting Configurations have been added to or removed from the Service Access Information and shall activate or deactivate the metrics reporting procedure as appropriate for the media delivery session in question.

A given client metrics reporting configuration contains information including:

1.	The subset of metrics from the provisioned metrics scheme to be collected and reported by the metrics reporting entity;

2.	The frequency at which these metrics are to be sampled by the metrics reporting entity;

2a.	Thresholds for certain metrics, the crossing of which drives their reporting by the metrics reporting entity;

2b.	The metrics reporting entity locations where metrics collection is requested;

3.	The proportion of media delivery sessions for which metrics reports are to be sent by the metrics reporting entity;

4.	The portion of the media session (represented by start offset and/or duration parameters) for which metrics reports are to be sent by the metrics reporting entity if reporting is enabled for that media delivery session;

5.	The interval at which metrics reports are to be sent by the metrics reporting entity if reporting is enabled for a media delivery session; and

6.	The Media AF address(es) to which metrics reports are to be sent.

The Service Access Information indicating whether Metrics Reporting is provisioned for media delivery sessions is specified in clause 9.2.3.

The metrics reporting entity shall decide whether to activate the metrics reporting procedure for a particular media delivery session at the start of that session and whenever any Client Metrics Reporting Configuration changes in the related Service Access Information.

-	When the samplePercentage property in a Metrics Reporting Configuration has a value of 100 percent, the metrics reporting entity shall activate the metrics reporting procedure for that configuration.

-	If the samplePercentage value in a Metrics Reporting Configuration is less than 100 percent, the metrics reporting entity shall generate a random number which is uniformly distributed in the range of 0 to 100, and the metrics reporting entity shall activate the metrics reporting procedure for the Metrics Reporting Configuration when the generated random number is of a lower value than the samplePercentage value.

If the metrics reporting procedure is activated for a particular Client Metrics Reporting Configuration, the metrics reporting entity shall produce and submit a metrics report to the Media AF using the procedure specified in clause 5.3.5.2 when any of the following conditions are met:

-	On determining the need to report ongoing QoE metrics for a media delivery session at periodic intervals determined by the reportingInterval property in the Client Metrics Reporting Configuration, provided that both of the following hold:

-	The time offset indicated in the reportingStartOffset property of the Client Metrics Reporting Configuration has passed since the start of the media delivery session; and

-	The time offset indicated in the reportingDuration property of the Client Metrics Reporting Configuration has not yet passed since the time offset indicated in the reportingStartOffset property.

-	At the end of the media delivery session.

Whenever a metrics report is produced for a given client metrics reporting configuration, the metrics reporting entity shall reset its reporting interval timer for that configuration to the value of the clientMetrics‌Reporting‌Configurations[].‌reportingInterval property and it shall begin countdown of the timer again. When the media delivery session comes to an end, the metrics reporting entity shall disable its reporting interval timer for all client metrics reporting configurations.

Details of the APIs supporting these procedures at reference points M3 and M5 are specified in clause 9.5.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.1.6 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.1.7 shall be included in the response message.

#### 5.3.5.2	Submit metrics report operation

This operation is used by the Media Session Handler or Media AS to submit a metrics report to the Media AF. If several Media AF addresses are listed in the serverAddresses array of the client metrics reporting configuration (see table 9.2.3.1-1), the metrics reporting entity shall choose one at random and shall send the metrics report to the selected server endpoint. The HTTP POST method shall be used for this purpose, citing the address of the chosen Media AF in the request URL. The request body shall be formatted according to the metrics scheme indicated in scheme property of one of the Client Metrics Reporting Configurations (see clause 5.3.2.3 and table 9.2.3.1-1) and the Content-Type HTTP request header set accordingly. Details of individual metrics reporting schemes and their corresponding metrics report formats are beyond the scope of the present document.

A reporting client identifier should be included in the metrics report if the metrics scheme supports carriage of this data. Metrics schemes designed for use with this operation should specify a means to convey a reporting client identifier. If available to the metrics reporting entity, its value should be a GPSI value as defined by TS 23.003 [16]. Otherwise, the reporting client identifier should be represented by a stable and globally unique string.

If the HTTP request is acceptable but the Media AF has not yet fully processed the submitted metrics report, the Media AF may return a 202 (Accepted) HTTP response message with an empty body and process the report later.

If the operation is otherwise successful, the Media AF shall return a 200 (OK) HTTP response message with an empty body to acknowledge successful processing of the metrics report.

If metrics reporting is not provisioned for the Provisioning Session in question, the Media AF shall return a 403 (Forbidden) HTTP response message with an error message body per clause 7.1.7 and the Media AF shall not process the submitted report.

If the HTTP request message indicates a MIME content type in the Content-Type request header that is not consistent with one of the provisioned metrics reporting schemes, the Media AF shall return a 415 (Unsupported Media Type) HTTP response message with an error message body per clause 7.1.7 and shall not process the submitted metrics report.

If the target Media AF endpoint is temporarily unable to accept the submitted metrics report (e.g. because it is overloaded), it shall return a 503 (Service Unavailable) HTTP response message with an empty body. The optional HTTP response header Retry-After should be included in such a response, indicating when the Media AF expects to be able to accept new submissions. In this case, the metrics reporting entity should store outstanding metrics reports and reattempt submission when the endpoint later becomes available. Details are left to implementation.

Next change

### 5.3.6	Consumption reporting

#### 5.3.6.1	Procedures

These procedures are used by the Media AS at reference point M3 or else by the Consumption Reporting functions of the Media Client and subsequently by the Media Session Handler at reference point M5 to submit a consumption report to one of the Media AF instances listed in the Client Consumption Reporting Configuration of the Service Access Information resource previously retrieved using the procedure in clause 5.3.2.3.

-	When the Consumption Reporting feature is provisioned for downlink media delivery sessions using the operations specified in clause 5.2.12, a Client Consumption Reporting Configuration shall be provided to the Media Session Handler (acting in this case as consumption reporting entity) in the Service Access Information.

NOTE:	The Consumption Reporting feature is not applicable to uplink media streaming.

-	When the Consumption Reporting feature is provisioned for RTC sessions using the operations specified in clause 5.2.12, a Client Consumption Reporting Configuration shall be provided to the Media Session Handler and the Media AS (both acting in this case as consumption reporting entity) in the Service Access Information.

Furthermore:

-	Before a media delivery session is started, the consumption reporting entity shall check if the Service Access Information includes a Client Consumption Reporting Configuration. If such a configuration is present, the consumption reporting entity shall initiate consumption reporting for the downlink media delivery session based on this configuration.

NOTE:	The Client Consumption Reporting Configuration is absent in the case of uplink media streaming.

-	During the course of the media delivery session, the consumption reporting entity shall periodically check if the Client Consumption Reporting Configuration has been added to or removed from the Service Access Information and shall activate or deactivate the consumption reporting procedure as appropriate for the media delivery session in question.

The Client Consumption Reporting Configuration signalled as part of the Service Access Information at reference point M5 is specified in clause 9.2.3.

The consumption reporting entity shall decide whether to activate the consumption reporting procedure for a particular media delivery session at the start of that session and whenever the Client Consumption Reporting Configuration changes in the related Service Access Information.

-	When the samplePercentage property in the Client Consumption Reporting Configuration has a value of 100 percent, the consumption reporting entity shall activate the consumption reporting procedure.

-	If the samplePercentage value in the Client Consumption Reporting Configuration is less than 100 percent, the consumption reporting entity shall generate a random number which is uniformly distributed in the range of 0 to 100, and the consumption reporting entity shall activate the consumption reporting procedure when the generated random number is of a lower value than the samplePercentage value.

If the consumption reporting procedure is activated, the consumption reporting entity shall produce and submit a consumption report to the Media AF using the procedure specified in clause 5.3.6.2 when any of the following conditions are met:

-	At the start of consumption of a media item in a media delivery session.

-	At the end of consumption of a media item in a media delivery session.

-	On determining the need to report ongoing content consumption for a media delivery session at periodic intervals determined by the reportingInterval property in the Client Consumption Reporting Configuration.

-	On detecting a location change during a media delivery session, if the locationReporting property in the Client Consumption Reporting Configuration is set to true.

-	On detecting a change of access network during a media delivery session, if the accessReporting property in the Client Consumption Reporting Configuration is set to true.

Whenever a consumption report is produced, the consumption reporting entity shall reset its consumption reporting interval timer to the value of the reportingInterval property of the Client Consumption Reporting Configuration and it shall begin countdown of the timer again. When the media delivery session comes to an end, the consumption reporting entity shall disable its consumption reporting interval timer.

Details of the APIs supporting these procedures at reference points M3 and M5 are specified in clause 9.6.

The consumption report shall comprise a time-ordered list of consumption reporting units. Each such unit shall describe the media selected for presentation during a continuous time period of a downlink media streaming session in terms of a start time and duration. The sequence of consumption reporting units shall be contiguous with no discontinuities in the reported timeline. When no media is being consumed (e.g., because the media streaming presentation is paused), the selected media shall still be indicated in the consumption reporting unit.

-	A consumption reporting unit shall be included in exactly one consumption report, although delivery of this report may be attempted more than once by the consumption reporting entity.

-	A new consumption reporting unit shall be created when the media consumed changes or (if provisioned in the consumption reporting configuration per clause 4.3.8) when the network used to access media at reference point M4d changes, including a change of network slice and/or Data Network.

-	The last (or only) consumption reporting unit in every consumption report describes the media currently being consumed in the media streaming session and indicates in the duration property how long this media has been consumed so far.

-	If there is no change in the media consumed when the next consumption report is sent to the Media AF, this consumption reporting unit shall be repeated as the first (and possibly only) consumption reporting unit in the next report with the same start time but with its duration updated to reflect the period of time that the media has been consumed up to the point of reporting.

-	The last (or only) consumption reporting unit in the final consumption report sent to the Media AF at the end of the media streaming session therefore describes the last media consumed.

HTTP responses for successful and operation-specific failure cases are specified in the following clauses. For all other failure cases, an HTTP response indicating a response code in accordance with clause 7.1.6 shall be returned to the API client. In all failure cases a message body in accordance with clause 7.1.7 shall be included in the response message.

#### 5.3.6.2	Submit consumption report operation

This operation is used by the Media Session Handler or Media AS to submit a consumption report to the Media AF. If several Media AF addresses are listed in the serverAddresses array of the Client Consumption Reporting Configuration (see table 9.2.3.1-1), the consumption reporting entity shall choose one at random and shall send the consumption report to the selected server endpoint. The HTTP POST method shall be used for this purpose, citing the address of the chosen Media AF in the request URL. The request body shall be a ConsumptionReport structure, as specified in clause 9.6.3.1.

A reporting client identifier shall be included in the consumption report. If available to the consumption reporting entity, its value should be a GPSI value as defined by TS 23.003 [16]. Otherwise, the reporting client identifier should be represented by a stable and globally unique string.

The location(s) of the UE when the media was consumed shall be included in every ConsumptionReportingUnit (see clause 9.6.3.2) if the locationReporting property in the Client Consumption Reporting Configuration is set to true.

If the HTTP request is acceptable but the Media AF has not yet fully processed the submitted consumption report, the Media AF may return a 202 (Accepted) HTTP response message with an empty body and process the report later.

If the operation is otherwise successful, the Media AF shall return a 200 (OK) HTTP response message with an empty body to acknowledge successful processing of the consumption report.

If consumption reporting is not provisioned for the Provisioning Session in question, the Media AF shall return a 403 (Forbidden) HTTP response message with an error message body per clause 7.1.7 and the Media AF shall not process the submitted consumption report.

If the target Media AF endpoint is temporarily unable to accept the submitted consumption report (e.g. because it is overloaded), it shall return a 503 (Service Unavailable) HTTP response message with an empty body. The optional HTTP response header Retry-After should be included in such a response, indicating when the Media AF expects to be able to accept new submissions. In this case, the consumption reporting entity should store outstanding consumption reports and reattempt submission when the endpoint subsequently becomes available. Details are left to implementation.

Next change

## 9.2 Service Access Information API

### 9.2.1	General

The Service Access Information API is used by the Media Session Handler and/or Media AS to obtain configuration information from the Media AF that enables it to use the other Media Session Handling APIs specified in clause 9.3 et seq.

Next change

### 5.3.3 Dynamic Policy invocation

#### 5.3.3.1	Procedures

TBD.

End of changes