3GPP TSG-SA4#130 	S4-241962

# Orlando, US, 18-22 November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.567 | CR |  | rev |  | Current version: | 0.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_IMS] Updates to procedures of session establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_IMS |  |  |  |  |  | Date: |  |  | 2024-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The general procedures for split rendering session establishment are mixing P2P and P2A scenarios in the same call flows in TS 26.567. It would be clearer to have a general procedure and separate procedures for different scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the general procedures in 7.1 Remove P2P related procedures from 7.1 and provide a separate procedure in 7.1.1. Fix bugs of procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Technical Specification and introducing misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 7.1	Procedures for session establishment

The figure 7.1-1 indicates the general procedures of split rendering session establishment including session setup and negotiation.

Figure7.1-1: General procedures for split rendering session establishment

The steps are as follows:

Step 1: The UE1 initiates an IMS session including bootstrap data channel establishment with the originating MF.

Step 2: UE1 sends a request to IMS AS to establish an application data channel for a split rendering session:

The UE1 can initiate a request of split rendering session establishment if its media capabilities cannot meet the related media rendering requirements. Then the UE1 calculates which objects can be rendered by itself based on its status and decides which part of the objects to be rendered in the UE1 and the others to be rendered in the IMS network.

Step 3: The IMS AS sends DCSF the event notifications including split rendering related information.

Step 4: The DCSF receives event notifications from the IMS AS and  processes the session establishment request based on the information in the notification (i.e. associated split rendering related information). The DCSF manages (if applicable) application data channel resources at the MF which meet the split rendering request, to instruct IMS AS to terminate the media flow of the UE1 to the MF.

Step 5 and 6: The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2.

Editor’s Note: MF discovery for split rendering is FFS.

Step 7: The IMS AS sends a split rendering session establishment request to the DC AS via the DCSF, , the request may include the information of the objects to be rendered in IMS network.

Step 8: The DC AS sends a description of the split rendering output to the IMS AS via the DCSF.

Step 9: The IMS AS sends the media resource allocation request to the MF, to reserve XR media rendering resource for the UE1.

Step 10: When the resources are allocated successfully, the MF returns a successful response to the IMS AS.

Step 11: The IMS AS returns a successful response to the UE1.

Step 12: Successful split rendering session is established between UE1 and MF through the application data channel.

Editor’s note: more details of session establishments for P2P and P2A scenarios are provided in clause 7.1.1 and 7.1.2.

7.1.1 Procedures for P2P session establishment

The figure 7.1.1-1 indicates the procedures of split rendering session establishment for P2P scenarios.

Figure7.1.1-1: Procedures for P2P split rendering session establishment

The steps are as follows:

Steps 1: IMS session establishment between UE1 and UE2, including bootstrap session establishment by UE1 and UE2 with originating MF.

Steps 2-12: Split rendering session establishment between UE1 and originating MF according to the steps 2-12 of clause 7.1.

Step 13: Split rendering session established for UE2 between the originating MF and UE2. Similarly, seps 2 to 12 are followed for UE2. Step 2 may be a reply to the application data channel establishment (re)INVITE from UE1.

Step 14: Successful split rendering session is established between UE1 and UE2 through the application data channel anchored by the MF.

_____________________ End of 1st Change________________________________