3GPP TSG-SA WG4 Meeting #130							S4-241970

USA, Orlando, 18 – 22 November 2024

Source:	Nokia

Title:	[FS_ARSpatial| pCR on spatial computing functions

Spec:	3GPP TR 26.819 v0.1.0

Agenda item:	9.9

Document for:	Agreement

1. Introduction

The Study on Spatial Computing for AR Services [FS_ARSpatial] was approved during SA#104 meeting. The core spatial computing functions have been documented in TR 26.819 in the SA4-129e meeting. Spatial computing includes functions such as world tracking, relocalization, anchoring, 3D model reconstruction, collider generation, segmentation and label, light extraction.

This contribution adds further details on the example given for 3D model reconstruction.

2. Reason for Change

Improve the clause describing 3D model reconstruction.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.819 v0.1.0.

* * * First Change * * * *

### 4.2.5	3D model reconstruction

Spatial computing enables the creation of accurate 3D models of surrounding space. It accurately captures real-world scenes and objects using 3D scanning techniques or photogrammetry. These 3D models can be displayed in immersive 3D environments in real-time to provide users with a sense of interactivity and presence.

The 3D model of a real-world environment may also be constructed collectively by aggregating meshes captured by an AR device.

HoloLens (Microsoft) and AR SDKs from Apple and Meta can all build a 3D model of the surrounding environment in real time.

To build the 3D model, the following input data can be used:

- Sensor data:

- Images captured by AR Device

- pose of AR Device

- Depth map (image or texture)

- Mesh captured by AR Device

The output of the 3D Model construction is a 3D Model. A 3D model is a broader term that encompasses the complete representation of a 3D object, including its geometry (which may be represented by a mesh), texture, and materials.

#### 4.2.5.1	Example: HoloLens (Microsoft)

HoloLens [9] has built-in cameras that continuously scan the environment, allowing it to construct virtual world geometry for real-world objects. Figure 4.2.5.1-1 demonstrates examples of 3D models constructed by HoloLens.

Figure 4.2.5.1-1: HoloLens 3D Model construction and spatial mapping (from Unity)

In HoloLens, spatial surfaces are represented by dense triangle meshes. To minimize the required processing and storage resources, applications can enable a mesh caching scheme appropriate for their needs. This scheme determines which meshes to keep, which to discard, and when to update the mesh for each spatial surface.

An application can define the regions of space in which the application wishes to receive spatial mapping data by defining bounding volumes. It is recommended in the documentation that new spatial surfaces and updated spatial surfaces near and in front of the user are given higher priority by the application when spatial data is requested.

HoloLens also enables visualization of the spatial mesh to give the user an understanding of the detected spatial surfaces. An application can use this feature e.g. to help the user when the user is trying to place a virtual object onto a surface.

More details on spatial mapping in HoloLens is provided here: https://learn.microsoft.com/en-us/windows/mixed-reality/design/spatial-mapping