Source:	Dolby Sweden AB

Title:	Proposal on IVAS renderer levels

Agenda item:	7.6

Document for:	    Discussion and Agreement

# Introduction

IVAS rendering complexity and memory requirements largely depend on the specifics of the used renderer option (internal, external or custom), on the specifics of a particular implementation, as well as on the rendered output format in relation to the received IVAS coding modes/coded format. The rendered output format generally depends on requirements imposed by the conditions of the audio backend.

To offer maximum immersive experience on a large range of UEs with different capabilities and audio backend conditions [3], the source suggests that IVAS compliant receive clients supporting levels 1, 2 or 3 [6] shall respectively meet the following requirements corresponding to renderer levels.

# Renderer level definition

# Binaural rendering levels

- Diotic headset UEs, electrical interface UEs (if intended for diotic headset usage), handheld hands-free UEs, table-mounted UEs:

  - Level 1: IVAS receive clients in these UEs shall support rendering of any IVAS level-1 decoded bitstream to stereo, binaural, head-tracked binaural.

  - Level 2: IVAS receive clients in these UEs shall support rendering of any IVAS level-2 decoded bitstream to stereo, binaural, head-tracked binaural, binaural with room reverb.

  - Level 3: IVAS receive clients in these UEs shall support rendering of any IVAS decoded bitstream to stereo, binaural, head-tracked binaural, binaural with room reverb.

- Digital interface UEs (if intended for split pre-renderer usage):

  - Level 1 (class a/b): IVAS receive clients in these UEs shall support non-diegetic and 0 DoF diegetic split pre-rendering of any IVAS level-1 decoded bitstream (except stereo).

  - Level 2 (class a/b): IVAS receive clients in these UEs shall support non-diegetic and 3 DoF diegetic low-complexity split pre-rendering of any IVAS level-2 decoded bitstream (except stereo).

  - Level 3 (class a/b): IVAS receive clients in these UEs shall support non-diegetic and 3 DoF diegetic high-quality split pre-rendering of any IVAS decoded bitstream (except stereo).

- Lightweight UEs, earbuds, HMDs, smartglasses, etc. (if intended for split post-renderer usage):

  - IVAS split rendering receive clients (class a/b) in these UEs shall support decoding and post rendering of any (class a/b) IVAS split renderer bitstream.

NOTE: Class a refers to LCLD coding [1] used for binaural transport channel. Class b refers LC3plus [4] coding used for binaural transport channel.

NOTE: Any IVAS receive clients shall be capable of decoding and rendering any IVAS bitstream to either MONO or immersive output formats.

# Loudspeaker rendering levels

The source suggests considering the following level definition for loudspeaker rendering.

- Loudspeaker hands-free UEs, electrical interface UEs (if intended for loudspeaker handsfree usage):

  - Level 1: IVAS receive clients in these UEs shall support rendering of any IVAS level-1 decoded bitstream to stereo.

  - Level 2: IVAS receive clients in these UEs shall support rendering of any IVAS level-2 decoded bitstream to stereo and multi-channel.

  - Level 3: IVAS receive clients in these UEs shall support rendering of any IVAS decoded bitstream to stereo and multi-channel.

NOTE: Mono or stereo decoded bitstreams do not need to be rendered to other formats than mono or stereo.

IVAS rendering is described in [1] and [2].

# Conclusion

A proposal on IVAS rendering levels was presented. The proposal follows the onion principal and defines the rendering levels as per playback device capability. Furthermore, the proposal allows for low complexity rendering options in lowest level and then incorporates relatively higher complexity rendering options in higher levels.

It is proposed to agree to the binaural rendering level definition and to discuss the loudspeaker rendering level definition and use both as a basis for a change request to IVAS general overview specification [5].

# References

- [1]	3GPP TS 26.253: "Codec for Immersive Voice and Audio Services (IVAS); Detailed Algorithmic Description including RTP payload format and SDP parameter definitions".

- [2]	3GPP TS 26.254: "Codec for Immersive Voice and Audio Services (IVAS); Rendering".

- [3]	3GPP TS 26.260: "Objective test methodologies for the evaluation of immersive audio systems”.

- [4]	ETSI TS 103 634: “Low Complexity Communication Codec Plus”.

- [5]	3GPP TS 26.250 - Codec for Immersive Voice and Audio Services (IVAS); General overview.

- [6]	Tdoc S4-241838: “TS26.250CR0002 on IVAS codec levels”, Dolby Sweden AB, Fraunhofer IIS, Nokia.