3GPP TSG-SA WG4 Meeting #130	S4-242000

# USA, Orlando, 18 – 22 November 2024

Source:	Nokia, Philips, Interdigital, Deutsche Telekom, Fraunhofer HHI, KDDI, Samsung, Sony, Huawei

Title:	Pseudo-CR on Streaming of Beyond 2D Produced VoD Content – Use Case Volumetric Video with single asset – Scenario sections 10, 11 and 12

Spec:	3GPP TR <26.956 version 0.1.1>

Agenda item:	9.8

Document for:	Decision

1. Introduction

This pCR adds elements for the scenario “Streaming of Beyond 2D Produced VoD Content – Use Case Volumetric Video with single asset” to the scenario clause 7.X in the TR.

2. Reason for Change

PD 0.0.4 (S4-241714) includes 5 scenarios including scenario “Streaming of Beyond 2D Produced VoD Content – Use Case Volumetric Video with single asset”, which relies on dense point clouds as representation format. TR 0.1.0 includes already dense point clouds as a representation format and therefore it is proposed to include the corresponding scenario. This document addresses detailed test conditions in section 10, external performance data in section 11 and additional information in section 12 of the scenario template.

3. Conclusions

Without the scenario description the motivation of the dense point cloud representation format may not be well understood, which makes the FS_Beyond2D study less comprehensive.

4. Proposal

It is proposed to agree the following changes to 3GPP TS <26.956  version 0.1.1>.

* * * First Change * * * *

<Proposed change in revision marks>

## 7.x	Scenario x: Streaming of Beyond 2D Produced VoD Content – Use Case Volumetric Video with single asset

### 7.x.1	Scenario name

Streaming of Beyond 2D Produced VoD Content – Use Case Volumetric Video with single asset.

NOTE: Streaming of Beyond 2D Produced Content - Multi-view Video is not a part of this scenario and handled separately in the scenario described in clause 7.y.

### 7.x.10	Detailed test conditions

Provides a proposal for detailed test conditions, for example based on a reference software 	together with the sequences and configuration parameters.

#### 7.x.10.1	V-PCC test model and configuration files

The public version of the MPEG V-PCC test model named tmc2 in release R24.0 master branch is used to encode and decode dense dynamic point clouds [R1].

For using tmc2 in Random Access (RA) mode, MPEG provides a configuration file [R2].

For each selected test sequence, a configuration file containing information needed for tmc2 configuration will be provided.

#### 7.x.10.2	Rate points and test conditions

In line with the V-PCC verification test [R3], 5 rate points R1 to R5 for Random Access (RA) will be used for each test sequence. Rates are obtained with the following V-PCC codec parameters:

|  | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| QP geometry | 32 | 28 | 24 | 20 | 16 |
| QP Texture | 42 | 37 | 32 | 27 | 22 |
| Occupancy Precision | 4 | 4 | 4 | 4 | 2 |

For using this rate configuration, MPEG provides configuration files for each rate [R4].

#### 7.x.10.3	Profiles

In line with the V-PCC verification test [R3], the following profiles are evaluated:

- Basic which corresponds to V-PCC profile HEVC Main10 V-PCC Basic Rec0. This profile uses the basic V-PCC toolset and a simple reconstruction of points in 3D space.

- Enhanced which corresponds to V-PCC profile HEVC Main10 V-PCC Basic Rec2. This profile uses the basic V-PCC toolset and a more sophisticated reconstruction of points in 3D space.

- High which corresponds to V-PCC profile HEVC Main10 V-PCC Extended Rec2. This profile uses the extended V-PCC toolset and a more sophisticated reconstruction of points in 3D space.

#### 7.x.10.4	Bitstream Generation, output

The MPEG V-PCC test model will be used to encode and decode test sequences as described previously [R1].

To compute metrics, a patched version of mpeg-pcc-mmetric is provided for internal usage [R5]. The software will be provided and derived from the branch “bugfixes_gcc8.5_eqTFAN” with version v1.1.6. The metric software implements the “point-based” metric and the PCQM metric as described in section 7 of this scenario. A patch to enable computation of the Point2Plane metric is needed on top of this version. The metric software with patch will be provided.

Python scripts will be provided to be able to:

- Encode each sequence for each condition, rate and profile

- Decode the corresponding sequence

- Compute metrics

- Generate tables and graphs

Below are examples of command lines for the basic profile for a vox11 sequence:

- to encode a test sequence:

mpeg-pcc-tmc2/bin/PccAppEncoder \

--config=mpeg-pcc-tmc2/cfg/common/ctc-common.cfg \

--config=mpeg-pcc-tmc2/cfg/condition/ctc-random-access.cfg \

--config=mpeg-pcc-tmc2/cfg/sequence/${test_sequence}.cfg \

--config=mpeg-pcc-tmc2/cfg/rate/ctc-r1.cfg \

--configurationFolder=mpeg-pcc-tmc2/cfg/ \

--uncompressedDataFolder=${source_sequence}/ \

--compressedStreamPath=${test_sequence}.bin \

--normalDataPath=${source_sequence}/${source_sequence}_%04d.ply \

--frameCount=32 \

--resolution=2047 \

--mapCountMinus1=0 \

--profileToolsetIdc=0 \

--profileReconstructionIdc=0

- to decode a test sequence:

mpeg-pcc-tmc2/bin/PccAppDecoder \

--startFrameNumber=0 \

--compressedStreamPath=${test_sequence}.bin \

--reconstructedDataPath=${test_sequence}_dec_%04d.ply \

--inverseColorSpaceConversionConfig=mpeg-pcc-tmc2/cfg/hdrconvert/yuv420toyuv444_16bit.cfg

- to compute metrics of a test sequence:

mpeg-pcc-mmetric/build/Release/bin/mm sequence \

--firstFrame 0 \

--lastFrame 31 END \

compare --mode pcc \

--inputModelA source_sequence_%04d.ply \

--inputModelB test_sequence_dec_%04d.ply END \

compare --mode pcqm \

--inputModelA source_sequence_%04d.ply \

--inputModelB test_sequence_dec_%04d.ply

For each test, outputs are:

- Bitstream file

- Log files containing metrics information

    - Encoder log output

    - Decoder log output

    - Metric log output

A CSV file containing concatenated metrics information for each condition and one profile is generated for all sequences and rates.

The following information will be stored:

- SeqId: identifier of the sequence

- CondId: tested condition (RA)

- RateId: tested rate number [R1..R5]

- nbFrame: number of tested frames

- NbInputPoints: number of points in the source sequence

- NbOutputPoints: number of points in the candidate test sequence

- MeanOutputPoints: mean number of points in the candidate test sequence

- MeanDuplicatePoints: mean number of duplicated points (with same geometry) in the candidate test sequence

- TotalBitstreamBits: size of the bistream in bits

- geometryBits: size of the geometry stream in bits

- metadataBits: size of the metadata stream in bits

- attributeBits: size of the attribute stream in bits

- D1Mean: mseF,PSNR (p2point)

- D2Mean: mseF,PSNR (p2plane)

- LumaMean: c[0],PSNRF

- CbMean: c[1],PSNRF

- CrMean: c[2],PSNRF

- PCQM: PCQM PSNR

- SelfEncoderRuntime: encoder time for current process

- ChildEncoderRuntime: encoder time for child processes

- SelfDecoderRuntime: decoder time for current process

- ChildDecoderRuntime: decoder time for child processes

From this CSV file, an excel spreadsheet is generated to get tables and graphs for interpretation of the results.

#### 7.x.10.5	Videos Generation for subjective tests

The representative renderer [R6] used is derived from the mpeg-pcc-renderer where a new rendering mode has been added named “splat blend” mode. This mode draws a camera facing semitransparent splat of radius PointSize centered on the point position. The transparency (or alpha) varies with the distance from the center from fully opaque at the center to nearly transparent at the edge, to provide blending between points and reduce aliasing. In order to correctly use alpha blending, the points are sorted relative to the camera plane. Two blending modes are available, gaussian and linear, and the alpha falloff speed is customizable using the "pointFocus" setting.

To avoid interference between the background and the test material, a neutral background with a color will be selected. A floor makes the rendered scene more realistic by preventing interference with the test material.

The representative renderer software will be provided. A script will be provided to generate videos with chosen camera path.

For each sequence, a video is generated. A camera path and blend parameter options adapted to each sequence is provided [R8]. The output of this video generation with a camera path is stored as high-quality video sequence of a length as close as possible to 10s.

The generation is done into two steps: first the generation of the RBG raw file with the camera path and then the conversion into YUV or MP4 files.

Here is an example of the two command lines:

- To generate a RGB raw file with a specific camera path

./PccAppRenderer.exe -d ${test_sequence}\

-i 1 \

-n 32 \

--play=1 \

--playBackward=1 \

--height=1080 --width=1920 \

--type=3 --alphaFalloff=6 --size=3.2 --blendMode=0 \

--floor=1 --overlay=0 \

--camera=${camera_path}.txt

--RgbFile=${test_sequence_rgb}

- To convert the RGB file to x265 lossless mp4 file

./scripts/convert_video.sh \

-o ${test_sequence}_x265lossless.mp4  \

--videoType=2

--ffmpeg=ffmpeg.exe

-i ${test_sequence_rgb}.rgb

The video sequences are generated with the following video parameters:

- Video resolution: progressive uncompressed full-range HD format (1920x1080). Note that upsampling by the TV set should be avoided

- Frame rate: The frame rate will be aligned with the frame rate in the test data set

- Color space: ITU-R BT.709

- Sub-sampling: 4:2:0 YUV 10 bits or x265 fast preset lossless

#### 7.x.10.6		Verification / crosschecks

All produced bitstreams, metric results and produced videos will be crosschecked by at least one other SA member to ensure that results are correct.

### 7.x.11	External Performance data

References to external performance data that can be added, for example other SDOs, public documents and so on.

The subjective verification test report for V-PCC can be downloaded from the public MPEG website [R3]

The Brazilian SBTVD Forum performed objective tests with V-PCC. Full results are available in chapter 6.10 (Candidate Technology I), 6.10.3.2 and 6.10.4 of the following document [R9]

### 7.x.12	Additional information

- Industry activities

Is there Relevant work in industry forums?

- Implementation constraints

Are there any indications about scalability of the technology with regards to network and devices?

- Innovation

Does the technology address a current or a future need on the market? Can it potentially disrupt existing markets?

Sequences can be decoded and visualized in real time using a 3D background or in Augmented Reality on a smartphone, tablet, head-mounted display using DASH streaming mode or local file system.

Nokia’s real-time V-PCC decoder implementation that was released as open source: https://github.com/nokiatech/vpcc

A simple scene description could be added to enable the placement of the asset in the scene (position, orientation, scale…) but is outside the scope of this document, which is focused on the format and codec evaluation.

* * * Next Change * * * *

<Proposed change in revision marks>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[R1] MPEG, V-PCC test model tmc2, https://github.com/MPEGGroup/mpeg-pcc-tmc2

[R2] MPEG, Random Access configuration, https://github.com/MPEGGroup/mpeg-pcc-tmc2/blob/master/cfg/condition/ctc-random-access.cfg

[R3] MPEG, Subjective verification test report for V-PCC, https://www.mpeg.org/wp-content/uploads/mpeg_meetings/136_OnLine/w20992.zip

[R4] MPEG, Rate configuration, https://github.com/MPEGGroup/mpeg-pcc-tmc2/tree/master/cfg/rate

[R5] Patched mpeg-pcc-mmetric, TBD

[R6] Representative renderer, TBD

[R8] List of camera paths and splat blend parameter options for each tested sequence

[R9] SBTVD, SBTVD TV 3.0 test report for video, https://forumsbtvd.org.br/wp-content/uploads/2021/12/SBTVD-TV_3_0-VC-Report.pdf

* * * Next Change * * * *

<Proposed change in revision marks>

* * * End of Changes * * * *