Agenda item: 	8.6

Source: 	Qualcomm Inc.

Title: 	[FS_AMD]  L4S for 5G Media Delivery

Document for	Discussion and Agreement

# Introduction

In this contribution, we propose an introduction to L4S and discuss the usage of L4S in the context of advanced media delivery.

# L4S Background

L4S stands for Low Latency, Low Loss, Scalable Throughput. L4S is based on the idea that delay is mainly caused by the classic congestion control algorithms introduced with TCP. L4S replaces these traditional congestion control algorithms with a class of scalable congestion control algorithms.

The L4S architecture relies on 3 components to operate:

- A scalable congestion control algorithm,

- A modified ECN marking behavior,

- An active queue management algorithm that isolates L4S traffic

An example of a scalable congestion control algorithm that is widely deployed today is TCP Prague. In TCP Prague the congestion window is adjusted proportionally to the probability of receiving an ECN mark.

cwnd= 1pα, where p is the probability of receiving an ECN mark, and α is a constant

TCP Prague further adjust the congestion window to implement an RTT independence, which is crucial to not overly react to RTT variations. The additive increase part of the algorithm is also adjusted to ensure that flows with short RTT are not penalized by a slow increase.

Another popular scalable congestion control algorithm is BBRv2, which stands for Bottleneck Bandwidth and Round-trip propagation time. BBRv2 continuously estimates the bottleneck bandwidth of the connection and the RTT. It then uses the 2 parameters to adjust its congestion window as follows:

cwnd=BtlBW*RTprop, where BtlBW is the estimated bottleneck bandwidth and RTprop is the estimated minimum round-trip propagation time. The algorithm’s reaction to packet loss and ECN marks is also modified to be less aggressive, compared to classic congestion control algorithms. To accurately estimate the bottleneck bandwidth, BBRv2 periodically sends data at a high rate (thank the estimated bottleneck bandwidth) to probe the network for the maximum throughput. BBRv2 also tracks the data that is sent and received during a RTT cycle.

The ECN mechanism is also modified in L4S. The default reaction to an ECN mark is to treat it as an equivalent to a packet loss. This is no more the case in L4S, which allows the network to signal the start of congestion more frequently without the risk of messing up the connection’s bandwidth. This change requires the identification of L4S connections by the network. The ECT code point is used for this purpose. When set to 1, the network identifies the sender as L4S capable and deploys the fine-tuned congestion notification mechanism.

Finally, the queue management approach closes the loop for L4S. RFC9332 [3] defines one such management approach, the Dual Queue Coupled Active Queue Management, which in essence separates the L4S and non-L4S traffic into two separate queues for differentiated latency treatment and ECN marking. The available bandwidth is still shared among both queues though through the coupling mechanism.

# Existing APIs for Application Access to L4S

Currently, only Apple platforms have functionality that allows access to ECN status through IP packet metadata [4]. With the proliferation of L4S, it is expected that more APIs will be introduced to enable this access.

The following example shows how this access can be achieved:

import Network class ECNMonitor { var connection: NWConnection? func setupConnection(to endpoint: NWEndpoint) { // Create connection parameters let parameters = NWParameters() parameters.allowLocalEndpointReuse = true // Enable IP metadata for ECN access parameters.requireIPMetadata = true // Create the connection connection = NWConnection(to: endpoint, using: parameters) // Set up receive handler connection?.receiveMessage { [weak self] content, context, isComplete, error in if let context = context { // Get IP metadata from context let metadata = context.protocolMetadata.first { $0 is NWProtocolIP.Metadata } as? NWProtocolIP.Metadata if let metadata = metadata { // Access ECN flags using nw_ip_metadata_get_ecn_flag let ecnFlag = nw_ip_metadata_get_ecn_flag(metadata) // Interpret ECN flags switch ecnFlag { case 0: // Non-ECT print("Packet is Not-ECN-Capable Transport (Non-ECT)") case 1: // ECT(1) print("Packet is ECN Capable Transport (1)") case 2: // ECT(0) print("Packet is ECN Capable Transport (0)") case 3: // CE print("Packet has Congestion Experienced (CE)") default: print("Unknown ECN flag value") } } } // Continue receiving … } // Start the connection connection?.start(queue: .main) } … }

This API allows applications to:

- Enable ECN metadata access through connection parameters

- Access raw ECN flags from each received packet

- Distinguish between all ECN codepoints (Non-ECT, ECT(0), ECT(1), and CE)

The ECN flags follow the standard encoding defined in RFC 3168:

- 00: Not-ECN-Capable Transport (Non-ECT)

- 10: ECN Capable Transport (0)

- 01: ECN Capable Transport (1)

- 11: Congestion Experienced (CE)

For L4S deployment in media delivery, applications can use this API to implement appropriate congestion control responses to ECN marks.

# Usage of ECN marking in Media Streaming

It is expected that the protocol stack, whether QUIC or TCP, will automatically handle the ECN marking on received IP packets by echoing them to the sender. This approach is not suitable for media streaming that uses a receiver-driven rate adaptation scheme. Instead of waiting for this ECN indication to trickle down to a reduced sender transmission window and ultimately into high latency for receiving a media segment, the client should pro-actively react to these ECN notifications upon discovering them. They can then activate their rate adaptation algorithm and switch to a lower Representation early on.

For this to work, the M7 interface between the MSH and the application needs to be enhanced with L4S negotiation information, allowing the MSH to indicate that L4S support is available and the application to activate it for its network connections and share the information back to the MSH.

# Proposal

We propose to add the content of section 3 to the description of WT#4 on QoS. We further propose to define the M7 API extensions to enable the negotiation of the usage of L4S between MSH and application.

# References

[1]		IETF RFC 9330, Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture

[2]		IETF RFC 9331, The Explicit Congestion Notification (ECN) Protocol for Low Latency, Low Loss, and Scalable Throughput (L4S)

[3]		IETF RFC 9332, Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)

[4]		Apple Developer Documentation, nw_ip_metadata_get_ecn_flag(_:) | Apple Developer Documentation