Title:		          Updates to Clause 4, 7 Annex of TR 26.933

Source:	Panasonic Holdings Corporation

Document for:	Discussion & Agreement

Agenda item:	7.6

1. Background

In the SA4 #129-e post meeting, a contribution (S4aA240046) was submitted, identifying gaps and inconsistencies in clauses 4-7. As a continuation, supplemental data has been collected for inclusion into TR 26.933 [1].

2. Discussion and Proposal

Discussion

Foldable mobile phones are gaining significant growth in the market adoption, with expansion expected to continue in the coming years as highlighted in [2]. Advances in flexible display technology and the introduction of new form factors are likely driving consumer interest.

The source has noted that the information in the TR does not address this category of devices. Additionally, these devices, when folded, are nearly half the size of conventional mobile phones depends on the foldable design, which is not reflected in the data available in the TR. The flexible design of foldable phones introduces considerable variation in audio capture, an aspect relevant to the study item’s scope. Therefore, the source analyzed the latest foldable devices on the market, focusing primarily on their dimensions, microphone configuration and designs. Foldable mobile phones come in differing structural approaches, primarily it can be categorized into book-style and clamshell-style.

Book-Style Foldable devices generally offer larger inner displays suitable for multitasking and media consumption, making them ideal for productivity. These devices when closed looks bulkier (in depth). Following figure presents the example book-style foldable mobile design.

Figure 1: Example Book-style foldable mobile design

Clamshell-style foldables are more compact when folded and lightweight, making them easier to carry while still offering a substantial display when opened. Following figure presents the example clamshell-style foldable mobile design.

Figure 2: Example Clamshell-style foldable mobile design

Dimension of the foldable devices

The Tendency of the book-style, clamshell-style foldable design dimensions since launch mainly shifting towards reducing the depth of the device when folded. Following figures presents the trend in book-style, clamshell foldable design length, width, depth in recent years.

Book-style Foldables

Unfolded

The maximum dimensions for length, width, and depth are 17.33 cm (L),14.62 cm (W) and 0.8 cm (D) respectively.

The minimum dimensions are 13.22 cm (L), 12.7 cm (W), and 0.435 cm (D).

The average dimensions are 15.55 cm (L), 13.75 cm (W) and 0.59 cm (D).

The 95% CI for length, width, and depth are (15.15 cm, 15.95 cm), (13.44 cm, 14.06 cm) and (0.54 cm, 0.64 cm) respectively.

Folded

The maximum dimensions for length, width, and depth are 17.33 cm (L),7.83 cm (W) and 1.72 cm (D) respectively.

The minimum dimensions are 13.22 cm (L), 6.3 cm (W), and 0.92 cm (D).

The average dimensions are 15.55 cm (L), 7.2 cm (W) and 1.29 cm (D).

The 95% CI for length, width, and depth are (15.15 cm, 15.95 cm), (7.02 cm, 7.38 cm) and (1.18 cm, 1.4 cm) respectively.

Figure 3: Tendency of Book-Style Mobile Phone Length (un-fold/fold)

Figure 4: Tendency of Book-Style Mobile Phone Width (Un-fold)

Figure 5: Tendency of Book-Style Mobile Phone Depth (Un-fold)

Figure 6: Tendency of Book-Style Mobile Phone Width (Fold)

Figure 7: Tendency of Book-Style Mobile Phone Depth (fold)

Clamshell-Style Foldables

Unfolded

The maximum dimensions for length, width, and depth are 17.13 cm (L), 7.98 cm (W) and 0.79 cm (D) respectively.

The minimum dimensions are 16.51 cm (L), 7.19 cm (W), and 0.69 cm (D).

The average dimensions are 16.8 cm (L), 7.43 cm (W) and 0.73 cm (D).

The 95% CI for length, width, and depth are (16.7 cm, 16.9 cm), (7.33 cm, 7.53 cm) and (0.71 cm, 0.75 cm) respectively.

Folded

The maximum dimensions for length, width, and depth are 9.17 cm (L),7.98 cm (W) and 1.73 cm (D) respectively.

The minimum dimensions are 7.4 cm (L), 7.19 cm (W), and 1.49 cm (D).

The average dimensions are 8.65 cm (L), 7.43 cm (W) and 1.58 cm (D).

The 95% CI for length, width, and depth are (8.47 cm, 8.83 cm), (7.33 cm, 7.53 cm) and (1.54 cm, 1.62 cm) respectively.

Figure 8: Tendency of Clamshell-Style Mobile Phone Length (Un-fold)

Figure 9: Tendency of Clamshell-Style Mobile Phone Length (fold)

Figure 10: Tendency of Clamshell -Style Mobile Phone width (Fold/Un-fold)

Figure 11: Tendency of Clamshell-Style Mobile Phone Depth (Un-fold)

Figure 12: Tendency of Clamshell-Style Mobile Phone Depth (Fold)

Microphone Integration in the foldable mobile phones

MEMS microphones are the standard for both foldable and non-foldable mobile phones. The number and placement of microphones on foldable devices (including both book-style and clamshell designs) vary by manufacturer and model. Basic configurations in the foldable segment includes 3 microphones (positioned at the top, bottom bezel and rear facing), while more advanced models offer up to 6 microphones (with configurations of 2 microphones at the top, 2 at the bottom bezel, and up to 2 rear-facing). An increased number of microphones can significantly enhance audio quality during calls, contributing to a better overall user experience. Given the flexible design of foldable devices, various usage modes can be expected (example: full-screen, half-folded at a 90-degree angle, single-handed mode, desk mode, etc.,), therefore more attention may be needed when processing captured signals from multi-microphones positioned strategically to support high-quality audio capture. Following figures provide example microphone configurations for book-style and clamshell-style design.

Figure 13: Example Book-Style foldable design with microphone configuration

Figure 14: Example Clamshell-Style foldable design with microphone configuration

The table below presents example characteristics of multi-microphone configurations for foldable devices. Due to the flexible design of these devices, various usage modes may alter the spacing between microphones, for example when clamshell device is half-folded at a 90-degree angle or in a desk mode spacing and microphone configuration varies. As a result, processing solutions may need to adapt to the adjustable microphone array configuration to deliver high-quality immersive audio quality.

Table 1: MEMS Microphone example characteristics for multi-array microphones in foldable mobile phones.

| Foldable Mobile Phone | Number of Microphones | Placement of Microphones | Spacing | Polar Pattern |
| --- | --- | --- | --- | --- |
| Book-Style (folded) | Triple Microphone array | Top, Bottom of bezel and Rear facing | 2 to 17 cm | Omni-directional |
|  | Hexa Microphone array | 2 microphones at the top, 2 at the bottom bezel, and up to 2 rear-facing | 4 to 17 cm |  |
| Book-Style (un-folded) | Triple Microphone array | Top, Bottom of bezel and Rear facing | 8 to 18 cm |  |
|  | Hexa Microphone array | 2 microphones at the top, 2 at the bottom bezel, and up to 2 rear-facing | 2 to 18 cm |  |
| Clamshell-Style (folded) | Triple Microphone array | Top, Bottom of bezel and Rear facing | 2 to 4 cm |  |
|  | Hexa Microphone array | 2 microphones at the top, 2 at the bottom bezel, and up to 2 rear-facing | 1 to 4 cm |  |
| Clamshell-Style (un-folded) | Triple Microphone array | Top, Bottom of bezel and Rear facing | 2 to 18 cm |  |
|  | Hexa Microphone array | 2 microphones at the top, 2 at the bottom bezel, and up to 2 rear-facing | 2 to 18 cm |  |

Proposal

Change-1

Source proposes to split clause 4.1 mobile phones into following two sub-clauses

4.1.1 Traditional Mobile Phones

4.1.2 Foldable Mobile Phones

Move the 4.1 contents in TR to 4.1.1 sub-clause and include the following information in sub-clause 4.1.2.

4.1.2 Foldable Mobile Phones

4.1.2.1 Introduction

Foldable mobile phones are gaining significant growth in the market adoption, advances in flexible display technology and the introduction of new form factors are likely driving consumer interest. Foldable smartphones come in various dimensions, primarily it can be categorized into book-style and clamshell-style designs. Book-style foldables are generally larger in width when opened and bulkier in depth when folded while the clamshell style foldables are compact when folded while offering a substantial display when opened. The flexible design of foldable phones introduces considerable variation in audio capture.

4.1.2.2 Book-Style Foldables

Book-style foldables are generally offer larger display when opened suitable for multi-tasking and media consumption. These devices when closed looks bulkier (in-depth), however recent trends have shown decrease in tendency in the depth. Detailed data on these dimensions are listed in Annex A.1.2.

Unfolded

The maximum dimensions for length, width, and depth are 17.33 cm (L),14.62 cm (W) and 0.8 cm (D) respectively.

The minimum dimensions are 13.22 cm (L), 12.7 cm (W), and 0.435 cm (D).

The average dimensions are 15.55 cm (L), 13.75 cm (W) and 0.59 cm (D).

The 95% CI for length, width, and depth are (15.15 cm, 15.95 cm), (13.44 cm, 14.06 cm) and (0.54 cm, 0.64 cm) respectively.

Folded

The maximum dimensions for length, width, and depth are 17.33 cm (L),7.83 cm (W) and 1.72 cm (D) respectively.

The minimum dimensions are 13.22 cm (L), 6.3 cm (W), and 0.92 cm (D).

The average dimensions are 15.55 cm (L), 7.2 cm (W) and 1.29 cm (D).

The 95% CI for length, width, and depth are (15.15 cm, 15.95 cm), (7.02 cm, 7.38 cm) and (1.18 cm, 1.4 cm) respectively.

4.1.2.3 Clamshell-Style Foldables

Clamshell-style foldables are more compact in length when folded and lightweight, making them easier to carry while still offering a substantial display when opened in full-screen. Detailed data on these dimensions are listed in Annex A.1.3.

Unfolded

The maximum dimensions for length, width, and depth are 17.13 cm (L), 7.98 cm (W) and 0.79 cm (D) respectively.

The minimum dimensions are 16.51 cm (L), 7.19 cm (W), and 0.69 cm (D).

The average dimensions are 16.8 cm (L), 7.43 cm (W) and 0.73 cm (D).

The 95% CI for length, width, and depth are (16.7 cm, 16.9 cm), (7.33 cm, 7.53 cm) and (0.71 cm, 0.75 cm) respectively.

Folded

The maximum dimensions for length, width, and depth are 9.17 cm (L),7.98 cm (W) and 1.73 cm (D) respectively.

The minimum dimensions are 7.4 cm (L), 7.19 cm (W), and 1.49 cm (D).

The average dimensions are 8.65 cm (L), 7.43 cm (W) and 1.58 cm (D).

The 95% CI for length, width, and depth are (8.47 cm, 8.83 cm), (7.33 cm, 7.53 cm) and (1.54 cm, 1.62 cm) respectively.

Change-2

Rename Annex A.1.1 to Traditional Mobile Phone Size

Add Annex A.1.2: Book-style foldable mobile phone size

Add Annex A.1.3: clamshell-style foldable mobile phone size

### A.1.2	Book-style foldable structure size

### A.1.2.1	Length

The trend in book-style foldable design length (fold/unfold) in recent years is shown in the following figure

Figure A.1.2.1.1: Tendency of Book-Style Mobile Phone Length (un-fold/fold)

### A.1.2.2	Width

Figure A.1.2.2.1: Tendency of Book-Style Mobile Phone Width (Un-fold)

Figure A.1.2.2.2: Tendency of Book-Style Mobile Phone Width (Fold)

### A.1.2.3	Depth

Figure A.1.2.3.1: Tendency of Book-Style Mobile Phone Depth (Un-fold)

Figure A.1.2.3.2: Tendency of Book-Style Mobile Phone Depth (fold)

### A.1.3	Clamshell-style foldable structure size

### A.1.3.1	Length

Figure A.1.3.1.1: Tendency of Clamshell-Style Mobile Phone Length (Un-fold)

Figure A.1.3.1.2: Tendency of Clamshell-Style Mobile Phone Length (fold)

### A.1.3.2	Width

Figure A.1.3.2.1: Tendency of Clamshell -Style Mobile Phone width (Fold/Un-fold)

### A.1.3.3	Depth

Figure A.1.3.3.1: Tendency of Clamshell-Style Mobile Phone Depth (Un-fold)

Figure A.1.3.3.2: Tendency of Clamshell-Style Mobile Phone Depth (Fold)

Change-3

Create two sub-clauses in 7.1: microphone integration in mobile phones

7.1.1 Traditional Mobile phone

7.1.2 Foldable Mobile phone

Move the data available in TR clause 7.1 to 7.1.1 and under 7.1.2 include following information

7.1.2 Foldable Mobile phone

As stated in clause 5 MEMS microphones are the standard for mobile phones. The number and placement of microphones on foldable devices (including both book-style and clamshell designs) vary by manufacturer and model. Basic configurations in the foldable segment includes 3 microphones (positioned at the top, bottom bezel and rear), while more advanced models offer up to 6 microphones (with configurations of 2 microphones at the top, 2 at the bottom bezel, and up to 2 rear-facing). An increased number of microphones can significantly enhance audio quality during calls, contributing to a better overall user experience. Given the flexible design of foldable devices, various usage modes can be expected (example: full-screen, half-folded at a 90-degree angle, single-handed mode, desk mode, etc.,), therefore more attention may be needed when processing captured signals from multi-microphones positioned strategically to support high-quality audio capture. Following figures provide example microphone configurations for book-style and clamshell-style design.

The table below presents example characteristics of multi-microphone configurations for foldable devices. Due to the flexible design of these devices, various usage modes may alter the spacing between microphones. As a result, processing solutions may need to adapt to the adjustable microphone array configuration to deliver high-quality immersive audio quality.

Table 7.1.2.1: MEMS Microphone example characteristics for multi-array microphones in foldable mobile phones.

| Foldable Mobile Phone | Number of Microphones | Placement of Microphones | Spacing | Polar Pattern |
| --- | --- | --- | --- | --- |
| Book-Style (folded) | Triple Microphone array | Top, Bottom of bezel and Rear facing | 2 to 17 cm | Omni-directional |
|  | Hexa Microphone array | 2 microphones at the top, 2 at the bottom bezel, and up to 2 rear-facing | 4 to 17 cm |  |
| Book-Style (un-folded) | Triple Microphone array | Top, Bottom of bezel and Rear facing | 8 to 18 cm |  |
|  | Hexa Microphone array | 2 microphones at the top, 2 at the bottom bezel, and up to 2 rear-facing | 2 to 18 cm |  |
| Clamshell-Style (folded) | Triple Microphone array | Top, Bottom of bezel and Rear facing | 2 to 4 cm |  |
|  | Hexa Microphone array | 2 microphones at the top, 2 at the bottom bezel, and up to 2 rear-facing | 1 to 4 cm |  |
| Clamshell-Style (un-folded) | Triple Microphone array | Top, Bottom of bezel and Rear facing | 2 to 18 cm |  |
|  | Hexa Microphone array | 2 microphones at the top, 2 at the bottom bezel, and up to 2 rear-facing | 2 to 18 cm |  |

3. Conclusion

The foldable mobile phone market is on an upward trajectory, fueled by technological innovations and growing consumers perception of foldable as premium devices. The future for this segment looks promising as they carve out a share of the overall mobile phone market. With their flexible design supporting various usage modes (example: full-screen, half-folded at a 90-degree angle, single-handed mode, desk mode, etc.,), foldable devices can effectively double or reduce their dimensions compared to traditional mobile phone, a feature not currently reflected in the TR.

This flexibility requires greater attention when processing audio captured from strategically positioned multi-microphones to ensure high-quality sound. Additionally, the design variability of foldable phones introduces significant changes in audio capture, an aspect relevant to the scope of this study. Therefore, the source proposes updating the contents of clause 4.1, clause 7.1 and incorporating the proposals 1, 2, 3 into the TR 26.933[1].

4 References

[1] S4aA240055:TR 26.933.v.1.3.0

[2] https://www.mordorintelligence.com/industry-reports/foldable-smartphone-market