3GPP TSG-SA WG4 Meeting post 130	S4-242025

Orlando,  18th - 22nd November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | pCR |  | rev | - | Current version: | 0.3.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### Modification to description to Key Issue #2: Monitoring and measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MediaEnergyGREEN |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The latest draft of 3GPP TR 26.942 contains Key Issue #2: Monitoring and measurement under clause 6.2. Some modifications are proposed to the content of the description |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new text to be added in TR 26.942 on “Clause 6.2 Key Issue #2: Monitoring and measurement”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed objectives will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change

## 6.2	Key Issue #2: Energy-related Monitoring and measurement

### 6.2.1 	Description

To facilitate efficient energy use and energy saving for media services, first, measurements are useful. Environmental impact of the consumption of media services depends various actors: device manufacturer, networks, datacentres, service providers but also users through their usage behaviour.

Requirements of monitoring and measurement related to energy consumption and efficiency have already been defined in TR 22.882 [22882] but those requirements are only related to energy consumption monitoring of 5G network or 5G system. TR 23.700-66 [23700-66] TR 23.700-66 [23700-66] study and identify potential enhancements on 5GS (e.g. including network energy related information exposure, enhancement for subscription and policy control to enable energy efficiency as service criteria) to improve energy efficiency and to support energy saving in the network, taking the EnergyServ requirements in TS 22.261 [22261] into consideration. The purpose of this Key Issue is to extend this work, to study and identify potential enhancements to the energy-related monitoring and measurement aspects for media architectures, applications and services (e.g., the 5G Media Streaming System according to TS 26.501 [26501], 5G Multicast–Broadcast User Services according to TS 26.502 [26502], the Real-time Media Communication according to TS 26.506 [26506], Split rendering Media session Enabler according to TS 26.565 [26565], etc.) including UE-related energy information measurement and monitoring. The more efficient way to address energy saving would be to consider the end-to-end chain, seeking to understand how decisions made on one end of the chain impact power consumption on other ends of the workflow. As explained in section 4.2.2, UE data collection has been defined to monitor QoE in the 5G system. But energy-related information is missing.

NOTE 1:	It is presumed that the user has granted consent for its UE data to be collected, reported and subsequently exposed by means outside the scope of the study or following normative work.

NOTE 2:	The collection, reporting and exposure of location-based UE data is expected to comply with regional regulatory requirements and may be further limited by MNO policy.

In this context, the subsequent analysis by this Key Issue will consider:

- Which UE energy-related information will be collected to measure, correlate, and optimize energy usage across the entire streaming distribution chain?

- Which UE entity is appropriate to measure this UE energy-related information?

- How the existing methods could be leveraged to measure/monitor the UE energy-related information?

This issue is even more important for advanced media services such as XR services, Split Rendered media services, etc. which are expected to incur substantial energy consumption both at the device and network levels, presenting significant challenges for operators and service providers.

For instance, when a Mobile Network Operator (MNO) deploys a communication service to fulfil application service requirements, such as those of a gaming application, it is crucial for the customer – whether an Application Service Provider (ASP) or an industry vertical – to ensure that the application service reduces energy consumption for both end users and the data network. Failure to do so could necessitate short-notice application layer adaptations within the Application Service Provider's domain. This may result in adjustments being made to service levels in response to anticipated high energy consumption in specific service areas or during peak hours that adversely affect the Quality of Experience for service users.

In a practical scenario, an ASP intends to deploy a gaming service within a designated service area, served by an MNO's 5G network. Various service levels may exist, each associated with specific Key Performance Indicators (KPIs), such as automation levels or video quality targets. To ensure energy efficiency the ASP monitors the energy efficiency of its application service across specified service levels. Monitoring of application energy consumption may occur periodically or may be event-triggered, depending on the ASP's requirements, which are typically outlined in the Service Level Agreement (SLA). The MNO and ASP may agree on a certain energy efficiency target for the application service and optionally for given service levels.

This analysis will need to take into consideration existing work done in 3GPP but also other market trends. Application Service Providers are often reluctant to deploy solutions specific to mobile networks. Proposing technologies already supported in their services or technologies that are agnostic to the network, or which operate passively without the active involvement or knowledge of applications, are more likely adopted.

## End of change